3GPP TSG-RAN WG4 Meeting #108                                R4-2311943                                                                                                                                                                                   Toulouse, France, 21th-26th, Aug, 2023

Source:	Samsung, KT corporation, Qualcomm

Title:	TP for TR 38.898: DC_1-8_n77

Agenda item:			7.17.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.898 to include DC_1-8_n77 according to the request in [1].

The corresponding 3-band PC3 combo has already been specified.

The 2-band PC2 DC_1A_n77A and DC_1A_n77(2A) have already been specified, while PC2 DC_8A_n77A is proposed in this meeting as TP R4-2311946, and PC2 DC_8A_n77(2A) is proposed in this meeting as CR R4-2311946.

2. Reference

[1] RP-231032 Revised WID on High power UE (power class m with 1<m<3) for a single FR1 band in UL of Dual Connectivity (DC) combinations of x bands (x=1,2,3, 4 for y=1 or x=1, 2 for y=2) LTE inter-band CA (xDL/1UL) and y bands NR inter-band CA (yDL/1UL)

# 3. Text Proposal

<Start of Text Proposal>

### 5.x    DC_1-8_n77

#### 5.x.1	Configuration for DC

Table 5.x.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-8A_n77A5,14 | DC_1A_n77A14 DC_8A_n77A14 |
| DC_1A-8A_n77(2A)5,14 | DC_1A_n77A14 DC_8A_n77A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.x.2	Maximum output power for DC

Based on studies of PC2 DC_1_n77 and PC2 DC_8_n77, this section can be omitted.

#### 5.x.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_1_n77 and DC_8_n77, own Rx impact of the 3rd band is the followings.

- the 5th order IMD generated by dual uplink of band 1 and band n77 may also impact the own Rx of band 8.

- the 3nd order IMD generated by dual uplink of band 8 and band n77 may also impact the own Rx of band 1.

The new MSDs are specified in below table.

Table 5.x.3-1: MSD test points for Scell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-8A_n77A | 1 | 1955 | 5 | 25 | 2145 | N/A | N/A |
| DC_1A-8A_n77(2A) | n77 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | 8 | 910 | 5 | 25 | 955 | 15.7 | IMD5 |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n77 | 3960 | 10 | 50 | 3960 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 23.4 | IMD3 |

#### 5.x.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

<End of Text Proposal>