3GPP TSG-RAN WG4 Meeting #108                                 R4-2311949

Toulouse, France, 21th-25th, Aug, 2023

Source:	Samsung, Reliance Jio

Title:	TP for TR 37.718-11-21: DC_5_n28-n77

Agenda item:			7.6.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.718-11-21 to include DC_5_n28-n77 with dual UL according to the request in [1].

All the fallbacks are completed.

2. Reference

[1] RP-231059 Revised WID on Rel-18 DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)

# 3. Text Proposal

<Start of Text Proposal>

## 6.X		DC_5_n28-n77

### 6.X.1	 Operating bands for DC

Table 6.X.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5_n28-n77 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 6.X.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_5A_n28A-n77A DC_5A_n28A-n77C | DC_5A_n77A |

### 6.X.2 	Channel bandwidths per operating band for DC

Table 6.X.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_5A_n28A-n77A |  | 5 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 145 |
|  |  | n28 | 15 | 5 | 10 | 15 | 207 | 257 |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 207 | 257 |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 6.X.3	 Co-existence studies

Based on co-existence studies of DC_5_n77, own Rx impact of the 3rd band is the following.

- 4th and 5th IMD products caused by UL DC_5_n77 falls into the third band n28.

### 6.X.4	 ∆TIB and ∆RIB values

ΔTIB,c and ΔRIB,c are specified as below.

Table 6.X.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_5_n28-n77 | 0.6 | 0.5 | 0.9 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.X.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_5_n28-n77 | 0.2 | 0.2 | 0.8 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 6.X.4	 MSD requirements

Referring to CA_n5-n14-n77, the MSD requirements can be specified as below:

Table 6.X.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A_n28A-n77A DC_5A_n28A-n77C | 5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |
|  | n28 | 715.5 | 5 | 25 | 765.5 | 11.6 | IMD44 |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

<End of Text Proposal>