3GPP TSG-RAN WG4 Meeting #108                                R4-2311953                                                                                                                                                                                   Toulouse, France, 21th-25th, Aug, 2023

Source:	Samsung, Reliance Jio

Title:	TP for TR 37.718-21-11: DC_5-40_n78

Agenda item:			7.4.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.718-21-11 to include DC_5-40_n78 according to the request in [1].

Some 2-band fallbacks are proposed in this meeting together in CR R4-2311948. Others are specified already.

2. Reference

[1] RP-231132 Revised WID on Rel-18 Dual Connectivity (DC) of 2 bands LTE inter- band CA (2DL/1UL) and 1 NR band (1DL/1UL)

# 3. Text Proposal

<Start of Text Proposal>

## 5.x	DC_5-40_n78

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A-40A_n78A DC_5A-40C_n78A DC_5A-40A_n78C DC_5A-40C_n78C | DC_5A_n78A DC_40A_n78A |

### 5.x.2	Co-existence studies

Based on co-existence studies of DC_5_n78 and DC_40_n78, own Rx impact of the 3rd band is the followings.

-	No IMD products generated by DC_5_n78 uplink may fall into own Rx of band 40.

-   3th order IMD products generated by DC_40_n78 uplink may fall into own Rx of band 5.

### 5.x.3	∆TIB and ∆RIB values

For DC_5-40_n78, the TIB,c and RIB,c values are reused from DC_5_n40-n78 are given in the tables below.

Table 5.x.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_5-40_n78 | 0.6 | 0.5 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.x.3-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_5-40_n78 | 0.2 | 0.4 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.x.4	Reference sensitivity exceptions

Table 5.x.4-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for CA_n5-n40-n78 are reused.

Table 5.x.4-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5-40_n78 | 5 | 835 | 5 | 25 | 880 | 15.2 | IMD3 |
|  | 40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
|  | n78 | 3740 | 10 | 50 | 3740 | N/A | N/A |

<End of Text Proposal>