3GPP TSG-RAN WG4 Meeting #108                                R4-2311959

Toulouse, France, 21-25 Aug, 2023

Source:	Samsung, DISH Network, Fujitsu

Title:	TP for TR 38.718-02-01 CA_n29-n48

Agenda item:			7.10.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.718-02-01 to include CA_n29-n48 with single UL according to the request in [1].

2. Reference

[1] RP-231115 Revised WID Rel-18 NR inter band CA DC 2 bands DL with x bands UL (x=1,2)

# 3. Text Proposal

<Start of Text Proposal>

## 5.x	CA_n29-n48

### 5.x.1	Common for 1 band UL

#### 5.x.1.1		Operating bands for CA

Table 5.x.1.1-1: CA band combination of band n29 and n48

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n29 | N/A | – | N/A | 717 MHz | – | 728 MHz | SDL |
| n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |

#### 5.x.1.2		Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n29+n48

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n29A-n48A | - | n29 | 5, 10 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |

#### 5.x.1.3		UE co-existence studies

Table 5.x.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n29-n48 shown that there are 5th harmonic mixing issues to consider.

Table 5.x.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n29 |  |  | 717 | 728 |  |  |  |  |  |  |  |  |
| n48 | 3550 | 3700 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 | 14200 | 14800 | 17750 | 18500 |

Table 5.x.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n29 |  |  | 717 | 728 | 1434 | 1456 | 2151 | 2184 | 2868 | 2912 | 3585 | 3640 |
| n48 | 3550 | 3700 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 | 14200 | 14800 | 17750 | 18500 |

#### 5.x.1.4		∆TIB and ∆RIB values

For CA_n29-n48, the TIB,c and RIB,c values are same as for CA_ n29-n77 and are given in the tables below.

Table 5.x.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n29-n48 | - | 0.8 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.x.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n29-n48 | 0.2 | 0.5 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

#### 5.x.1.5 		REFSENS requirements

- The 5th harmonic mixing falls into Rx frequencies of band 29.

- For CA_n29-n48, the MSD values due to harmonic mixing are same as for CA_ n29-n77 and are given in the tables below.

Table 5.x.1.5-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n48 | n29 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 5 | UL1/DL5 |
| n48 | n29 | 10 | 15 | 50 (RBstart=0) | 10 | 28 | NOTE 5 | UL1/DL5 |

#### 5.x.1.6	OOB blocking exception requirements

There is no OOB exception for this CA combination.

Table 5.x.1.6-1: CA band combination with exceptions allowed

CA band combination

<End of Text Proposal>