3GPP TSG-RAN WG4 Meeting #108                                R4-2311960

Toulouse, France, 21-25 Aug, 2023

Source:	Samsung, DISH Network

Title:	TP for TR 38.718-02-01 CA_n66-n70

Agenda item:			7.10.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.718-02-01 to include CA_n66-n70 with dual UL according to the request in [1].

2. Reference

[1] RP-231115 Revised WID Rel-18 NR inter band CA DC 2 bands DL with x bands UL (x=1,2)

# 3. Text Proposal

<Start of Text Proposal>

## 5.x	CA_n66-n70

### 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1		Operating bands for CA

Table 5.x.1.1-1: CA band combination of band n66 and n70

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
| n70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz | FDD |

#### 5.x.1.2		Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n66+n70

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n66A-n70A | CA_n66A-n70A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
| CA_n66(2A)-n70A | CA_n66A-n70A | n66 | CA_n66(2A)_BCS0 | 0 |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
|  |  | n66 | CA_n66(2A)_BCS1 | 1 |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
| CA_n66(3A)-n70A | CA_n66A-n70A | n66 | CA_n66(3A)_BCS0 | 0 |
|  |  | n70 | 5, 10, 15, 20, 25 |  |

Please note that BCS0 for CA_n66A-n77A with single UL has already been specified in TS 38.101-1 v18.2.0 which means harmonic/harmonic mixing/cross band isolation have already been analysed and specified if any, BCS1 has no new MSD issue compared with BCS0.

### 5.x.2		Specific for 2 bands UL CA

#### 5.x.2.1	Maximum output power for inter-band CA

Table 5.x.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n66A-n70A | 23 | +2/-3 |

#### 5.x.2.2		UE co-existence studies

Table 5.x.2.2-1 lists Band n66 + Band n70 2UL bands CA 2nd, 3rd, 4th and 5th order IMD analysis.

Table 5.x.2.2-1: Band n66 and Band n70 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1710 | 1780 | 1695 | 1710 |
| 2nd order IMD products | ¦fx_low – fy_high¦ | ¦fx_high – fy_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 0 | 85 | 3405 | 3490 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1710 | 1865 | 1610 | 1710 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5115 | 5270 | 5100 | 5200 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3420 | 3645 | 3305 | 3420 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6825 | 7050 | 6795 | 6910 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 0 | 170 | 6810 | 6980 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5000 | 5130 | 5130 | 5425 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8490 | 8620 | 8535 | 8830 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1525 | 1710 | 1710 | 1950 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8505 | 8690 | 8520 | 8760 |

Based on the table above it can be seen that there is no IMD due to two UL falls into own DL.

In terms of protected bands for this 2UL band CA configuration, the constituent bands of these band combination have no frequency range defined for protection, so no frequency range need to be defined for this band combination.

#### 5.x.2.3		REFSENS requirements

Based on co-existence studies on 5.x.2.2, no need to define exceptional REFSENS requirements.

<End of Text Proposal>