3GPP TSG-RAN WG4 Meeting #108	R4-2311962

Toulouse, France, 21-25 Aug, 2023

Source:	Samsung, DISH Network, Fujitsu

Title:	TP for TR 38.718-03-01 to include CA_n26-n66-n70

Agenda item:			7.11.2

Document for:	Approval

1. Introduction

This contribution is a text proposal for TR 38.718-03-01 to include CA_n26-n66-n70.

All, except CA_n66-n70, fallbacks have been specified. TP to add CA_n66-n70 is proposed in this meeting as R4-2311960.

2. Text Proposal

---Start of changes---

## 5.x	CA_n26-n66-n70

### 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n26-n66-n70 | n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz | FDD |

#### 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n26+n66+n70

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n26A-n66A-n70A | CA_n66A-n70A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n26A-n66(2A)-n70A | CA_n66A-n70A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n26A-n66(3A)-n70A | CA_n66A-n70A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
| NOTE 1: This UE channel bandwidth is applicable only to downlink |  |  |  |  |

### 5.x.2	Specific for 2 bands UL CA

#### 5.x.2.1	UE co-existence studies

Table 5.x.2.2-1 lists Band n66 + Band n70 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE coexistence analysis.

Table 5.x.2.2-1: Band n66 and Band n70 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1710 | 1780 | 1695 | 1710 |
| 2nd order IMD products | ¦fx_low – fy_high¦ | ¦fx_high – fy_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 0 | 85 | 3405 | 3490 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1710 | 1865 | 1610 | 1710 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5115 | 5270 | 5100 | 5200 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3420 | 3645 | 3305 | 3420 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6825 | 7050 | 6795 | 6910 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 0 | 170 | 6810 | 6980 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5000 | 5130 | 5130 | 5425 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8490 | 8620 | 8535 | 8830 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1525 | 1710 | 1710 | 1950 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8505 | 8690 | 8520 | 8760 |

Based on the table above it can be seen that there is no n66 + n70 IMD may affect Rx frequencies of band n26.

#### 5.x.2.2	REFSENS requirements

Based on co-existence studies on 5.x.2.1, no need to define exceptional REFSENS requirements.

---End of changes---

Reference

[1] 	RP-231100, Revised WID on Rel-18 NR Inter-band CA/DC for 3 bands DL with x bands UL (x=1,2),  ZTE