3GPP TSG-RAN WG4 Meeting #108                                R4-2311971

Toulouse, France, 21-25 Aug, 2023

Source:	Samsung, KDDI, Qualcomm

Title:	TP for TR 38.898 HPUE DC_18A_n77A

Agenda item:			7.17.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.898 to include DC_18_n77 according to the request in [1].

2. Reference

[1] RP-231032 Revised WID on High power UE (power class m with 1<m<3) for a single FR1 band in UL of Dual Connectivity (DC) combinations of x bands (x=1,2,3, 4 for y=1 or x=1, 2 for y=2) LTE inter-band CA (xDL/1UL) and y bands NR inter-band CA (yDL/1UL)

# 3. Text Proposal

<Start of Text Proposal>

### 5.x	DC_18_n77

#### 5.x.1	Configuration for DC

When requested EN-DC configuration is 2DL2UL, it is not needed to update the inter-band EN-DC configuration table. By referring to the maximum output power table, it can be checked whether these configurations support PC2. This band combination for PC3 is already specified in TS 38.101-3, so this section is omitted.

#### 5.x.2	Maximum output power for DC

Table 5.x.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_18A_n77A | 266 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |  |  |

#### 5.x.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC.

- No new harmonic issue since band 18 can only transmit PC3.

- the 4th order harmonic mixing may fall into Rx frequencies of band 18.

- IMD4/5 of dual UL may fall into Rx frequencies of Band 18.

- No cross band isolation interference.

There is the 4th order harmonic mixing, which is not mandatory to be defined according to Rel-16 agreement.

Similar with PC3 CA_n18-n77 and DC_18_n77, as clarified by note 8 in table 7.3A.5-1, there is no need to define exact values for IMD4/5.

New PC2 MSDs are defined in the following tables.

Table 5.x.3-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_18A_n77AX | 18 | N/A | N/A | N/A | N/A | N/A | IMD4/5 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE X: There is no IMD4/5 products in band n18 downlink for n77 operating in 3520 – 3560 MHz, 3700 – 3800MHz and 4000 - 4100MHz frequency range. |  |  |  |  |  |  |  |

#### 5.x.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC.

<End of Text Proposal>