3GPP TSG-RAN WG4 Meeting #108	R4-2311981

Toulouse, France, Aug 21 – 25, 2023

Agenda item:	8.22.1

Source: 	Qualcomm Incorporated

Title: 	Reply LS on SRS and PRS BW aggregation for positioning

Document for:	Approval

# Introduction

RAN1 has sent an LS on SRS and PRS BW aggregation for positioning [1]. Our views and proposals can be found in a companion paper [2]. A draft reply LS is included below in the Appendix.

# References

[1] R1-2306216, LS to RAN4 on SRS and PRS bandwidth aggregation for positioning, RAN1#113

[2] R4-2311986, On requirements for PRS/SRS BW aggregation, Qualcomm Inc., RAN4#108

# Appendix

3GPP TSG-RAN WG4 Meeting #108	R4-23xxxxx

Toulouse, France, Aug 21 – 25, 2023

Title:	Reply LS on SRS and PRS BW aggregation for positioning

Response to:	R1-2306216 (LS to RAN4 on SRS and PRS bandwidth aggregation for positioning)

Release:	Release 18

Work Item:	NR_pos_enh2-Core

Source:	RAN4

To:	RAN1

Cc:

Contact Person:

- Name:	Carlos Cabrera-Mercader

- E-mail Address:	ccmercad@qti.qualcomm.com

Attachments: -

1. Overall Description:

RAN4 would like to thank RAN1 for their LS on SRS and PRS BW aggregation for positioning (R1-2306216).

Agreement At least from UE capability perspective, the UE support of positioning SRS bandwidth aggregation in RRC_CONNECTED state is decoupled from the UE support of communication CA. Agreement When an SRS resource configured within a CC without PUSCH/PUCCH is linked for aggregation with an SRS resource configured within an UL active BWP of a UL communication CC, a guard period is needed before and after the aggregated SRS transmissions. - Send an LS to RAN4 with the above information and a request to provide the retuning time values needed. Agreement For PRS bandwidth aggregation, with regards to the signaling in the location information request message, introduce the following: - A request to indicate UE which two or three PFLs to be used for performing joint measurement - A new ReportingGranularityfactor smaller than 0 which can be applicable at least when the LMF requests aggregated measurements - Support at least the values of k={-1,-2} - FFS other values e.g. -3, -4, -5, -6 - Send RAN4 an LS to confirm the feasibility

Response from RAN4:

Regarding the first two agreements in the RAN1 LS, RAN4 discussed the guard period needed before and after an aggregated SRS transmission when an SRS resource configured within a CC without PUSCH/PUCCH is linked for aggregation with an SRS resource configured within an UL active BWP of a UL communication CC. RAN4 concluded that feasible switching/guard period values before and after each aggregated SRS transmission are {n0us, n30us, n100us, n140us, n200us, n300us, n500us, n900us}.

Regarding the third agreement in the RAN1 LS, RAN4 considers that the values k={-1,-2} are feasible based on agreements from RAN4#107. RAN4 will evaluate whether it is beneficial to introduce even finer granularities and provide feedback once it concludes the evaluation.

2. Actions:

To RAN1:

RAN4 kindly requests RAN1 to consider the above response in their future work.

3. Date of Next TSG-RAN4 Meetings:

TSG-RAN4 Meeting #108-bis			Oct 9 – 13, 2023, Xiamen, China

TSG-RAN4 Meeting #109			Nov 13 – 17, 2023, Chicago, USA