3GPP TSG-RAN WG4 Meeting #108	R4-2311984

Toulouse, France, Aug 21 – 25, 2023

Agenda item:	8.22.3.4

Source: 	Qualcomm Incorporated

Title: 	Simulation results for 1Rx RedCap UEs without frequency hopping – updated side-condition for fading channels

Document for:	Information

# Introduction

RAN4 revised the simulation assumptions for PRS measurement accuracy for 1Rx RedCap UEs without frequency hopping [1]. This paper includes simulation results with revised side-conditions for the following measurements in fading propagation conditions:

- UE Rx-Tx time difference

- PRS-RSRP

- RSTD

# Simulation results for UE Rx-Tx time difference

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Num Rx | Accuracy [90%] (Tc) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | SINR = -6 dB | SINR = -10 dB |
| FR1 | 24 | 15 | 256 | 4 | 4 | 1 |  |  |
|  | 52 |  | 128 | 4 | 1 | 1 | 149 | 174 |
|  | 104 |  | 64 | 4 | 1 | 1 | 103 | 114 |
|  | 24 | 30 | 128 | 4 | 4 | 1 |  |  |
|  | 48 |  | 64 | 4 | 1 | 1 | 121 | 143 |

Table 1: UE Rx-Tx simulation results for FR1 in TDL-A.

# Simulation results for PRS-RSRP

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Num Rx | Accuracy [5%, 95%] (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | SINR = -6 dB | SINR = -10 dB |
| FR1 | 24 | 15 | 256 | 4 | 4 | 1 |  |  |
|  | 52 |  | 128 | 4 | 1 | 1 | 4.9 | 4.2 |
|  | 104 |  | 64 | 4 | 1 | 1 | 3.9 | 3.5 |
|  | 24 | 30 | 128 | 4 | 4 | 1 |  |  |
|  | 48 |  | 64 | 4 | 1 | 1 | 4.5 | 4.1 |

Table 2: PRS-RSRP simulation results for FR1 in TDL-A.

# Simulation results for RSTD

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Num Rx | Accuracy [5%, 95%] (Tc) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | SINRref = -6 dB, SINRi = -6 dB | SINRref = -6 dB, SINRi = -10 dB |
| FR1 | 24 | 15 | 256 | 4 | 4 | 1 |  |  |
|  | 52 |  | 128 | 4 | 1 | 1 | ±125 | ±144 |
|  | 104 |  | 64 | 4 | 1 | 1 | ±68 | ±77 |
|  | 24 | 30 | 128 | 4 | 4 | 1 |  |  |
|  | 48 |  | 64 | 4 | 1 | 1 | ±72 | ±97 |

Table 3: RSTD simulation results for FR1 in TDL-A.

# References

[1] R4-2310075, Updated simulation assumptions for 1Rx RedCap UE PRS measurement without FH, RAN4#107