3GPP TSG-RAN WG4 Meeting #108	R4-2311988

Toulouse, France, Aug 21 – 25, 2023

Agenda item:	5.2.3

Source: 	Qualcomm Incorporated

Title: 	[NR_pos_enh-Perf] Updated simulation results for PRS-RSRPP

Document for:	Information

# Introduction

Measurement accuracy requirements for PRS-RSRPP in Rel-17 are still pending in RAN4. In RAN4#107, some companies updated simulation results and a new summary of results was provided [1].

In this paper we provide updated simulation results for PRS-RSRPP accuracy requirements. These new simulation results supersede our previously submitted results [2, 3].

# Simulation results

Simulation assumptions to derive PRS-RSRPP measurement accuracy requirements were agreed in RAN4#103-e [4].

Simulated accuracy for the two-tap channel model (38.101-4 Annex B.2.4) with parameters a = 1, τd=0.45 µs and fD=5 Hz is shown in the tables below for FR1 (SCS = 15 kHz, 30 kHz and 60 kHz) and FR2 (60 kHz and 120 kHz). The reported accuracy does not include RF impairment margin.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [5%, 95%] (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -13 dB |
| FR1 | 24 | 15 | 256 | 4 | 1 | ±1.2 | ±3.2 |
|  | 52 |  | 128 | 4 | 1 | ±1.4 | ±2.1 |
|  | 104 |  | 64 | 4 | 1 | ±1.3 | ±1.5 |
|  | 24 | 30 | 128 | 4 | 1 | ±1.6 | ±3.2 |
|  | 48 |  | 64 | 4 | 1 | ±0.8 | ±2.3 |
|  | 132 |  | 32 | 4 | 1 | ±0.7 | ±1.9 |
|  | 24 | 60 | 64 | 4 | 1 | ±1.6 | ±3.3 |
|  | 64 |  | 32 | 4 | 1 | ±0.8 | ±2.4 |
|  | 132 |  | 16 | 4 | 1 | ±0.8 | ±1.8 |

Table 1: PRS-RSRPP simulation results for FR1 with 4 samples.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [5%, 95%] (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = 0 dB | SINR = -6 dB |
| FR1 | 52 | 15 | 128 | 1 | 1 | ±1.3 | ±1.5 |
|  | 104 |  | 64 | 1 | 1 | ±1.3 | ±1.3 |
|  | 48 | 30 | 64 | 1 | 1 | ±0.8 | ±1.6 |
|  | 132 |  | 32 | 1 | 1 | ±0.7 | ±0.8 |
|  | 64 | 60 | 32 | 1 | 1 | ±0.7 | ±1.6 |
|  | 132 |  | 16 | 1 | 1 | ±0.7 | ±0.8 |

Table 2: PRS-RSRPP simulation results for FR1 with 1 sample.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [5%, 95%] (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -13 dB |
| FR2 | 24 | 60 | 64 | 4 | 1 | ±1.6 | ±3.3 |
|  | 64 |  | 32 | 4 | 1 | ±0.8 | ±2.4 |
|  | 132 |  | 16 | 4 | 1 | ±0.8 | ±1.8 |
|  | 32 | 120 | 32 | 4 | 1 | ±1.6 | ±2.7 |
|  | 64 |  | 16 | 4 | 1 | ±1.4 | ±2.0 |
|  | 128 |  | 8 | 4 | 1 | ±1.3 | ±1.6 |

Table 3: PRS-RSRPP simulation results for FR2 with 4 samples.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [5%, 95%] (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = 0 dB | SINR = -6 dB |
| FR2 | 64 | 60 | 32 | 1 | 1 | ±0.7 | ±1.6 |
|  | 132 |  | 16 | 1 | 1 | ±0.7 | ±0.8 |
|  | 64 | 120 | 16 | 1 | 1 | ±0.7 | ±1.5 |
|  | 128 |  | 8 | 1 | 1 | ±1.3 | ±1.4 |

Table 4: PRS-RSRPP simulation results for FR2 with 1 sample.

# References

[1] R4-2308956, Summary of PRS-RSRPP simulations, Ericsson, RAN4#107

[2] R4-2218374, Discussion on R17 RRM maintenance issues, Qualcomm Inc., RAN4#105

[3] R4-2212198, On performance requirements for PRS-RSRPP, Qualcomm Inc., RAN4#104-e

[4] RP-2211061, Simulation assumptions for PRS-RSRPP, RAN4#103-e