3GPP TSG-RAN WG4 Meeting RAN4#108      				R4-2311991 Toulouse, France

August 22 – 26, 2023

Agenda Item:	8.22.3.6

Source:	Lenovo, Motorola Mobility

Title:	Effect of Carrier Frequency Offset on Carrier Phase Positioning

Document for:  Discussion

- Introduction

The issue of carrier frequency offset as it relates to carrier phase positioning was considered previously in [1]. In this contribution, we extend the analysis in [1] to address the impact of carrier frequency offsets on the carrier phase difference measurements due to the relative phase of the carriers changing from one symbol to the next.

- Carrier Frequency Offset

Carrier phase positioning assumes the existence of initial phase offsets among the DL-PRS transmitters, i.e., gNBs, and between the DL-PRS transmitters and the UE and PRU receivers which are static.  In general, however, there will be frequency offsets among the TRP transmitters which will cause the carrier phase offsets to change relative to each other over time.  Similarly, there will be carrier frequency offsets between the TRP transmitters and the UE and PRU receivers which will cause these phase offsets to change over time.

The carrier frequency tolerance for the base station is defined in TS 38.104 and is given in the following two tables:

Table 6.5.1.2-1: Frequency error minimum requirement

| BS class | Accuracy |
| --- | --- |
| Wide Area BS | ±0.05 ppm |
| Medium Range BS | ±0.1 ppm |
| Local Area BS | ±0.1 ppm |

Table 9.6.1.3-1: OTA frequency error minimum requirement

| BS class | Accuracy |
| --- | --- |
| Wide Area BS | ±0.05 ppm |
| Medium Range BS | ±0.1 ppm |
| Local Area BS | ±0.1 ppm |

The carrier frequency tolerance for the UE is defined in TS 38.101-1 for FR1.  In section 6.4.1 of TS 38.101-1, we have the requirement:

The UE basic measurement interval of modulated carrier frequency is 1 UL slot. The mean value of basic measurements of UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to the carrier frequency received from the NR Node B.

and a similar requirement can be found for UL MIMO in Section 6.4D.1.

Thus, except for the wide area base station, the frequency accuracy requirement is ± 0.1 PPM which corresponds to a maximum frequency error of ± 102 fc, where fc is expressed in GHz.

- Effect of CFO on Carrier Phase Difference Measurements

To examine the effect of carrier frequency offset, we consider the simplest case in which two TRP’s with indices l and m, are transmitting DL-PRS and the channels between each of two TRP’s and the UE are assumed to be line-of-sight (no multipath).  We further assume that the carrier transmitted by each TRP has a fixed frequency offset (within the required ± 0.1 ppm frequency tolerance) relative to the frequency of the serving cell of the UE.

Let yi,k,l denote the demodulated DL-PRS symbol received from TRP l on the i-th subcarrier and k-th symbol which is given by

yi,k,l=ri,k,l ej-ωc+i Δω+Δωl,UETX τl+Δωl,UETXπ Δω+φlTX0+k Δφl,UETX-φUERX0+ni,k ,

where ri,k,l denotes the known reference symbol, ωc denotes the carrier frequency (in radians), Δω denotes the subcarrier spacing, Δωl,UETX denotes the frequency offset of the carrier of the l-th TRP relative to that of the UE receiver, τl=dlc denotes the propagation delay corresponding to the distance dl between the l-th TRP and the UE, φUERX0 denotes the initial phase of the UE carrier, φlTX0 denotes the initial phase (for n=0) of the transmitter carrier for the l-th TRP, and ni,k is the additive noise at the output of the demodulator for the k-th symbol of the i-th subcarrier.

The phase increment Δφl,UETX denotes the change in the phase of the carrier of the l-th TRP relative to that of the UE carrier over the period of one symbol given by

Δφl,UETX= Δωl,UETX Ts,

where Ts is the symbol period.  The phase term Δωl,UETXπ Δω is due to the frequency offset of the subcarrier with the correlator demodulator and is computed as

argΔω2π02πΔωej Δωl,UETX t dt .

It can be noted that the term Δωl,UETX τl due to carrier frequency offset was considered previously in [1].

Multiplying yi,k,l by the conjugate ri,k,l* of the known positioning reference symbol for the l-th TRP (assumed to have unit amplitude) yields

wi,k,l=yi,k,l ri,k,l*=ej-ωc+i Δω+Δωl,UETX τl+Δωl,UETXπ Δω+φlTX0+k Δφl,UETX-φUERX0+ri,k,l* ni,k.

Similarly, for the i-th subcarrier of the k+n-th symbol received from TRP m, we have

wi,k+n,m=yi,k+n,m ri,k+n,m*=ej-ωc+i Δω+Δωm,UETX τm+Δωm,UETXπ Δω+φmTX0+k+n Δφm,UETX-φUERX0+ri,k+n,m* ni,k+n

Multiplying wi,k+n,m by the conjugate of wi,k,l yields

wi,k,l*wi,k+n,m=ej-ωc+i Δω τm-τl-Δωm,UETXτm+Δωl,UETXτl+Δωm,UETXπ Δω-Δωl,UETXπ Δω+φmTX0-φlTX0+ k Δφm,UETX-Δφl,UETX+ n Δφm,UETX+ejωc+i Δω τl+Δωl,UETXτl- Δωl,UETXπ Δω-φlTX0-k Δφl,UETX+φUERX0 ri,k+n,m* ni,k+n+ej-ωc+i Δω τm-Δωm,UETXτm+Δωm,UETXπ Δω+φmTX0+k+n Δφm,UETX-φUERX0ri,k,lni,k*+ri,k,lni,k* ri,k+n,m* ni,k+n,

=ej-ωc+i Δω τm-τl-Δωm,UETXτm+Δωl,UETXτl+Δωm,UETXπ Δω-Δωl,UETXπ Δω+φmTX0-φlTX0+k Δφm,UETX-Δφl,UETX+ n Δφm,UETX        +ni,k,k+n,l,m,

where the term

Δωm,UETXπ Δω-Δωl,UETXπ Δω+φmTX0-φlTX0+k Δφm,UETX-Δφl,UETX+ n Δφm,UETX

depends on the initial phases of the TRP transmitters and the carrier frequency offsets of the TRP transmitters relative to the UE receiver and is unknown to the UE.  The term

-Δωm,UETXτm+Δωl,UETXτl,

depends on both the carrier frequency offsets and the propagation delays τm and τl and is also unknown to the UE.  The noise term ni,k,k+n,l,m is implicitly defined in the above as the sum of three noise terms.

We define the term φm,lTX0 as the difference between the initial phase offsets of the m-th and l-th TRP’s given by

φm,lTX0=φmTX0-φlTX0 ,

the term Δωm,lTX  as the difference between the frequency offsets of the m-th and l-th TRP’s relative to the UE given by

Δωm,lTX=Δωm,UETX-Δωl,UETX ,

and Δφm,lTX as the change in the phase of the carrier of the m-th TRP relative to that of the l-th TRP over the period of one symbol given by

Δφm,lTX=Δφm,UETX-Δφl,UETX .

With this notation, we have

wi,k,l*wi,k+n,m=ej-ωc+i Δω τm-τl-Δωm,UETXτm+Δωl,UETXτl+Δωm,lTXπ Δω +φm,lTX0+k Δφm,lTX+ n Δφm,UETX+ni,k,k+n,l,m .

It should be noted that the term Δφm,UETX depends on the frequency offset of the m-th TRP to the carrier frequency of the UE.

Taking the same measurements on the same slot at the PRU yields

ui,k,l*ui,k+n,m=ej-ωc+i Δω τm'-τl'-Δωm,PRSTXτm'+Δωl,PRSTXτl'+Δωm,lTXπ Δω+φm,lTX0+k Δφm,lTX+ n Δφm,PRUTX+n'i,k,k+n,l,m .

where ui,k,l and ui,k+n,m are defined in the same manner as the measurements wi,k,l and wi,k+n,m taken at the UE, and the delays τm' and τl' between the TRP’s and the PRU are known. The phase increment Δφm,PRUTX denotes the change in the phase of the carrier frequency of the m-th TRP relative to that of the PRU receiver over the period of one symbol.

As noted in [1], the terms

-Δωm,UETXτm+Δωl,UETXτl , and

-Δωm,PRSTXτm'+Δωl,PRSTXτl'

are very small both in absolute terms and relative to the other terms.  In particular

-Δωm,UETXτm+Δωl,UETXτl≤foffset,maxτm+τl

-Δωm,PRSTXτm'+Δωl,PRSTXτl'≤foffset,maxτm'+τl' .

For a carrier frequency of 5 GHz with a frequency error of 0.1ppm and a maximum range of 200 meters, the corresponding phase term is given by

2 π ∙ 5∙109∙10-7∙200+2003∙108=0.004 radians=0.24 degrees ,

and as a result, these terms can be ignored.

With this approximation, we have

wi,k,l*wi,k+n,m≈ej-ωc+i Δω τm-τl+Δωm,lTXπ Δω+φm,lTX0+k Δφm,lTX+ n Δφm,UETX+ni,k,k+n,l,m ,

and

ui,k,l*ui,k+n,m≈ej-ωc+i Δω τm'-τl'+Δωm,lTXπ Δω+φm,lTX0+k Δφm,lTX+ n Δφm,PRUTX+n'i,k,k+n,l,m .

With knowledge of the delays τl' and τm' from the known locations of the TRP’s and the PRU, the LMF can use ui,k,l*ui,k+n,m to estimate the phase term

Δωm,lTXπ Δω+φm,lTX0+k Δφm,lTX+ n Δφm,PRUTX

If we define the arg∙ function to have range in the interval 0, 2π, then

argwi,k,l*wi,k+n,m

=argej-ωc+i Δω τm-τl+Δωm,lTXπ Δω+φm,lTX0+k Δφm,lTX+ n Δφm,UETX+ni,k,k+n,l,m

=modargej-ωc+i Δω τm-τl+Δωm,lTXπ Δω+φm,lTX0+k Δφm,lTX+ n Δφm,UETX

+ arg1+ni,k,k+n,l,me-j-ωc+i Δω τm-τl+Δωm,lTXπ Δω+φm,lTX0+k Δφm,lTX+ n Δφm,UETX,2π

=modmod-ωc+i Δω τm-τl+Δωm,lTXπ Δω+φm,lTX0+k Δφm,lTX+n Δφm,UETX,2π

+ arg1+ni,k,k+n,l,me-j-ωc+i Δω τm-τl+Δωm,lTXπ Δω+φm,lTX0+k Δφm,lTX+ n Δφm,UETX,2π .

Since the mean of ni,k,k+n,l,m is zero, the expected value of the “arg” term is zero and

Eargwi,k,l*wi,k+n,m

=mod-ωc+i Δω τm-τl+Δωm,lTXπ Δω+φm,lTX0+k Δφm,lTX+n Δφm,UETX,2π .

Subtraction of the PRU-based estimate of the estimate of the transmitter phase term yields

modEargwi,k,l*wi,k+n,m-Δωm,lTXπ Δω+φm,lTX0+k Δφm,lTX+ n Δφm,PRUTX ,2π

=modmod-ωc+i Δω τm-τl+Δωm,lTXπ Δω+φm,lTX0+k Δφm,lTX+n Δφm,UETX,2π-Δωm,lTXπ Δω+φm,lTX0+k Δφm,lTX+ n Δφm,PRUTX ,2π

≈mod-ωc+i Δω τm-τl+n Δφm,UETX- Δφm,PRUTX,2π.

where we have assumed that the estimate is equal to the actual value so that

Δωm,lTXπ Δω+φm,lTX0+k Δφm,lTX+n Δφm,UETX-Δωm,lTXπ Δω+φm,lTX0+k Δφm,lTX+ n Δφm,PRUTX ≈n Δφm,UETX- Δφm,PRUTX .

As a result,

modEargwi,k,l*wi,k+n,m-Δωm,lTXπ Δω+φm,lTX0+k Δφm,lTX+ n Δφm,PRUTX ,2π

≈modωc+i Δω τl-τm+n Δφm,UETX- Δφm,PRUTX,2π .

From this, it can be observed that the expected value of the estimate has a non-zero error term given by

n Δφm,UETX- Δφm,PRUTX,

where n is the separation in time, measured in symbols, between the DL-PRS symbols transmitted from TRP l and TRP m, and Δφm,UETX and Δφm,PRSTX denotes the symbol-to-symbol phase rotation of the m-th TRP transmitter carrier relative to the receiver carriers of the UE and DL-PRS, respectively.

Given that the frequency error of the TRP, the UE, and the DL-PRS are all limited to ± 0.1 ppm, the maximum frequency difference between the TRP and the UE and between the TRP and the DL-PRS are both limited to 0.2 ppm.  In the case that the UE carrier frequency is at the range minimum and the DL-PRS carrier frequency is at the range maximum, this error term can be expressed in degrees as

n Δφm,UETX- Δφm,PRSTX=n Ts∙2∙10-7 fc ∙360 .

The maximum phase error as a function of carrier frequency and the number of symbols separating the DL-PRS symbols is shown in Figure 1 for the case of subcarrier spacing equal to 15 kHz.

Figure 1:  Maximum phase error vs. time between DL-PRS symbol from different TRP’s (in symbols)

From Figure 1, it is apparent that carrier frequency offsets between the TRP transmitters and the UE and DL-PRS receivers can result in large measurement errors which depend on both the time distance between the DL-PRS symbols and the carrier frequency.  If the DL-PRS measurements are taken within the same slot, then the maximum distance between DL-PRS symbols is 13.  Conversely, if the DL-PRS symbols are in different slots, then the maximum distance can be much larger.

It can be argued (similar to [1]) that the measurement error n Δφm,UETX- Δφm,PRSTX is small relative to the desired measurement ωc+i Δω τl-τm.  However, because phase can only be measured modulo 2π, the significance of the measurement error must be considered relative to the interval 0, 2π.

Observation 1: Unless the carrier frequency offset of the TRP relative to the UE and the DL-PRS is the same, the carrier frequency offset can result in very substantial errors in the measurement of carrier phase difference.

Observation 2: The carrier phase difference measurement errors resulting from carrier frequency offset are a function of the difference in the carrier frequency offsets of the TRP relative to the UE and the PRU, and the separation in time of the DL-PRS symbols from the two TRP’s.

Observation 3: The expected value of the carrier phase measurement error can be computed as

n Δφm,UETX- Δφm,PRUTX

where Δφm,UETX and Δφm,UETX denote the phase rotation over one symbol of the of the carrier of the m-th TRP relative to the UE and the PRU, respectively, and n is the difference in the symbol indices of the two DL-PRS symbols.

An example of the placement of the DL-PRS symbols for three TRP’s is shown in Figure 2 below based on comb-6 configuration. For this example, there are two DL-PRS symbols from each of the three TRP’s in each row (subcarrier).  The distance between the DL-PRS for a pair of TRP’s depends both on the subcarrier selected and the DL-PRS selected within the subcarrier.

Figure 2: Example of a PRB with a comb- 6 positioning reference configuration for three TRP’s

- Correction of Carrier Phase Difference Measurements

The error due to the carrier frequency offsets are predictable if the carrier frequency offsets between the TRP transmitters and the UE and PRU receivers are known.  For this reason, in addition to the carrier phase and/or the carrier phase difference measurements, the UE and the PRU can measure and report the carrier frequency offsets for each of the TRP’s over which the carrier phase difference measurements are taken.  The carrier frequency offset can be determined by measuring the phase difference between reference symbols transmitted by the same TRP.

As above, let yi,k,l denote the demodulated DL-PRS symbol received from TRP l on the i-th subcarrier and k-th symbol. After multiplication by the conjugate of the reference symbol we have

wi,k,l=yi,k,l ri,k,l*=ej-ωc+i Δω+Δωl,UETX τl+Δωl,UETXπ Δω+φl,UETX0+k Δφl,UETX-φUERX0+ri,k,l* ni,k .

Similarly, for the DL-PRS symbol received from TRP l on the i-th subcarrier and the k+m-th symbol, we have

wi,k+m,l=yi,k+m,l ri,k+m,l*=ej-ωc+i Δω+Δωl,UETX τl+Δωl,UETXπ Δω+φl,UETX0+k+mΔφl,UETX-φUERX0+ri,k+m,l* ni,k+m .

Multiplication of wi,k+m,l by the conjugate of wi,k,l yields

wi,k+m,l wi,k,l*=ej-ωc+i Δω+Δωl,UETX τl+Δωl,UETXπ Δω+φl,UETX0+k+mΔφl,UETX-φUERX0+ri,k+m,l* ni,k+m∙e-j-ωc+i Δω+Δωl,UETX τl+Δωl,UETXπ Δω+φl,UETX0+k Δφl,UETX-φUERX0+ri,k,lni,k*

=ej m Δφl,UETX+ej-ωc+i Δω+Δωl,UETX τl+Δωl,UETXπ Δω+φl,UETX0+k+mΔφl,UETX-φUERX0ri,k,lni,k*+e-j-ωc+i Δω+Δωl,UETX τl++Δωl,UETXπ Δωφl,UETX0+k Δφl,UETX-φUERX0ri,k+m,l* ni,k+m+ri,k+m,l* ni,k+mri,k,lni,k*

from which the expected value of the argument of wi,k+m,l wi,k,l* is given by

Eargwi,k+m,l wi,k,l*=m Δφl,UETX

and the phase offset per symbol of the l-th TRP transmitter relative to the UE receiver is given by

Eargwi,k+m,l wi,k,l*m=Δφl,UETX .

More than two DL-PRS reference symbols can be used to estimate the frequency offset and these DL-PRS reference symbols can be in different DL-PRS slots.  Additionally, the frequency offset can be estimated for each subcarrier, and these estimates can be averaged since the carrier frequency offset is independent of the subcarrier. As a result, the carrier frequency offset can be averaged across both frequency and time.

To enable the LMF to correct the carrier phase difference measurement errors due to carrier frequency offsets, the carrier frequency offsets must be measured for each TRP at both the UE and the PRU, and these measurements must be reported to the LMF.

If the UE and the PRU report carrier phase measurements, then the LMF must know the number of symbols separating the DL-PRS for each pair of TRP’s for which the carrier phase measurements were taken in order to remove the measurement error when taking the phase differences.  Similarly, if the UE and the PRU report carrier phase difference measurements, the LMF must know the number of symbols separating the DL-PRS used to generate the carrier phase difference measurement.  If the UE and the PRU average carrier phase difference measurements across multiple pairs of DL-PRS symbols, then each of the DL-PRS pairs must have the same number of symbols separating the DL-PRS symbol used for the reference TRP and the DL-PRS symbol used for the target TRP.

Proposal 1:	To enable the LMF to correct carrier phase difference measurement errors due to carrier frequency offsets, the carrier frequency offset should be measured by both the UE and the PRU for each TRP for which measurements are taken, and these carrier frequency offsets should be reported to the LMF.

Proposal 2:	If carrier phase measurements are reported by the UE and the PRU, the number of symbols separating the measured DL-PRS for any two TRP’s must be reported to the LMF.

Proposal 3:	If carrier phase difference measurements are reported by the UE and the PRU, the number of symbols separating the measured DL-PRS used for the carrier phase difference measurement must be reported to the LMF.

- Summary

Carrier frequency offsets between the TRP transmitters and the UE and PRU receivers can results in large phase measurement errors which depend on:

- The carrier frequency offsets between the TRP transmitters and the UE and PRU receivers;

- The time interval between the DL-PRS symbols being used to measure the phase difference.

Observation 1: Unless the carrier frequency offset of the TRP relative to the UE and the DL-PRS is the same, the carrier frequency offset can result in very substantial errors in the measurement of carrier phase difference.

Observation 2: The carrier phase difference measurement errors resulting from carrier frequency offset are a function of the difference in the carrier frequency offsets of the TRP relative to the UE and the PRU, and the separation in time of the DL-PRS symbols from the two TRP’s.

Observation 3: The expected value of the carrier phase measurement error can be computed as

n Δφm,UETX- Δφm,PRUTX

where Δφm,UETX and Δφm,UETX denote the phase rotation over one symbol of the of the carrier of the m-th TRP relative to the UE and the PRU, respectively, and n is the difference in the symbol indices of the two DL-PRS symbols.

Proposal 1:	To enable the LMF to correct carrier phase difference measurement errors due to carrier frequency offsets, the carrier frequency offset should be measured by both the UE and the PRU for each TRP for which measurements are taken, and these carrier frequency offsets should be reported to the LMF.

Proposal 2:	If carrier phase measurements are reported by the UE and the PRU, the number of symbols separating the measured DL-PRS for any two TRP’s must be reported to the LMF.

Proposal 3:	If carrier phase difference measurements are reported by the UE and the PRU, the number of symbols separating measured DL-PRS used for the carrier phase difference measurement must be reported to the LMF.

# References

- R1-2204807, “Design Aspects of Carrier Phase Measurements for NR Positioning”, Intel, #109-e.

- 3GPP TS 38.104 NR; Base Station (BS) radio transmission and reception

- 3GPP TS 38.101-1 User Equipment radio transmission and reception; Part 1: Range1 Standalone