3GPP TSG-RAN WG4 # 108	R4-2311994

Toulouse, France, August 21 – August 25, 2023

Agenda item:	8.7.4.2

Source:	MediaTek Inc.

Title:	Simulation results of PDSCH requirements of FR2 Multi-Rx DL Demod

Document for:	Discussion

# Introduction

In RAN4#107, the Way Forward document was approved [1]. We provide some simulation results of PDSCH requirements for simulation alignment for the Rel-18 work in this contribution.

# Simulation results

In this chapter we provide some simulation results of PDSCH requirements for simulation alignment.

We have simulated only Multi-DCI transmission schemes for this document including both fully overlapping and non-overlapping modes. Correlation matrix follow agreement from the previous meeting [1]. MCS13 is used in all simulations. Receiver is using joint processing assumption in all simulations.

First, in Figure 1 there are simulation curves of Multi-TRP Multi-DCI fully overlapping scheme with Rank 1. Next, in Figure 2 there are simulation curves of Multi-TRP Multi-DCI fully overlapping scheme with Rank 2. Next, in Figure 3 there are simulation curves of Multi-TRP Multi-DCI non-fully overlapping scheme with Rank 2. Finally, all required SNRs to achieve 70% of maximum throughput are collected into summary Table 1. The simulated cross-talk power ratios ρ are 0dB, -3dB, -6dB, -9dB, -12dB and -15dB. Here, ρ in [dB] is defined as 10·log10(ρ).

Figure 1: Multi-TRP Multi-DCI fully overlapping Rank 1 with different cross-talk power ratios.

Figure 2: Multi-TRP Multi-DCI fully overlapping Rank 2 with different cross-talk power ratios.

Figure 3: Multi-TRP Multi-DCI non-overlapping Rank 2 with different cross-talk power ratios.

|  | Multi-DCI fully overlapping Rank 1 | Multi-DCI fully overlapping Rank 2 | Multi-DCI non overlapping Rank 2 |
| --- | --- | --- | --- |
| Cross-talk power -15 dB | 5.5 dB | 12.8 dB | 11.1 dB |
| Cross-talk power -12 dB | 5.6 dB | 13.1 dB | 10.7 dB |
| Cross-talk power -9 dB | 5.7 dB | 13.4 dB | 10.1 dB |
| Cross-talk power -6 dB | 5.9 dB | 13.8 dB | 9.6 dB |
| Cross-talk power -3 dB | 6.1 dB | 14.0 dB | 9.1 dB |
| Cross-talk power 0 dB | 6.1 dB | 14.1 dB | 9.0 dB |

Table 1: Simulation result collection showing SNR needed to achieve 70% of maximum throughput.

From simulation results collection in Table 1, we can do following observations:

Observation #1: In Multi-DCI fully overlapping scheme higher cross-talk increases inter-layer interference.

Observation #2: In Multi-DCI non-overlapping scheme higher cross-talk increases diversity gain.

# Conclusion

In this paper we provided simulation results on the PDSCH requirements of FR2 multipanel RX downlink demodulation requirements for performance alignment. The following observations and proposals are made:

Observation #1: In Multi-DCI fully overlapping scheme higher cross-talk increases inter-layer interference.

Observation #2: In Multi-DCI non-overlapping scheme higher cross-talk increases diversity gain.

# Reference

- R4-2309824, “WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception”, Qualcomm