3GPP TSG-RAN WG4 Meeting #108	R4-2312007

Toulouse, France, August 21 – August 25, 2023

Agenda Item:	8.9.2.3

Source:	Ericsson

Title:	Discussion on NCSG and Con-MGs

Document for:	Discussion

# Introduction

In last meetings, RAN4 discussed the combination of NCSG and Con-MGs [1]. Some terminology for gap were used as follow.

Issue 2-2: Definitions: legacy, concurrent, baseline and component gaps < Agreement >: - Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17

In this contribution, we will continue to discuss the related issues for NCSG and ConMGs with the agreed terminology.

# Rel-17 MG rule enhancement

Gap association rule

When UE capable of NCSG reports the NCSG indication for each band, the NW can configure a NCSG and a Type-2 MG. The MOs in the bands which UE indicates ‘no gap with interruption’ can be configured with the association of NCSG and the MOs in the bands which UE indicates ‘gap’ can be configured with the association of Type-2 MG. The other MOs in the bands which UE indicates ‘no gap no interruption’ and the MOs related to gapless measurement capability (inter-frequency without gap in Rel-16) will be measured outside the gap. Thus, when NW configures a NCSG and a Type-2 MG, RAN4 also needs to consider each frequency layers’ association among these three sets: NCSG, Type-2 MG and outside gap.

Issue 4-4-1: [Case 2] Potential changes for NCSG upon SCell activation < Agreements from GTW >: - UE behavior for deactivated SCell measurements with NCSG in Case 2 is FFS - Option 1: Legacy UE behavior (i.e. UE measures the deactivated SCell outside of MG) - Option 2: When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped. < Agreements from online session >: - Option 1: - UE measures the deactivated SCell outside of MG - Option 2: - When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped with NCSG. - FFS whether a new indication shall be introduced enable support of NCSG for deactivated SCell only. < Agreement >: - Align the understanding of Rel-17 UE behaviours - Only up to 1 NCSG can be configured. All activated Scell MOs are implicitly associated to the NCSG - In the dynamic UE capability signalling, there is no separate indication for activated/deactivated serving cells. This implies UE only indicate the capability if it supports all scenarios, including - deactivated Scell - activated Scell but SSB not in active BWP - Understanding to be clarified: - Will all deactivated Scell be measured via NCSG regardless the UE capability report of intraFreq-needForNCSG? - New in Rel-18 - When Type-2 MG and NCSG are both configured, some serving cell MOs may associated to the NCSG and some are not. - Question 1: What is the expected UE behaviour (assume SMTC partially overlapped with NCSG) - Option 1: skip gap association, all deactivated Scells are measured within NCSG. (This implies some new rule to override the existing gap association rule) - Option 2: Still follow the gap association, i.e., (This implies we follow Rel-17 gap association rule) - Deactivated Scell MO associated with NCSG is measured within NCSG - Deactivated Scell MO not associated with NCSG is measured outside NCSG - Question 2: Whether additional UE capability indication is needed

In our understanding, this issue is about further clarify UE’s association other than an optimization. In Rel-17, deactivated SCell is agreed to be measured within NCSG instead of outside gap if NW configures NCSG. However, when NW activates the SCell, the related SSB may not be measured if the SSB is outside active BWP when UE reports ‘gap’ for the band.

In Rel-18, when UE supports both MG and NCSG, deactivated SCell can be measured within NCSG. After SCell activation, the MO may further decide to be measured within gap or NCSG or outside gap depends on UE’s band reporting and BWP status.

- If UE reports ‘ncsg’ for the band, and

  - if the SSB is within the active BWP, the MO will be measured outside MG and NCSG;

  - if the SSB is outside the active BWP, the MO will be measured within NCSG.

  - in this case, the MO can be associated within the NCSG.

- If UE reports ‘gap’ for the band, and

  - if the SSB is within the active BWP, the MO will be measured outside MG and NCSG;

  - if the SSB is outside the active BWP, the MO will be measured within MG.

  - in this case, the MO can be associated within the Type-2 MG.

Figure 1. The transition from NCSG to MG for a MO with deactivated SCell

Observation 1: In Rel-17, when UE supports NCSG, deactivated SCell measurement will be performed within NCSG.

Proposal 1: When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped regardless of gap association.

# Conclusion

In this contribution, we continue discussing the remaining issue for NCSG and Con-MGs. The observations and proposals are as follow.

Observation 1: In Rel-17, when UE supports NCSG, deactivated SCell measurement will be performed within NCSG.

Proposal 1: When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped regardless of gap association.

# Reference

- R4-2310175, “WF on further enhancements on measurement gaps and measurements without gaps”, MTK