3GPP TSG-RAN WG4 Meeting #108	R4-2312013

Toulouse, France, August 21 – August 25, 2023

Agenda Item:	5.2.3

Source:	Ericsson

Title:	Discussion on remaining issues of Rel-17 MG enh.

Document for:	Discussion

# Introduction

Some of the Rel-17 WI were closed in previous meetings. However, in RAN4 #107 meeting, some remaining issues were identified in WF[1].

In this paper, we will discuss the remaining issues of Rel-17 MG enhancements.

# The remaining issues of NCSG

In RAN4 #107 meeting, some remaining issues for NCSG were odentified as follow.

Sub-topic 4-4: Other Rel-17 rules to be revisited Issue 4-4-1: [Case 2] Potential changes for NCSG upon SCell activation < Agreements from meeting RAN4#106-bis-e >: - UE behavior for deactivated SCell measurements with NCSG in Case 2 is FFS - Option 1: Legacy UE behavior (i.e. UE measures the deactivated SCell outside of MG) - Option 2: When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped. < Agreements from online session >: - Option 1: - UE measures the deactivated SCell outside of MG - Option 2: - When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped with NCSG. - FFS whether a new indication shall be introduced enable support of NCSG for deactivated SCell only. < Agreement >: - Align the understanding of Rel-17 UE behaviours - Only up to 1 NCSG can be configured. All activated Scell MOs are implicitly associated to the NCSG - In the dynamic UE capability signalling, there is no separate indication for activated/deactivated serving cells. This implies UE only indicate the capability if it supports all scenarios, including - deactivated Scell - activated Scell but SSB not in active BWP - Understanding to be clarified: - Will all deactivated Scell be measured via NCSG regardless the UE capability report of intraFreq-needForNCSG?

One of the main purposes to introduce the NCSG is to control the interruption locations for deactivated SCell measurement. In Rel-15, UE will perform deactivated SCell measurement outside gaps with interruption. In Rel-17, RAN4 agrees to introduce NCSG. One of the use cases is to handle the deactivated SCell measurement. Deactivated SCell measurement is different with other inter-frequency measurement since no additional RF chain is needed. When UE supports NCSG capability which means UE can support the NCSG patterns. The intention of intraFreq-needForNCSG will be used to indicate whether the intra-frequency with SSB outside active BWP can be measured within NCSG other than MG. Thus, UE can perform deactivated SCell measurement within NCSG without any further band reporting.

Proposal 1: In Rel-17, when UE supports NCSG capability, all deactivated SCell will be measured within NCSG regardless of further UE capability reporting of intraFreq-needForNCSG.

# Summary and Conclusion

In the contribution, we discuss the remaining issues for Rel-17 MG enh with the following proposals:

Proposal 1: In Rel-17, when UE supports NCSG capability, all deactivated SCell will be measured within NCSG regardless of further UE capability reporting of intraFreq-needForNCSG.

# References

- R4-2310175, “WF on further enhancements on measurement gaps and measurements without gaps”, MTK