# 3GPP TSG- RAN WG4 Meeting # 108	R4-2312021

# Toulouse, France, 21th Aug–25th Aug, 2023

# Source: 	ZTE Corporation

# Title: 	Discussion on UE capability

# Agenda item:	8.5.3

# Document for:	Approval

# 1 Introduction

Since the study item “Study on efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths” raised long-standing RAN4 discussions on the location of the channel bandwidth on the (100 kHz) channel raster entries and did not reach an agreement, the revised work item [1] on channel raster enhancement was agreed. In this contribution, we want to share some views on UE and BS channel raster enhancement.

The objectives of the work item are the following: - Specify necessary changes to the UE channel raster such that configuring a narrower UE channel BW inside a wider gNB channel BW is always possible [RAN4]. - Changes to BS channel raster can be considered if required [RAN4]. - Specify the corresponding UE capability, if needed, to enable changes to the channel raster [RAN2, RAN4]: - RAN4 is to identify the release(s) of the core specifications and the possibility of early implementation. If corresponding capability signalling is provided for early implementation and such early implementation is possible, the change is to be release independent from the identified release. NOTE: Changes to channel raster need to be compliant with the definition of global channel raster in RAN4 specification.

# 2 Discussion

- 3- UE capability Open issues (to be further discussed once RAN4 has agreed on one of the alternatives): FFS whether/how a new UE capability would be specified to support the WI objectives FFS if UE capability should be per band or per UE. FFS from which release should the UE capability be applicable.

- As discussed in [2], to resolve the issue of locating a UE specific channel bandwidth with even/odd PRBs within a wider SIB1 carrierBandwidth with odd/even PRBs, a new UE capability of allowing UE specific channel bandwidth off 100 kHz channel raster should be introduced. Whether one band needs to do channel raster enhancement is unclear, there is no reason to require the new capability supported by some bands, and it’s better that new UE capability is applicable to per UE.

- Proposal 1: Whether one band needs to do channel raster enhancement is unclear, there is no reason to require the new capability supported by some bands, and it’s better that new UE capability is applicable to per UE.

- As specified in WID, the change is to be release independent from the identified release. To increase configuration flexibility and avoid NBC issue, this UE capability shall be optional in R17. It’s better to be mandatory from Rel-18.

Proposal 2: To increase configuration flexibility and avoid NBC issue, this UE capability shall be optional in R17. It’s better to be mandatory from Rel-18.

# 3 Conclusion

In this contribution, we shared some views on UE capability and the observations and proposals are made as following:

- Proposal 1: Whether one band needs to do channel raster enhancement is unclear, there is no reason to require the new capability supported by some bands, it’s better that new UE capability is applicable to per UE.

Proposal 2: To increase configuration flexibility and avoid NBC issue, this UE capability shall be optional in R17. It’s better to be mandatory from Rel-18.

# 4 Reference

- RP-231469, “Revised WID: Channel raster enhancement”, Ericsson

- R4-2307847, “Discussion on UE and BS channel raster”, ZTE Corporation