# 3GPP TSG- RAN WG4 Meeting # 108	R4-2312023

# Toulouse, France, 21th Aug–25th Aug, 2023

# Source:	ZTE Corporation

# Title:	Discussion on band definition for 900 MHz NR Band

# Agenda Item:	7.33.2

# Document for:	Approval

- Introduction

During RAN#99-e meeting, a revised Work Item [1] has been approved on introduction of 900 MHz NR Band in the US. In this contribution, we want to share some initial views on band definition.

- Specify a new 5G NR FDD operating band operating with

  - UL: 896 – 901 MHz and DL 935 – 940 MHz.

  - To support 3 MHz channel bandwidths with 15 kHz SCS.

    - NOTE: 3MHz CBW will be added once it is specified in Rel-18 NR_FR1_lessthan_5MHz_BW.

- Study and specify the co-existence requirements

  - Support FCC OOB emission mask in FCC-20-67A1 for all the defined channel bandwidths in the proposed band to ensure co-existence with adjacent narrowband and broadband networks to avoid interference.

  - Assume use of 3GPP general SEM or FCC SEM and study whether additional coexistence protection is needed for Band 5.

  - Assume reusing band B8 duplexer

- Specify UE RF core requirements to operate in North America with primary operations in the US.

  - Assume power class 3 (23 dBm).

- Specify necessary BS RF core requirement.

- Specify the corresponding RRM requirements, i.e., add the new band for frequency bands grouping.

- Discussion

  - Operating band and band numbering

Band plan Way forward: - Band number 106 and n106 are reserved for this band for LTE and NR, respectively

Above was the agreement that band number n106 reserved for this band for NR [2]. As indicated in the objective, a new 5G NR operating band should be defined as following.

Table 2.1-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n106 | 896 MHz – 901 MHz | 935 MHz – 940 MHz | FDD |

Proposal 1: To define the new NR FDD band as in Table 2.1-1.

  - Maximum transmission bandwidth configuration

For the new NR FDD operating band, as stated in the Work Item, it support 3 MHz channel bandwidths with 15 kHz SCS. The transmission bandwidth configuration NRB for 3 MHz channel bandwidth should follow the requirement which is defined in Rel-18 NR_FR1_lessthan_5MHz_BW [3].

Table 2.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 3MHz |
| --- | --- |
|  | NRB |
| 15 | 15 |

Proposal 2: For band n106, it supports 3MHz channel bandwidth with 15 kHz SCS.

Proposal 3: For band n106, to define the transmission bandwidth configuration NRB = 15 for 3 MHz CBW.

  - Minimum guardband

The minimum guardband for 3 MHz channel bandwidth has been defined in Rel-18 NR_FR1_lessthan_5MHz_BW, which is as follows.

Table 2.3-1: Minimum guardband for each UE channel bandwidth and SCS (kHz)

| SCS (kHz) | 3MHz |
| --- | --- |
| 15 | 142.5 |

Proposal 4: For band n106, it is proposed to define the minimum guardband as 142.5 kHz for 3 MHz CBW.

  - Channel raster and sync raster

For NR-ARFCN, we propose to use the similar descending manner to define as the following table:

Table 2.4-1: NR-ARFCN

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n106 | 100 | 179200 – <20> – 180200 | 187000 – <20> – 188000 |
| NOTE 5: In the present version of the specification, only NR-ARFCN 179800 and 187600 is applicable for 3 MHz channel bandwidth. |  |  |  |

Proposal 5: For NR-ARFCN, it is proposed to define as Table 2.4-1.

Issue 1-1: The value of A in N * 600kHz + M * 50 kHz + A kHz, N ϵ {1:2499}, M ϵ {1,3,5} - Proposals - Option 1: 300 - Option 2: 345 - Agreement - Agree on 300. Issue 1-6: Finer synchronization raster index for 3 MHz channel bandwidth - Proposals - Option 1: GSCN = 22255 + 3*N + (M-1)/2, N=1:2499, M = {1,3,5} - Option 2: NREF = (N * 600 + M * 50 + 345)/5, N=1534:1538, M ϵ {1,3,5} - Agreement - TBD based on finer synchronization raster design.

Above was the agreement of sync raster design and index for 3 MHz channel bandwidth in Rel-18 NR_FR1_lessthan_5MHz_BW. For band n106, it is proposed to follow the sync raster design for 3 MHz CBW and wait for the conclusion of sync raster index in NR_FR1_lessthan_5MHz_BW. It should be noted that the range of N shall be 1 to 4998 instead of 1 to 2499.

Proposal 6: For band n106, it is proposed to follow the sync raster design of N * 600kHz + M * 50 kHz + 300 kHz, N ϵ {1:4998}, M ϵ {1,3,5} for 3 MHz CBW and wait for the conclusion of sync raster index in NR_FR1_lessthan_5MHz_BW.

  - TX–RX frequency separation

For TX–RX frequency separation, we propose to define as follows:

Table 2.5-1: UE TX-RX frequency separation

| NR Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n106 | 39 MHz |

Proposal 7: For TX–RX frequency separation, it is proposed to define as Table 2.5-1.

- Conclusion

In this contribution, we give some initial discussion on band definition for 900 MHz NR band and the proposals are made as following:

Proposal 1: To define the new NR FDD band as in Table 2.1-1.

Proposal 2: For band n106, it supports 3MHz channel bandwidth with 15 kHz SCS.

Proposal 3: For band n106, to define the transmission bandwidth configuration NRB = 15 for 3 MHz CBW.

Proposal 4: For band n106, it is proposed to define the minimum guardband as 142.5 kHz for 3 MHz CBW.

Proposal 5: For NR-ARFCN, it is proposed to define as Table 2.4-1.

Proposal 6: For band n106, it is proposed to follow the sync raster design of N * 600kHz + M * 50 kHz + 300 kHz, N ϵ {1:2499}, M ϵ {1,3,5} for 3 MHz CBW and wait for the conclusion of sync raster index in NR_FR1_lessthan_5MHz_BW.

Proposal 7: For TX–RX frequency separation, it is proposed to define as Table 2.5-1.

- Reference

RP-230723, Revised WID: Introduction of 900 MHz NR Band in the US, Anterix

R4-2303486, WF on US 900MHz bands, Qualcomm

R4-2310407, draftCR to 38.101-1: Introduction of dedicated spectrum less than 5MHz for FR1, Nokia, Ericsson, ZTE Corporation, Apple, Qualcomm Inc, Huawei, Mediatek