# 3GPP TSG- RAN WG4 Meeting # 108	R4-2312024

# Toulouse, France, 21th Aug–25th Aug, 2023

# Source:	ZTE Corporation

# Title:	Discussion on UE RF requirements for 900 MHz NR band

# Agenda Item:	7.33.3

# Document for:	Approval

- Introduction

During RAN#99-e meeting, a revised Work Item [1] has been approved on introduction of 900 MHz NR Band in the US. In this contribution, we want to share some initial views on band definition.

- Specify a new 5G NR FDD operating band operating with

  - UL: 896 – 901 MHz and DL 935 – 940 MHz.

  - To support 3 MHz channel bandwidths with 15 kHz SCS.

    - NOTE: 3MHz CBW will be added once it is specified in Rel-18 NR_FR1_lessthan_5MHz_BW.

- Study and specify the co-existence requirements

  - Support FCC OOB emission mask in FCC-20-67A1 for all the defined channel bandwidths in the proposed band to ensure co-existence with adjacent narrowband and broadband networks to avoid interference.

  - Assume use of 3GPP general SEM or FCC SEM and study whether additional coexistence protection is needed for Band 5.

  - Assume reusing band B8 duplexer

- Specify UE RF core requirements to operate in North America with primary operations in the US.

  - Assume power class 3 (23 dBm).

- Specify necessary BS RF core requirement.

- Specify the corresponding RRM requirements, i.e., add the new band for frequency bands grouping.

- Discussion

  - Maximum output power

For the new NR FDD operating band, as the objective stated, it assumes power class 3 (23 dBm).

Table 2.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n106 |  |  |  |  |  |  | 23 | ±2 |

Proposal 1: For band n106, to define UE power class as Table 2.1-1.

  - Reference sensitivity

If we reuse band B8 duplexer, we can follow the reference sensitivity requirement for B8 defined in 36.101.

Table 2.2-1: Reference sensitivity

| SCS (kHz) | 3MHz(dBm) |
| --- | --- |
| 15 | -99.2 |

Proposal 2: For band n106, it is proposed to reuse the reference sensitivity requirement for B8 defined in 36.101 and define the reference sensitivity as -99.2 dBm for 3MHz CBW.

  - Summary

In the following Table 2.3-1, we share some initial views on the UE RF requirements for band n106 and details could be found as the following table:

Table 2.3-1: Summary of the proposals for UE RF requirement for band n106

| NR UE Tx/Rx requirement | Proposal |
| --- | --- |
| 6.2.1 UE maximum output power | 23 dBm maximum output power (i.e. PC3) should be needed for the band n106. |
| 6.2.2 MPR | No specification impact once the 3MHz CBW has been introduced into the spec in the WI NR_FR1_lessthan_5MHz_BW. |
| 6.2.3 A-MPR | - No A-MPR. |
| 6.2.4 Configured transmitted power | No specification impact. |
| 6.3.1 Minimum output power | No specification impact once the 3MHz CBW has been introduced into the spec in the WI NR_FR1_lessthan_5MHz_BW. |
| 6.3.2 Transmit OFF power | No specification impact once the 3MHz CBW has been introduced into the spec in the WI NR_FR1_lessthan_5MHz_BW. |
| 6.3.3 Transmit ON/OFF time mask | No specification impact. |
| 6.3.4 Power control | No specification impact. |
| 6.4.1 Frequency error | No specification impact. |
| 6.4.2.1 EVM | No specification impact. |
| 6.4.2.2 Carrier leakage | No specification impact. |
| 6.4.2.3 In-band emissions | No specification impact. |
| 6.4.2.4 EVM equalizer spectrum flatness | No specification impact. |
| 6.5.1 Occupied bandwidth | No specification impact once the 3MHz CBW has been introduced into the spec in the WI NR_FR1_lessthan_5MHz_BW. |
| 6.5.2.2 Spectrum emission mask | No specification impact once the 3MHz CBW has been introduced into the spec in the WI NR_FR1_lessthan_5MHz_BW. |
| 6.5.2.3 Additional Spectrum emission mask | No specification impact. |
| 6.5.2.4.1 NR ACLR | No specification impact once the 3MHz CBW has been introduced into the spec in the WI NR_FR1_lessthan_5MHz_BW. |
| 6.5.2.4.2 UTRA ACLR | No specification impact. |
| 6.5.3.1 General spurious emissions | Boundary between E-UTRA out of band and spurious emission domain is defined generally --> no specification impact |
| 6.5.3.2 Spurious emissions for UE co-existence | An emission limit of -30 dBm/MHz is specified for UE coexistence to protect the downlinks of band n5 and n26 when the 3 MHz NR channel is centered at 899 MHz. |
| 6.5.3.3 Additional spurious emissions | No specification impact. |
| 6.5.4 Transmit intermodulation | Channel bandwidth agnostic --> no specification impact |
| 7.3.2 Reference sensitivity | To reuse 3MHz REFSENS for Band 8. |
| 7.4 Max input level | No specification impact once the 3MHz CBW has been introduced into the spec in the WI NR_FR1_lessthan_5MHz_BW. |
| 7.5 Adjacent channel selectivity | No specification impact once the 3MHz CBW has been introduced into the spec in the WI NR_FR1_lessthan_5MHz_BW. |
| 7.6.2 In-band blocking | Once the 3MHz CBW has been introduced into the spec in the WI NR_FR1_lessthan_5MHz_BW, band n106 needs to added into Table 7.6.2.1-2. |
| 7.6.3 Out of band blocking | Once the 3MHz CBW has been introduced into the spec in the WI NR_FR1_lessthan_5MHz_BW, band n106 needs to added into Table 7.6.3.1-2. |
| 7.6.4 Narrow band blocking | Once the 3MHz CBW has been introduced into the spec in the WI NR_FR1_lessthan_5MHz_BW, band n106 needs to added into Table 7.6.4-1. |
| 7.7 Spurious response | No specification impact once the 3MHz CBW has been introduced into the spec in the WI NR_FR1_lessthan_5MHz_BW. |
| 7.8 Intermodulation characteristics | No specification impact once the 3MHz CBW has been introduced into the spec in the WI NR_FR1_lessthan_5MHz_BW. |
| 7.9 Spurious emissions | Channel bandwidth agnostic --> no specification impact |

- Conclusion

In this contribution, we give some initial discussion on UE RF requirements for NR band n106 and the proposals are made as following:

Proposal 1: For band n106, to define UE power class as Table 2.1-1.

Proposal 2: For band n106, it is proposed to reuse the reference sensitivity requirement for B8 defined in 36.101 and define the reference sensitivity as -99.2 dBm for 3MHz CBW.

In addition, the summary of the corrections on TS38.101-1 are given in Table 2.3-1.

- Reference

RP-230723, Revised WID: Introduction of 900 MHz NR Band in the US, Anterix