# 3GPP TSG- RAN WG4 Meeting # 108	R4-2312025

# Toulouse, France, 21th Aug–25th Aug, 2023

# Source:	ZTE Corporation

# Title:	Discussion on BS RF requirements for 900 MHz NR Band

# Agenda Item:	7.33.4

# Document for:	Approval

- Introduction

During RAN#99-e meeting, a revised Work Item [1] has been approved on introduction of 900 MHz NR Band in the US. In this contribution, we want to share some initial views on band definition.

- Specify a new 5G NR FDD operating band operating with

  - UL: 896 – 901 MHz and DL 935 – 940 MHz.

  - To support 3 MHz channel bandwidths with 15 kHz SCS.

    - NOTE: 3MHz CBW will be added once it is specified in Rel-18 NR_FR1_lessthan_5MHz_BW.

- Study and specify the co-existence requirements

  - Support FCC OOB emission mask in FCC-20-67A1 for all the defined channel bandwidths in the proposed band to ensure co-existence with adjacent narrowband and broadband networks to avoid interference.

  - Assume use of 3GPP general SEM or FCC SEM and study whether additional coexistence protection is needed for Band 5.

  - Assume reusing band B8 duplexer

- Specify UE RF core requirements to operate in North America with primary operations in the US.

  - Assume power class 3 (23 dBm).

- Specify necessary BS RF core requirement.

- Specify the corresponding RRM requirements, i.e., add the new band for frequency bands grouping.

- Discussion

  - Summary

The summary of the corrections on TS 38.104 are given in Table 2.1.

Table 2.1. Summary of related RF requirement

| NR BS RF Tx/Rx requirement | Proposal |
| --- | --- |
| 6.2 Base station output power | No specification impact. |
| 6.3.2 RE power control dynamic range | No specification impact. |
| 6.3.3 Total power dynamic range | No specification impact once the 3MHz CBW has been introduced into the spec in the WI NR_FR1_lessthan_5MHz_BW. |
| 6.3.4 NB-IoT RB power dynamic range for NB-IoT operation in NR in-band | No specification impact |
| 6.4 Transmit ON/OFF power | No specification impact |
| 6.5 Transmitted signal quality | No specification impact |
| 6.6.2 Occupied bandwidth | No specification impact |
| 6.6.3 Adjacent Channel Leakage Power Ratio | No specification impact once the 3MHz CBW has been introduced into the spec in the WI NR_FR1_lessthan_5MHz_BW. |
| 6.6.4 Operating band unwanted emissions | Once the 3MHz CBW has been introduced into the spec in the WI NR_FR1_lessthan_5MHz_BW, band n106 needs to added. |
| 6.6.5 Transmitter spurious emissions | Band n106 needs to added in Table 6.6.5.2.3-1, Table 6.6.5.2.4-1. |
| 6.7 Transmitter intermodulation | No specification impact |
| 7.2 Reference sensitivity level | No specification impact once the 3MHz CBW has been introduced into the spec in the WI NR_FR1_lessthan_5MHz_BW. |
| 7.3 Dynamic range | No specification impact once the 3MHz CBW has been introduced into the spec in the WI NR_FR1_lessthan_5MHz_BW. |
| 7.4.1 Adjacent Channel Selectivity (ACS) | No specification impact once the 3MHz CBW has been introduced into the spec in the WI NR_FR1_lessthan_5MHz_BW. |
| 7.4.2 In-band blocking | No specification impact once the 3MHz CBW has been introduced into the spec in the WI NR_FR1_lessthan_5MHz_BW. |
| 7.5 Out-of-band blocking | No specification impact. |
| 7.6 Receiver spurious emissions | No specification impact. |
| 7.7 Receiver intermodulation | No specification impact once the 3MHz CBW has been introduced into the spec in the WI NR_FR1_lessthan_5MHz_BW. |
| 7.8 In-channel selectivity | No specification impact once the 3MHz CBW has been introduced into the spec in the WI NR_FR1_lessthan_5MHz_BW. |

- Conclusion

In this contribution, the summary of the corrections on TS 38.104 are given in Table 2.1

- Reference

RP-230723, Revised WID: Introduction of 900 MHz NR Band in the US, Anterix