3GPP TSG-RAN WG4 Meeting # 108											R4-2312032

Toulouse, France, August 21 – August 25, 2023

Agenda item:	7.31.3

Source: 	ZTE Corporation

Title: 	Discussion on UE RF requirements for the satellite L-/S-band

Document for:	Approval

Introduction

In RAN#98e meeting, the work item [RP-223485] on Introduction of the satellite L-/S-band was approved. Part of the objective is as follows:

-	Specify a new NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz;

-	Support channel bandwidths and SCS as presented in Table 4.1-1 below;

-	Support channel raster points at step of 100kHz;

NOTE: Support for channel raster points at step of 5kHz can be considered later based on the outcome of the corresponding RAN WG4 for potential channel raster enhancements.

-	Support UE Power class 3 (+23dBm);

NOTE: Work on UE Power class 2 can commence based on the RAN#99 decision.

-	Introduce the corresponding SAN and UE RF core requirements;

-	Introduce the corresponding RRM requirements.

Table 4.1-1: Channel bandwidth and SCS system parameters.

| SCS (kHz) | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- |
| 15 | 5 | 10 | 15 |  |
| 30 |  | 10 | 15 |  |
| 60 |  | 10 | 15 |  |

- In this contribution, we want to share some initial views on UE RF requirements for Introduction of the satellite L-/S-band.

Discussion

## 2.1. Asymmetric channel bandwidths

The introduction of asymmetric channel bandwidth combinations can reduce the complexity of UE implementation. If the asymmetric channel combinations 5+10MHz, 5+15MHz, and 10+15MHz UL/DL are necessary, we can support the introduction of asymmetric channel bandwidth which under the same principle defined in section 5.3.6 in TS 38.101-1.

Proposal 1: The asymmetric channel bandwidth combinations 5+10MHz, 5+15MHz, and 10+15MHz UL/DL can be introduced under the same principle defined in section 5.3.6 in TS 38.101-1.

## 2.2 Synchronization raster entries for each operating band

The detailed spectrum status between NTN band and surrounding TN bands are shown as following Figure 2.2-1.

Figure 2.2-1 NTN band and surrounding TN bands

Based on the above adjacent spectrum for the new NTN band, DL frequency range of band n254 is adjacent with band n53 and n41. In the current spec [2], band n41 and n53 already support both 15 kHz with Case A SS Block pattern and 30 kHz with Case C SS Block pattern as shown in Table 2.2-1. Technically, the applicable SS Block SCS and SS Block pattern should be determined based on the feedback by the operator. If there is not any feedback from operator in this agenda, we prefer that SS block pattern Case C should be added for flexibility.

Table 2.2-1: Applicable SS raster entries for band n41 and n53

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n41 | 15 kHz | Case A | 6246 – <3> – 6717 |
|  | 30 kHz | Case C | 6252 – <3> – 6714 |
| n53 | 15 kHz | Case A | 6215 – <1> – 6232 |
|  | 30 KHz | Case C | 6221 – <1> – 6226 |

Therefore, the synchronization raster for operating bands n254 is specified in Table2.2-2.

Table 2.2-2: Applicable SS raster entries for band n254

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n254 | 15 kHz | Case A | 6215 – <1> – 6244 |
|  | 30 kHz | Case C | 6218 – <1> – 6241 |
| NOTE : SS Block pattern is defined in clause 4.1 in 3GPP TS 38.213 [7]. |  |  |  |

Proposal 2: For the NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz, the synchronization raster of SS Block pattern Case C also should be added as Table 2.2-2.

## 2.3 TX–RX frequency separation

The default TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation for operating bands n254 can be specified as Table 2.3-1.

Table 2.3-1: UE TX-RX frequency separation

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n254 | 873.5 MHz |

Proposal 3: For the NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz, the TX–RX frequency separation can be specified as Table 2.3-1.

## 2.4 Reference sensitivity level

The following agreement has been approved at RAN4 #106bis:

Agreement： - Reference sensitivity: - n53 REFSENS + "insertion loss" dB; where potential options for the insertion loss are as follows - Option 1: 0dB (resulting in same REFSENS as band n53); - Option 2: 0.5dB (resulting in same REFSENS as band n255); - Other options are not excluded.

For the reference sensitivity level of band n254, it can be derived based on current band n256 or n53. Considering the TX-RX frequency separation of band n254 is lager than band n256 and new RF filter can be used for band n254, we prefer to reuse the reference sensitivity of band n53 to define the reference sensitivity of band n254. Therefore, Option 1 defined as Table 2.4-1 is our preference and we can also accept the option 2 which is same REFSENS as band n256.

Table 2.4-1: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n254 | 15 | -100.0 | -96.8 | -95.0 |  |  |  |  |  |  |  |
|  | 30 |  | -97.1 | -95.1 |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 |  |  |  |  |  |  |  |

Proposal 4: For the reference sensitivity of the new NTN FDD band, we prefer to option 1 which is same REFSENS as band n53.

## 2.5 Out-of-band blocking requirements

For out-of-band blocking requirements of n254, the general requirements for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz can be reused. Out-of-band blocking requirements for band n53 and band n255 also used this general requirements. Therefore, for band n254, the out-of-band blocking requirements can be specified in Table 2.5-1.

Table 2.5-1: Out of-band blocking for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n2542 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 lower frequency ranges are modified to enable specific implementations NOTE 2: Band n254 power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. NOTE 3: void NOTE 4: void |  |  |  |  |  |

Proposal 5: For the new NTN FDD band, the out-of-band blocking requirements can be specified in Table 2.5-1.

Conclusions

In this contribution, we want to share some further views on UE RF requirements for Introduction of the satellite L-/S-band. Some proposals are made as following:

Proposal 1: The asymmetric channel bandwidth combinations 5+10MHz, 5+15MHz, and 10+15MHz UL/DL can be introduced under the same principle defined in section 5.3.6 in TS 38.101-1.

Proposal 2: For the NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz, the synchronization raster of SS Block pattern Case C also should be added as Table 2.2-2.

Proposal 3: For the NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz, the TX–RX frequency separation can be specified as Table 2.3-1.

Proposal 4: For the reference sensitivity of the new NTN FDD band, we prefer to option 1 which is same REFSENS as band n53.

Proposal 5: For the new NTN FDD band, the out-of-band blocking requirements can be specified in Table 2.5-1.

References

- RP-223485, “New WID on Introduction of the satellite L-/S-ban”, Globalstar, Apple.

- TS 38.101-1, “User Equipment (UE) radio transmission and reception”