3GPP TSG-RAN WG4 Meeting # 108											R4-2312036

Toulouse, France, August 21 – August 25, 2023

Agenda item:	7.34.3

Source: 	ZTE Corporation

Title: 	Discussion on SAN RF requirement for NR NTN 30 MHz Channel Bandwidth

Document for:	Approval

Introduction

In RAN#100 meeting, the work item [RP-231499] on NR NTN 30 MHz Channel Bandwidth was approved. Part of the objective is as follows:

The objective of the core part is to:

- Define 30 MHz channel bandwidth for n256 and n255 defined for 5G NR-NTN;

- Introduce the corresponding SAN and UE RF core requirements, limited to clause 5 (Operating Bands and Channel Arrangement) requirements for these specifications.

The specified requirements will be introduced in a REL-independent way starting with REL-17

- In this contribution, we want to share some initial views on SAN RF requirements for NR NTN 30 MHz Channel Bandwidth.

Discussion

## 2.1. Transmission bandwidth configuration

The transmission bandwidth configuration NRB for each SAN channel bandwidth and subcarrier spacing can be specified as Table 2-1.

Table 2-1: Transmission bandwidth configuration NRB for FR1

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 160 |
| 30 | 11 | 24 | 38 | 51 | 78 |
| 60 | N/A | 11 | 18 | 24 | 38 |

Proposal 1: The transmission bandwidth configuration NRB for each SAN channel bandwidth and subcarrier spacing can be specified as Table 2-1.

## 2.2 Minimum guardband and transmission bandwidth configuration

The minimum guard band for each SAN channel bandwidth and SCS is specified in table 2-2 for FR1.

Table2-2: Minimum guard band (kHz) (FR1)

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz |
| --- | --- | --- | --- | --- | --- |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | 592.5 |
| 30 | 505 | 665 | 645 | 805 | 945 |
| 60 | N/A | 1010 | 990 | 1330 | 1290 |

Proposal 2: The minimum guard band for each SAN channel bandwidth and SCS is specified in table 2-2 for FR1.

## 2.3 SAN channel bandwidth per operating band

The SAN channel bandwidth of 30 MHz can be introduced for band n256 and n255 as Table 2-3.

Table 2-3: SAN channel bandwidths and SCS per operating band in FR1

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 30 |
|  | 15 | 5 | 10 | 15 | 20 | 30 |
| n256 | 30 |  | 10 | 15 | 20 | 30 |
|  | 60 |  | 10 | 15 | 20 | 30 |
|  | 15 | 5 | 10 | 15 | 20 | 30 |
| n255 | 30 |  | 10 | 15 | 20 | 30 |
|  | 60 |  | 10 | 15 | 20 | 30 |

Proposal 3: The SAN channel bandwidth of 30 MHz can be introduced for band n256 and n255 as Table 2-3.

Conclusions

In this contribution, we want to share some further views on SAN RF requirements for NR NTN 30 MHz Channel Bandwidth. Some proposals are made as following:

Proposal 1: The transmission bandwidth configuration NRB for each SAN channel bandwidth and subcarrier spacing can be specified as Table 2-1.

Proposal 2: The minimum guard band for each SAN channel bandwidth and SCS is specified in table 2-2 for FR1.

Proposal 3: The SAN channel bandwidth of 30 MHz can be introduced for band n256 and n255 as Table 2-3.

References

- RP-231499, “30 MHz Channel Bandwidth for NR NTN in FR1”, THALES.