3GPP TSG-RAN WG4 Meeting # 108											R4-2312040

Toulouse, France, August 21 – August 25, 2023

Agenda item:	9.4.3

Source: 	ZTE Corporation

Title: 	Discussion on UE RF requirements for the Extended L-band

Document for:	Approval

Introduction

In RAN#99 meeting, the work item [RP-230790] on introduction of the Extended L-band was approved. Part of the objective is as follows:

- Specify a new E-UTRA NTN FDD band, with initial scope for Region 1, with UE transmitting at 1668 – 1675 MHz and SAN transmitting at 1518 – 1525 MHz

  - NOTE: The introduction of this new NTN band through this WID has no impact on existing 3GPP terrestrial band specifications.

- Support UE Categories NB1, NB2, M1;

- Support UE Power class 3 (+23dBm) and Power Class 5 (+20dBm);

- Introduce the corresponding SAN and UE RF core requirements;

- Introduce the corresponding RRM requirements.

- In this contribution, we share some views on UE RF requirement for the Extended L-band.

Discussion

## 2.1. Operating bands

Based on the agreement numbered extended L-band and the analysis [2], the extended L-band can be specified as shown in Table 2.1-1.

Table 2.1-1: E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| [253] | 1668 MHz | – | 1675 MHz | 1518 MHz | – | 1525 MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from 256 |  |  |  |  |  |  |  |

Proposal 1: The Extended L-band should be defined as Table 2.1-1.

## 2.2 Channel raster, carrier frequency and EARFCN

The granularity of the global frequency raster and the relation between EARFCN and the carrier frequency of band 256/255 can be reused to the Extended L-band. Therefore, the channel raster, carrier frequency and EARFCN can be defined as Table 2.2-1.

Table 2.2-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| [253] | 100 | 1518 | 228501 | 228501-<1>-228570 | 1668 | 261269 | 261269-<1>-261338 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

Proposal 2: For the Extended L-band, the channel raster, carrier frequency and EARFCN can be defined as Table 2.2-1.

## 2.3 Reference sensitivity power level

The extended L-band is adjacent to the DL of NTN band 255 and the reference sensitivity power level REFSENS of band 255 can be reused to the extended L-band as shown in Table 2.3-1, Table and Table 2.3-2.

Table 2.3-1: Reference sensitivity for FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| [253] | -102.7 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5- in TS 36.101. |  |  |

Table 2.3-2: Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| [253] | -103.5 | HD-FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 in TS 36.101 [7]. |  |  |

Table 2.3-3: Reference sensitivity for UE category NB1 and NB2

| Operating band | REFSENS [dBm] |
| --- | --- |
| According to subclause 5.2B in TS 36.102 | - 108.2 |

Proposal 3: For the extended L-band, the reference sensitivity level for eMTC NTN can be defined as Table 2.3-1 and Table 2.3-2 and the reference sensitivity level for NB-IoT can be defined as Table 2.3-3.

## 2.4 Receiver blocking requirement

The receiver blocking requirement should be specified based on the conclusion of ECC report 263 [3].

Based on the final results of its compatibility studies, it is concluded that: the minimum in-band blocking characteristic for land mobile earth stations receivers from a 5 MHz broadband signal interferer (LTE) operating below 1518 MHz shall be −30dBm above 1520 MHz,

Proposal 4: For the extended L-band, the receiver blocking requirement should be specified based on the conclusion of ECC report 263.

Conclusions

In this contribution, we want to share some further views on UE RF requirements for Introduction of the satellite L-/S-band. Some proposals are made as following:

Proposal 1: The Extended L-band should be defined as Table 2.1-1.

Proposal 2: For the Extended L-band, the channel raster, carrier frequency and EARFCN can be defined as Table 2.2-1.

Proposal 3: For the extended L-band, the reference sensitivity level for eMTC NTN can be defined as Table 2.3-1 and Table 2.3-2 and the reference sensitivity level for NB-IoT can be defined as Table 2.3-3.

Proposal 4: For the extended L-band, the receiver blocking requirement should be specified based on the conclusion of ECC report 263.

References

- RP-230790, “New WID on Introduction of the Extended L-band (UL 1668-1675, DL 1518-1525) for IoT NTN”, Inmarsat

- R4-2312039, “Band definition and system parameters for Extended L-band”, ZTE

- ECC Report 263, “Adjacent band compatibility studies between IMT operating in the frequency band 1492-1518 MHz and the MSS operating in the frequency band 1518-1552 MHz,” Approved 03 March 2017.

- 3GPPP TR 38.863: “Non-terrestrial networks (NTN) related RF and co-existence aspects”.