3GPP TSG-RAN WG4 Meeting #108	R4-2312073

Toulouse, France, 21 – 25 August, 2023

Source:	ZTE Corporation

Title:	Discussion on the Interoperability and testability aspects of AI/ML

Agenda Item:	8.21.3

Document for:	Approval

- Introduction

In RAN plenary #94 meeting, a new SID[1] for Rel-18 was approved to study the AI/ML for NR air interface. The objectives for RAN4 of this SID are listed as follows.

- Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. - Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

In this document, we will provide some initial views on the interoperability and testability of AI/ML from the general test framework perspective.

- Discussion

Based on the progresses in the last meeting, there were several typical use cases incorporated in [2], which are listed below.

Agreement: - Following use cases and sub-use cases will be handled in RAN4: - CSI feedback enhancement - time domain CSI prediction - spatial-frequency domain CSI compression - Beam management - Spatial-domain DL beam prediction - Temporal DL beam prediction - Positioning accuracy enhancements - direct AI/ML positioning - AI/ML assisted positioning

In addition, the tentative framework of AI/ML is illustrated in Fig 1 during RAN1 discussion.

Fig 1: AI/ML framework

To facilitate discussion, RAN1 has agreed the following terminologies (the specific definition of terminologies is in the Annex):

- Data collection

- The process of collecting data for the purpose of AI/ML model training, data analytics and inference.

- Model training

- A process to train an AI/ML Model in a data driven manner and obtain the trained AI/ML Model for inference.

- Model inference

- A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs.

- Model management

- Including multiple functions in the LCM so as to manage the AI/ML model, such as model monitoring, model selection, model switch, model activation/deactivation and model fallback.

- From RAN4 perspective, the discussion should be held around the core requirements related to the functionality/model and the performance requirements so as to guarantee the availability of all the above essential components during the LCM.

- 2.1 The interoperability of AI/ML for NR air interface

- From RAN4 perspective, the interoperability of AI/ML for NR air interface is a kind of a new aspect. We would like to analyse the interoperability based on RAN1 progress and outcomes as below:

- RAN1 109e meeting: Agreement Take the following network-UE collaboration levels as one aspect for defining collaboration levels 1. Level x: No collaboration 2. Level y: Signaling-based collaboration without model transfer 3. Level z: Signaling-based collaboration with model transfer Note: Other aspect(s), for defining collaboration levels is not precluded and will be discussed in later meetings, e.g., with/without model updating, to support training/inference, for defining collaboration levels will be discussed in later meetings FFS: Clarification is needed for Level x-y boundary - RAN1 110bis-e meeting: Agreement Clarify Level x/y boundary as: - Level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signalling, RS) collaboration between network and UE. (Note: The AI/ML operation may rely on future specification not related to AI/ML collaboration. The AI/ML approaches can be used as baseline for performance evaluation for future releases.) Working Assumption - Define Level y-z boundary based on whether model delivery is transparent to 3gpp signaling over the air interface or not. - Note: other procedures than model transfer/delivery are decoupled with collaboration level y-z - Clarifying note: Level y includes cases without model delivery. - RAN1 112 meeting: Agreement To facilitate the discussion, consider at least the following Cases for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models. Case; Model delivery/transfer; Model storage location; Training location. y; model delivery (if needed) over-the-top; Outside 3gpp Network; UE-side / NW-side / neutral site. z1; model transfer in proprietary format; 3GPP Network; UE-side / neutral site. z2; model transfer in proprietary format; 3GPP Network; NW-side. z3; model transfer in open format; 3GPP Network; UE-side / neutral site. z4; model transfer in open format of a known model structure at UE; 3GPP Network; NW-side. z5; model transfer in open format of an unknown model structure at UE; 3GPP Network; NW-side. Note: The Case definition is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization. Note: The Case definition is NOT intended to introduce sub-levels of Level z. Note: Other cases may be included further upon interest from companies. FFS: Z4 and Z5 boundary

And in the last meeting RAN4 also has the suggestions on interoperability aspect as below:

2.3.6 Interoperability aspects RAN4 will further study interoperability aspects and how they are related to RAN4 aspects. The table in R4-2305199 can be taken as reference. Interested companies are invited to bring analysis on different issues such as relationship/need to consider interoperability and different network-UE collaboration levels.

- a、Collaboration level-x

- In RAN1 110bis-e and 109-e meeting we listed above, level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signaling, RS) collaboration between network and UE and no collaboration for level x, so we shall not consider the interoperability for level x.

- Observation 1: Level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signaling, RS) collaboration between network and UE and no collaboration for level x based on RAN1 outcomes.

- Proposal 1: RAN4 shall not study the interoperability aspect for level x based on previous meetings in RAN1.

- b、Collaboration level-y

- Based on RAN1 outcomes in 109-e and 112 meeting, for the level y collaboration, it is clarified as the signaling-based collaboration without model transfer, and the signaling-based collaboration exists between the UE and over-the-top which is outside 3GPP network. Compared to level x, level y is signaling-based NW-UE collaboration, we deem that more RAN2 progress is need which the 3GPP signaling for use case to enable or disable the AI/ML model function or feature.

- Observation 2: For the level y collaboration, it is clarified as the signaling-based collaboration without model transfer.

- Proposal 2: RAN4 needs to consider the interoperability for collaboration level y based on more RAN2 progress since it is the signaling-based collaboration.

- c、Collaboration level-z

- Based on RAN1 progress, two separate model format categories were identified, i.e. Proprietary-format models and Open-format models, The descriptions for these two modes are:

| Proprietary-format models | - ML models of vendor-/device-specific proprietary format, from 3GPP perspective - NOTE: An example is a device-specific binary executable format |
| --- | --- |
| Open-format models | ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspective |

- In terms of the above description, it can be assumed that the Proprietary-format models are not mutually recognizable across vendors, the model designs are vendor-specific, and usually it is invisible between UE/network vendors. So from the specification perspective, it is hard to realize inter-operation between different different vendors due to different models would be used for different vendors, which would cause incompatible problem. Even though there is not any specific definition of interoperability yet, it can be determined that due to lack of inter-operation between different vendors, it is hard to standardize based on the unified specification identification. So in our view, we need to study the feasibility and necessity of the Proprietary-format models for RAN4 core part and performance part discussion.

- Observation 3: Two categories of models including Proprietary-format models and Open-format models were proposed by RAN1. For the Proprietary-format models, due to the lack of inter-operation and recognition between vendors, it is hard to standardize based on the unified specification identification.

- While regarding the Open-format models, the models can be mutually recognizable across different vendors, and it can be visible between vendors when the model information is shared. So from the perspective of 3GPP, it is feasible to define the unified interface to guarantee the functionality. Of course from RAN4 perspective, the related core part and performance part should be discussed. However, the unified interaction format between vendors should be studied.

- Observation 4: Regarding to the Open-format models, the interoperability is feasible.

- Based on the above analysis, it seems that RAN4 can focus on the Open-format models firstly, and discuss which core part and performance part requirements should be identified and how to define. On the other side, the test framework and procedure should also be discussed. At the meanwhile, RAN4 needs to wait for RAN1 progress on Open-format models Since it may involve other working groups (RAN2, SA) and other proprietary information sharinf between different vendors. And we shall also consider what the structure of the open-format is and between different vendors shall have the common understanding.

Proposal 3: RAN4 can focus on the Open-format models firstly, and discuss which core part and performance part requirements should be identified and how to define. On the other side, the test framework and procedure should also be discussed. At the meanwhile, RAN4 needs to wait for RAN1 progress on Open-format models.

Proposal 4: RAN4 shall study the basic structure of the open-format and consider how the common understanding defined between different vendors.

- 2.2 The testability of AI/ML for NR air interface

When defining the core part and performance part requirements, the testability should also be considered. For AI/ML, since it is a module based enhancement which has a bit difference from legacy RRM procedure, and the details within the module would be algorithm implementations, so maybe some new types of test should be considered. We provide some analysis based on the general AI/ML framework below.

RAN1 has achieved some progress referring to the model/functionality identification as follows:

Agreement For UE-side models and UE-part of two-sided models: - For AI/ML functionality identification - Reuse legacy 3GPP framework of Features as a starting point for discussion. - UE indicates supported functionalities/functionality for a given sub-use-case. - UE capability reporting is taken as starting point. - For AI/ML model identification - Models are identified by model ID at the Network. UE indicates supported AI/ML models. - In functionality-based LCM - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI). - Models may not be identified at the Network, and UE may perform model-level LCM. - Study whether and how much awareness/interaction NW should have about model-level LCM - In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID.

No matter what the functionality-based LCM procedure or the model-ID-based LCM procedure is, the basic work procedure is still based on the UE capability reporting and NW signaling indication. We believe the differences are how to package the process/method, to package into model or functionality. The latter is more straightforward.  From the perspective of test, both functionality test and performance test should be considered.

Proposal 5: From the perspective of test, both functionality test and performance test should be considered.

For the functionality test, whether the module can realize the predicted function should be tested. Beside the functionality test, the performance test can verify the performance gain caused by the AI/ML framework compared with the traditional operation. Starting from the AI/ML framework, at least the function of the following modules should be verified:

- Data collection

- Model training

- Model inference

- Model monitoring

- Model transfer

- Model (de)activation

- Model update

- Model download

- Model switching

- In the last meeting we have several agreements approved in WF [2] as below:

LCM related functional tests: - RAN4 to investigate how to define performance requirements/tests for the following candidate procedures: - model/functionality monitoring - model/functionality selection - model/functionality activation/deactivation/switching/fallback - FFS whether data collection should be considered - FFS whether model update/transfer/delivery should be considered

Data collection

Regarding the verification of data collection, it provides dataset for model training, model inference and model management. Such dataset can be generated online during the test or be pre-generated before the test. For the former, the TE needs some time to generate such dataset during the test, so for such case, the requirement of dataset generation latency maybe needed. While for the case of pre-generated before the test, the availability of the pre-generated data may need to be verified, and whether such dataset availability verification can be replaced by the performance of model inference needs to be discussed. Furthermore, whether the dataset is scenario/use case specific or common for all scenario/use case needs also to be discussed since it would also impact the dataset availability verification. For the case of pre-generated dataset, the fairness and uniformity of dataset cross different vendors should be guaranteed, so maybe a unified data collection principle is necessary.

- Observation 5: Data collection provides dataset for multiple modules. Such dataset can be generated online during the test or be pre-generated before the test. From the perspective of test, at least the following aspects should be considered:

- Whether the latency of data collection needs to be verified;

- Whether and how to guarantee the fairness and uniformity of dataset cross different vendors;

- Whether the verification of dataset availability can be replaced by the performance of the model inference.

Model inference

Model inference is the core component of AI/ML. It is the process of using a trained AI/ML model to produce a set of outputs based on a set of inputs. For the test of model inference, the following aspects should be verified:

- The outputs are the results from the AI/ML inference model rather than the traditional solution.

- The accuracy of outputs meet the requirement.

- For the 1st aspect, maybe some new test setups and procedures are needed, which is different from the traditional test case design. For the 2nd aspect, the legacy requirement of accuracy can be referred, depend on the exact use case or scenario to be tested.

- Observation 6: model inference is the core component of AI/ML. Two aspects should be considered to verify: 1) The outputs are the results from the AI/ML inference model rather than the traditional solution; 2) The accuracy of outputs meet the requirement.

- Model monitoring(model transfer/(de)activation/update/switching)

- Regarding the model management, wherein the model monitoring and model switching are specially considered, since such two model operations would largely impact on the need of model update. The model update aims to provide timely model adapted to the change of inference requirement. However for different use cases or scenarios, the requirement of model update maybe different, so such test should be use case/scenario specific. To verify such model management operation, the performance gain between after and before the model update can be tested, so as to verify the necessary and reliability of model monitoring and model switching.

- More specifically (model monitoring) with RAN1’s AI/ML framework,

- Different UE capabilities maybe check it

- Check the deifferent companies’ proposals

- For model inference, model monitoring continuously monitors the output results of model discussions, combined with the specific functions of the corresponding use cases, and provides feedback to model discussions through model activation and model deactivation to selectively activate the corresponding models, thereby improving the efficiency of AI/ML.

- For model training, the model monitoring feedback is provided to the model training, which continuously monitors the performance of the model and provides feedback to the model training for retraining to determine an optimal model. This feedback process improves accuracy.

- Based on above analysis, we shall conclude that (model monitoring):

- Firstly, Model monitoring refers to the process of monitoring the performance of the model In particular this stage may contain inter-relations with RAN4 specifications.

- Secondly, model monitoring is a key module to ensure the inference performance of AI/ML models, as generalization is widely considered a problem with AI/ML operations. In short, different AI/ML models can be utilized to ensure performance under different environmental conditions.

- More specifically (model switching) with RAN1’s AI/ML framework,

- For example, UE needs to switch models when moving between different cells or environments may require model switching. If UE is responsible for switching to ensure the performance of the UE for decision-making and implementing switching between models algorithm. If RAN4 does not define its requirements, it may lead to unpredictable UE behavior.

- Another consideration for switching is that timing related requirements may be required; For example, the activation time or the time period during which sending or receiving is impossible. This requirement is crucial for ensuring that the network can manage UE correctly.

Observation 7: The model update operations such as model monitoring and model switching aim to provide timely model accompanied by the change of the inference requirement. To verify such model management operation, the performance gain between after and before the model update can be tested.

Proposal 6: Based on the analysis for model management especially for the model monitoring and model switching, RAN4 shall consider the specification impact for both of them more specifically.

Issue 3-3: Encoder/decoder for 2-sided model - Option 1: reference decoder is provided by the vendor of the encoder under test so that the encoder and decoder are jointly designed and trained - Option 2: reference decoder is provided by the vendor of the decoder(infra-vendors) so that the encoder and decoder are jointly designed and trained - Option 3: The reference decoder(s) are fully specified and captured in RAN4 spec to ensure identical implementation across equipment vendors without additional training procedure needed. - Option 4: The reference decoder(s) are partially specified and captured in RAN4 spec. - Option 6: Test decoder is specified and captured in RAN4 and is provided by test environment vendor. The encoder and decoder can be jointly trained. - Other options not precluded Companies are invited to bring further input on merits/de-merits/feasibility of Options 1- 4. Proponents of Option 6 should bring clarifications on how this option would be used to implement RAN4 tests.

Figure 2 the encoder and the reference decoder

In the last meeting, we discussed the encoder/decoder for 2-sided model, the pros and cons on each option:

For option 1: reference decoder is provided by the vendor of the encoder under test so that the encoder and decoder are jointly designed and trained

Pros:

- Mitigate the impact of the model mismatch in tests.

- Allow UE vendor to update the reference decoder in the future.

Cons:

- UE can obviously pass the tests.

- TE has to support all the reference decoders provided by different UE vendors.

- Impact of the model mismatch may happen in the field since network vendor would not use reference decoder.

- It is not clear how to define RAN4 requirement for models provided by different UE vendors.

For option 2: reference decoder is provided by the vendor of the decoder(infra-vendors) so that the encoder and decoder are jointly designed and trained

Pros:

- UE may not obviously pass the tests.

Cons:

- The encoder(s) of UE vendor has to match all the reference decoders provided by different network vendors.

- TE has to support all the reference decoders provided by different network vendors.

- It is not clear how UE vendors train their encoders.

- It is hard to align RAN4 requirement for different models provided by different network vendors.

For option 3: The reference decoder(s) are fully specified and captured in RAN4 spec to ensure identical implementation across equipment vendors without additional training procedure needed.

Pros:

- Easy to align the test requirement and easy to implement for TE vendors.

- If the reference decoder(s) is generated from the well-designed datasets, RAN4 test result can ensure the AI/ML model performance in the field.

Cons:

- The encoder passed the test may not work for the decoder in the field since the fully specified reference decoder may be mismatch with the decoder in the field. UE may have to implement an additional encoder only for the RAN4 test.

- Possible lengthy RAN4 discussion to agree on one (or more) fully specified reference decoder.

For option 4: The reference decoder(s) are partially specified and captured in RAN4 spec.

Pros:

- The issues regarding different models having different performance can be improved.

Cons:

- Different performance may be achieved by different reference decoders implemented by TE vendors.

For option 1 and option 2, they are similar to each other, the former one is the UE vendors for CSI compression use case and the latter one is gNB vendors for CSI compression use case. Both of them have boundedness and absoluteness. For option 1, the UE will obviously pass the test since the reference decoder is totally known to the vendor of the encoder, it has no benefit for the generalization, that is, it will perform the bad performance and the model mismatch may happen in the field since network vendor would not use reference decoder in the field, so the option 1 is not proper for us. For option 2, although the UE may not pass the test easily, the encoder(s) of UE vendor has to match all the reference decoders provided by different network vendors, this will cause the mismatch problems which will impact the performance not only in the test but in the field. For option 3 and option 4, the former one has less flexibility and may not perform well in the field, the latter one is lack of details when captured in RAN4 spec. Both of them will not perform well. For option 3, the reference decoder(s) are totally specified and captured in RAN4 spec, the encoder passed the test may not work for the decoder in the field since the fully specified reference decoder may be mismatch with the decoder in the field since it loses the flexibility and too absoluteness. For option 4, it is not clear that how the reference decoder is partially specified and captured in RAN4 spec. If RAN4 would like to chose anyone of them, not only between the UE, network and TE but in the field the bad performance will happen and it is not satisfy the request in the latest SID and also the limitation of TU is the key factor for our discussion.

Based on the above analysis, I deem that we shall study the one-sided model firstly in this stage also for for the limitation of TU since the performance for all options will not perform well not only in test but in the field.

Observation 8: The test environment is quite different from the field, so the assumptions is a little limitations and the generalization will be degraded.

Observation 9: The options may not work well, and the requirements based on these assumptions may not have significance.

Observation 10: The TU is limited in RAN4 for AI/ML.

Proposal 7: RAN4 shall study one-sided model firstly and the discussion of two-sided model shall be deprioritized since the two-sided model is only for CSI case.

The next issue is the test dataset generation, RAN4 has three options as below:

Test dataset generation should be studied. Different generating methods can be used for different tests. The following candidate methods are to be considered or down-selected: - Option a: Dataset based on TR 38.901, e.g. UMa channel, UMi channel, CDL channel, “legacy approach”, etc. - “Legacy approach” refers legacy test in which a channel model is used - Option b: Field dataset (data collected directly from field measurements) - Option c: TE generates dataset for test based on assumptions/parameters defined by RAN4 (e.g. by defining some rules/function to generate data) - Other methods are not precluded

- For option a,

- Currently most of simulation results of AI/ML models in RAN1 are provided based on the dataset 	generated by 3GPP channel models. 3GPP channel models have stable performance and sufficient 	physical meanings. It is convenient to generate large number of samples using 3GPP channel 	models.

- For option b, the problem of Option-b for field dataset (data collected directly from field measurements), because of the difficulty/cost to get field dataset which can be fully recognized by 3GPP. Unless a fully trustable 3rd party want to spend efforts to build up the field dataset, it is hard for 3GPP to follow this Option b for test data generation. And the next question how to guarantee the proposed field dataset is representative enough.

- For option c, compared to the option a, there is a few difference that the test dataset could be very large by considering all possible random variables which may be presented for the rule/function to generate data.

- Based on the stable performance and convenience, the option a can be the starting point to be studied in RAN4.

Observation 11: 3GPP channel models have stable performance and sufficient 	physical meanings. It is convenient to generate large number of samples using 3GPP channel 	models.

Proposal 8: In order to guarantee the stable performance and convenience, RAN4 shall study and use the dataset based on TR 38.901 firstly.

- Conclusion

In this contribution, we have the following observations and proposals for the AI/ML :

- Observation 1: Level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signaling, RS) collaboration between network and UE and no collaboration for level x based on RAN1 outcomes.

- Proposal 1: RAN4 shall not study the interoperability aspect for level x based on previous meetings in RAN1.

- Observation 2: For the level y collaboration, it is clarified as the signaling-based collaboration without model transfer.

- Proposal 2: RAN4 needs to consider the interoperability for collaboration level y based on more RAN2 progress since it is the signaling-based collaboration.

- Observation 3: Two categories of models including Proprietary-format models and Open-format models were proposed by RAN1. For the Proprietary-format models, due to the lack of inter-operation and recognition between vendors, it is hard to standardize based on the unified specification identification.

- Observation 4: Regarding to the Open-format models, the interoperability is feasible.

Proposal 3: RAN4 can focus on the Open-format models firstly, and discuss which core part and performance part requirements should be identified and how to define. On the other side, the test framework and procedure should also be discussed. At the meanwhile, RAN4 needs to wait for RAN1 progress on Open-format models.

Proposal 4: RAN4 shall study the basic structure of the open-format and consider how the common understanding defined between different vendors.

Proposal 5: From the perspective of test, both functionality test and performance test should be considered.

- Observation 5: Data collection provides dataset for multiple modules. Such dataset can be generated online during the test or be pre-generated before the test. From the perspective of test, at least the following aspects should be considered:

- Whether the latency of data collection needs to be verified;

- Whether and how to guarantee the fairness and uniformity of dataset cross different vendors;

- Whether the verification of dataset availability can be replaced by the performance of the model inference.

- Observation 6: model inference is the core component of AI/ML. Two aspects should be considered to verify: 1) The outputs are the results from the AI/ML inference model rather than the traditional solution; 2) The accuracy of outputs meet the requirement.

Observation 7: The model update operations such as model monitoring and model switching aim to provide timely model accompanied by the change of the inference requirement. To verify such model management operation, the performance gain between after and before the model update can be tested.

Proposal 6: Based on the analysis for model management especially for the model monitoring and model switching, RAN4 shall consider the specification impact for both of them more specifically.

Observation 8: The test environment is quite different from the field, so the assumptions is a little limitations and the generalization will be degraded.

Observation 9: The options may not work well, and the requirements based on these assumptions may not have significance.

Observation 10: The TU is limited in RAN4 for AI/ML.

Proposal 7: RAN4 shall study one-sided model firstly and the discussion of two-sided model shall be deprioritized since the two-sided model is only for CSI case.

Observation 11: 3GPP channel models have stable performance and sufficient 	physical meanings. It is convenient to generate large number of samples using 3GPP channel 	models.

Proposal 8: In order to guarantee the stable performance and convenience, RAN4 shall study and use the dataset based on TR 38.901 firstly.

- Reference

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP RAN Plenary

- [2] R4-2310433, “WF on RAN4 requirements for NR AI/ML”, Qualcomm Incorporated