3GPP TSG RAN WG4 Meeting #108	R4-2312122

Toulouse, France, Aug 21st – Aug 25th, 2023

Agenda item:	8.24.3

Source: 	vivo

Title: 	Discussion on NR-DC with selective activation of cell groups via L3 enhancements

Document for:	Discussion

- Introduction

After the discussion in the past meetings, most of issues on requirements for subsequent CPAC have been solved. The latest agreements were captured in WF [1] in RAN4#107 meeting. In this meeting, we would like to further clarify on the evaluation after subsequent CPAC execution and revisit the related conclusion which was agreed in the last meeting.

- Discussion

2.1 Subsequent CPAC

- Issue 1-2: whether to UE shall evaluate the execution condition of other candidate PSCells while executing CPC Agreements: - No. According to TS37.340 and RAN2 agreements, UE is not evaluating the execution condition of other candidate PSCells while executing CPC, and the evaluation is continued after finishing the PSCell addition or change.

In the last meeting, RAN4 reached the above common understanding which the evaluation of other candidate PSCells would not to be performed during the execution process and it is continued after PSCell addition or change is finished. This agreement means if the evaluation is stopped because of the execution process, the evaluation will continue to complete the previous evaluation that was suspended after CHO execution is completed. However, according to the latest RAN2 agreement below, the execution conditions for subsequent CPC would be generated by candidate PSCell.

After the previous execution of CHO, the new round of evaluation is based on the conditions which is newly generated by candidate PSCell rather than following the conditions configured at the beginning of subsequent CPC procedure. That is to say, the evaluation process would be restarted after each execution of CPC and UE is not able to continue the previous evaluation.

- Agreements in RAN2 #122 meeting For SN-initiated SCG selective activation, candidate SN generates execution conditions for subsequent CPC. FFS if it shall be possible to do something like MN-initiated CPA/CPC where Candidate SN generate execution conditions for subsequent CPC The UE shall skip the condition evaluation for a candidate which is a current PScell. The reference configuration is provided to all candidates involved in preparation, FFS which node initially generates it. Assume it can be provided in MN initiated and in SN initiated procedures. Will not spend specific efforts for supporting nested configurations for candidate cell configuration. Rapporteur take initiative on naming offline

Therefore, from this perspective, the RAN4 agreement reached in the last meeting needs to be modified accordingly and be consistent with RAN2 procedure. It could be modified as

- For subsequent CPAC, UE is not evaluating the execution condition of other candidate PSCells while executing CPC, and the evaluation is continued restarted after finishing the PSCell addition or change

Proposal 1: For subsequent CPAC, UE is not evaluating the execution condition of other candidate PSCells while executing CPC, and the evaluation is restarted after finishing the PSCell addition or change

- Conclusions

Proposal 1: For subsequent CPAC, UE is not evaluating the execution condition of other candidate PSCells while executing CPC, and the evaluation is restarted after finishing the PSCell addition or change

- Reference

[1] R4-2310064 WF on NR Mobility Enhancements RRM requirements (part 2), Apple.