3GPP TSG RAN WG4 Meeting #108	R4-2312137

Toulouse, 21– 25 August, 2023

Agenda item:			8.22.1

Source:	vivo

Title:	Draft reply LS on SRS and PRS bandwidth aggregation for positioning

Document for:	Discussion

Introduction

In the RAN1 LS [1] on measurement definitions for positioning with bandwidth aggregation, RAN4 is asked to provide the retuning time values, i.e. the guard period values, and to check the feasibility of the negative k values based on RAN1 agreements.

1. Overall Description: With regard to SRS and PRS bandwidth aggregation for NR positioning, RAN1 made the following agreements Agreement At least from UE capability perspective, the UE support of positioning SRS bandwidth aggregation in RRC_CONNECTED state is decoupled from the UE support of communication CA. Agreement When an SRS resource configured within a CC without PUSCH/PUCCH is linked for aggregation with an SRS resource configured within an UL active BWP of a UL communication CC, a guard period is needed before and after the aggregated SRS transmissions. - Send an LS to RAN4 with the above information and a request to provide the retuning time values needed. Agreement For PRS bandwidth aggregation, with regards to the signaling in the location information request message, introduce the following: - A request to indicate UE which two or three PFLs to be used for performing joint measurement - A new ReportingGranularityfactor smaller than 0 which can be applicable at least when the LMF requests aggregated measurements - Support at least the values of k={-1,-2} - FFS other values e.g. -3, -4, -5, -6 - Send RAN4 an LS to confirm the feasibility. Based on the first agreement, SRS for bandwidth aggregation may be configured within an UL CC without PUSCH/PUCCH. So, RAN1 would like RAN4 to provide the retuning time values, i.e. the guard period values as described in the second agreement, With regard to the third agreement, RAN1 would like RAN4 to check the feasibility of the negative k values. 2. Actions: To RAN4: ACTION: RAN1 respectfully asks RAN4 to take the above information into account, and provide the feedback to RAN1.

In this contribution, we provide our views on the guard period values and the feasibility of the negative k values and draft reply LS.

Discussion

Guard period

The guard period is needed only for the case that the SRS resource configured within a CC without PUSCH/PUCCH is linked for aggregation with an SRS resource configured within an UL active BWP of a UL communication CC. RAN1 has reached the agreement in the last meeting.

Agreement For a carrier including positioning SRS for aggregation, - Positioning SRS can be transmitted only when the carrier is activated - This is also applicable for the carrier only including positioning SRS for aggregation

Note that the SRS for positioning can only be transmitted when the carrier is activated, thus the interruption and delay due to SCC activation/deactivation may be needed before and after the aggregated SRS transmissions. However, we only concern about the RF retuning time which is RAN1 asked. Since SRS bandwidth aggregation for intra-band contiguous carriers is performed with single PA/RF chain Tx architectures and the numerology of CCs are all the same, the factors influencing retuning time consists of the following: filtering transients, sample rate change and the local oscillator retuning. The filtering transients delay and sample rate change delay are caused by the bandwidth change and the local oscillator retuning delay is caused by the center frequency change. Thus, the value of UE capability posSRS-RRC-Inactive-OutsideInitialUL-BWP-r17 captured in TS 38.331 can be reused since the retuning mechanism is similar.

TS 38.331 PosSRS-RRC-Inactive-OutsideInitialUL-BWP-r17::= SEQUENCE { -- R1 27-15b: Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP differentCenterFreqBetweenSRSposAndInitialBWP-r17 ENUMERATED {supported} OPTIONAL, switchingTimeSRS-TX-OtherTX-r17 ENUMERATED {us100, us140, us200, us300, us500} OPTIONAL, -- R1 27-15c: Support of positioning SRS transmission in RRC_INACTIVE state outside initial BWP with semi-persistent SRS

Proposal 1: The value of UE capability posSRS-RRC-Inactive-OutsideInitialUL-BWP-r17 can be reused for the guard period.

Feasible values of K

In the last meeting, RAN4 has reached the agreement

- Agreements - Report mapping for RSTD and UE Rx-Tx - Define additional mapping tables for reporting granularity of - 0.5Tc - FFS for 0.25Tc - FR1: support range of reporting granularity from 32Tc to X1 Tc - FR2: support range of reporting granularity from 32Tc to X2 Tc. - X1 and X2 are FFS. - X1 is FFS between 0.25, 0.5 or 1 Tc - X2 is FFS between 0.25 and 0.5 Tc

RAN4 has agreed to support at least the value of k=-1, whether negative k values can be smaller than -1 is still FFS. To meet the higher accuracy requirement of Rel-18 positioning, smaller report granularity is needed. Since the estimation of TOA can be finer when the bandwidth is larger and the PRS/SRS CA can across up to 3PFL, the report granularity should be smaller than the Rel-16/Rel-17 positioning. From TS 38.133, the value of k is 2 to 5 for FR1 and 0 to 5 for FR2. Since

,

Thus, the minimum value of k can be calculated as 0 for FR1 and -2 for FR2.

Proposal 2: Support the values of k={5,4,3,2,1,0} for FR1 and k={5,4,3,2,1,0,-1,-2} for FR2.

Conclusion

In this contribution, we provided our views on the guard period values and the feasibility of the negative k values. Based on analysis following proposals are present.

Proposal 1: The value of UE capability posSRS-RRC-Inactive-OutsideInitialUL-BWP-r17 can be reused for the guard period.

Proposal 2: Support the values of k={5,4,3,2,1,0} for FR1 and k={5,4,3,2,1,0,-1,-2} for FR2.

References

- R1-2306216, LS to RAN4 on SRS and PRS bandwidth aggregation for positioning, ZTE, RAN1

Appendix: Draft reply LS

3GPP TSG RAN WG4 Meeting #108	R4-23xxxxx

Toulouse, 21– 25 August, 2023

Title:	Reply LS on SRS and PRS bandwidth aggregation for positioning

Response to:	R1-2306216

Release:	Rel-18

Work Item:	NR_pos_enh2

Source:	RAN4

To:	RAN1

Cc:

Contact Person:

Name:		Qian Yang

E-mail Address: 	qian9.yang@vivo.com

1. Overall Description:

RAN4 thanks RAN1 for the LS on SRS and PRS bandwidth aggregation for positioning.

From RAN4 perspective, the value of UE capability posSRS-RRC-Inactive-OutsideInitialUL-BWP-r17 can be reused for the guard period, which is

{100us, 140us, 200us, 300us, 500us}

For the feasibility of the negative k values, support the value of k={-1,-2} for FR2.

2. Actions:

To: RAN WG1

ACTION: 	RAN4 kindly asks RAN1 to take the above RAN4 answers into account in the future work.

3. Reference:

- R1-2306216, LS to RAN4 on SRS and PRS bandwidth aggregation for positioning, ZTE, RAN1

4. Date of Next TSG RAN WG4 Meetings:

TSG-RAN4 Meeting #108-bis			9th Oct –13th Oct, 2023	                China

TSG-RAN4 Meeting #109			13th Nov –17th Nov, 2023	   US