3GPP TSG RAN WG4 Meeting #108		 R4-2312178

# Auguest 21 – Auguest 25, 2023 in Toulouse, France

Source:	MediaTek Inc.,

Title:	Discussion for MSD due to supporting FDD PC2 on n70

Agenda item:			7.22.2

Document for:	Approval

1. Introduction

In the latest RAN plenary meeting in June, more bands were added to the single band FDD HPUE basket work item for PC2 capability [1]. Among these bands is band n70. This contribution provides some desense evaluation on n70 supporting PC2.

2. Discussion

PC2 RSD for 2Tx and 1Tx

The operating frequency and some parameters we use in our evaluation are listed in Table 1.

Table 1 Receiver performance parameters for MSD analysis

| Parameter | Value | Unit |
| --- | --- | --- |
| Antenna isolation | 10 | dB |
| Front-end loss | 4 | dB |
| PCB isolation | 70 | dB |
| PA RXBN noise | -130 | dBm/Hz |
| Thermal noise at LB RX ANT port | -165 | dBm/Hz |
| Transceiver effective phase noise | -150 | dBc/Hz |
| SNR requirement for QPSK | -1 | dB |

Referring to existing 38.101-1, we made initial calculation in Table 2:

Table 2: Calculations for PC2 1Tx and 2Tx REFSENS for n70

|  |  | n70 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SCS kHz | 5 | 10 | 15 | 20 | 25 |
|  |  | MHz | MHz | MHz | MHz | MHz |
| PC3 REFSENS 38.101-1 | 15 | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 |
|  | 30 |  | -97.1 | -95.1 | -94.0 | -92.8 |
|  | 60 |  | -97.5 | -95.4 | -94.2 | -93.0 |
| UL config | 15 | 25 | 501 | 751 | Note 3 | Note 3 |
|  | 30 |  | 24 | 361 | Note 3 | Note 3 |
|  | 60 |  | 101 | 18 | Note 3 | Note 3 |
| PC3 scaled REFSENS | 15 | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 |
|  | 30 |  | -97.1 | -95.1 | -94.0 | -92.8 |
|  | 60 |  | -97.5 | -95.4 | -94.2 | -93.0 |
| PC3 scaled vs REFSENS | 15 | 0 | 0 | 0 | 0 | 0 |
|  | 30 |  | 0 | 0 | 0 | 0 |
|  | 60 |  | 0 | 0 | 0 | 0 |

Considering that the operating frequency of n70 is 1695 MHz – 1710 MHz for uplink and 1995 MHz – 2020 MHz for downlink where a large UL/DL separation could be observed, the uplink has almost no impact on the downlink receiving path. The uplink IMD order fall into self-downlink receiving channel could be more than 40th order thus the impairment can be ignored. In addition, by referring to another comparable discussion outcomes on band n66, a 0dB RSD can be used for n70 supporting PC2.

Proposal: For PC2 RSD of n70 for for both 1Tx and 2Tx architecture, RAN4 to set 0dB RSD and agree the following RSD tables:

Table 3 Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n70 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |

Table 4 Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n70 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |

3. Conclusion

Proposal: For PC2 RSD of n70 for for both 1Tx and 2Tx architecture, RAN4 to set 0dB RSD and agree the following RSD tables:

Table 3 Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n70 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |

Table 4 Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n70 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |

Reference

- RP-231064, Revised WID: High power UE (power class 2) for NR FR1 FDD single band, RAN_P#100