3GPP TSG RAN WG4 Meeting #108		 R4-2312184

# Auguest 21 – Auguest 25, 2023 in Toulouse, France

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for DL interruption note improvement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_enh-Core |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to endorsed WF R4-2310496, introduce the optional capability for resolving TX switching ambiguity issue for the parallel switching of two Tx chain in the case {1T, 1T, 0T, 0T} to {0T, 0T, 1T, 1T} on bands {A, B, C, D} |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce optional UE capability [BandOrdering1T1Tto1T1T] to indicate the support of the band ordering based approach - Introduce per Tx chain-based Tx switching capability [preferredBandPairsTxChain] to indicate the UE’s preferred switching band pairs for each Tx chain |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is ambiguity on 4-band Tx switching that switching period cannot be optimized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

===================Start of 1st changes=================

5.5A.3.0	General

–	if dualUL and [BandOrdering1T1Tto1T1T] are supported, UE supports associate the ordering of bands for defining switch-from and switch-to pairs when UE is configured parallel uplink transmission switching within four bands. UE is allowed to report per transmission chain-based Tx switching capability [preferredBandPairsTxChain] to indicate the UE’s preferred switching band pairs for corresponding Tx chain

===================End of 1st changes=================