3GPP TSG-RAN WG4 Meeting #108              	R4- 2312209

Toulouse, France, 21th Aug-25th Aug, 2023

Agenda item:	8.12.5.4

Source:	Samsung

Title:	View on demodulation requirements for tunnel deployment scenario for Rel-18 FR2 HST

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the deployment, test scenario, channel model and test scope for demodulation requirements were under discussion. The related agreements were captured in the WF [1]. Regarding the test scope of demodulation requirement in the tunnel scenario, the following agreement are made as

- Test scope of demodulation requirements of tunnel scenario - Whether to define UE demodulation requirements for tunnel deployment scenario in FR2 HST - Option 1 - Introduce the UE demodulation requirements with DPS transmission scheme with 1a and 1b for tunnel scenario. - Uni-directional with DPS scheme 1b - Bi-directional with DPS scheme 1a - Option 2 - RAN4 to define new demodulation requirements for tunnel deployment scenario only if a new channel model in the tunnel is introduced. - Option 3 - RAN4 should agree to not introduce dedicated Demodulation requirements for Tunnel Deployment, considering this scenario covered by existing test cases; - Option 4: - UE demodulation requirements can be defined only if new channel model with multi-path propagation introduced. - Whether to define BS demodulation requirements for tunnel deployment scenario in FR2 HST - Option 1 - Introduce single set requirement for PUSCH in tunnel scenario based on Bi-directional scenario in the tunnel scenario. - Option 2 - RAN4 to define new demodulation requirements for tunnel deployment scenario only if a new channel model in the tunnel is introduced. - BS demodulation requirements can be defined only if new channel with multi-path propagation introduced - Option 3 - BS demodulation requirements can be defined only if new channel with multi-path propagation introduced

Meanwhile, there is some discussion on tunnel deployment in the RRM session with the following agreement as

- Solution to the mobility issue in the tunnel - Consider bi-directional deployment with simultaneous multi-panel reception in the tunnel scenario to alleviate the mobility issues - Use of DRX in the tunnel deployment - Indication of tunnel deployment - FFS, whether indication of tunnel deployment is necessary

In this contribution, we provide the overview on the test scope of demodulation requirements in tunnel deployment scenario.

# 2	Discussion

UE demodulation requirement

Based on the RRM agreement, both Uni-directional and Bi-directional are considered for tunnel scenario for UE with single -panel reception with DPS transmission scheme. Meanwhile, to address the mobility issues in tunnel scenario, bi-directional deployment scenario with simultaneous multi-panel is considered.

In Rel-17 FR2 HST for open space scenario, RAN4 has introduced PDSCH requirement with DPS scheme 1a and DPS scheme 1b pending on UE capability for both Uni-directional and Bi-directional scenarios.

Regarding to whether introducing UE demodulation requirement, there is no conclusion yet in the last meeting. In our understanding, whether to define demodulation requirements, is pending on whether there is a different processing or new scenario, instead of the performance difference with existing FR2 HST requirement. From baseband processing aspect, there is no difference foreseen for open space and tunnel scenarios, the performance may be similar.

While from deployment aspect, open space and tunnel scenario belongs different deployment scenarios. Compared with open space scenario, the RRH deployment parameter is different in terms of Dmin, then, the doppler shift trajectory will be different. Considering the Dmin=1m in tunnel scenario, the maixum Doppler shift is higher than that in open space for both Un-directional and Bi-directional.

| Scenario | Maximum Doppler |
| --- | --- |
| Uni-directional with scenario A (Ds_offset=10 m) | 9627Hz |
| Uni-directional with scenario B (Ds_offset=100 m) | 9647Hz |
| Bi-directional with scenario B | 9502Hz |
| Uni-directional in tunnel scenario (Ds_offset=5 m) | 9722Hz |
| Bi-directional in tunnel scenario | 9722Hz |

Observation 1: The maximum Doppler shift experienced by UE in tunnel scenario is equal to 9722Hz, high than that in Open space scenario.

Considering both Uni-directional and Bi-directional are considered for tunnel scenario, similar as open space scenario, If PDSCH requirements with DPS transmission scheme is introduced, the DPS 1a and 1b can be considered for Uni-directional and Bi-directional scenario, separately

Proposal 1: If PDSCH requirements with DPS transmission scheme is introduced, the DPS 1a and 1b can be considered for Uni-directional and Bi-directional scenario, separately

BS demodulation requirement

Different with downlink, regardless the deployment or transmission scheme, e.g,bi-directional or uni-directional

only single tap is available for each RRH, since there is no multiple RRH combination assumption.

From baseband processing perspective, the tracking algorithm is also same for Uni-directional and Bi-directional. In FR1 HST, PUSCH requirements were introduced for both open space and tunnel scenario. Due to the different RRH deployment parameters for open space and tunnel scenario, the doppler shift trajectory will be different compared with existing sscenario 3-NR350/500. Considering the test coverage, we think PUSCH requirement could be introduced in tunnel scenario, which is comparable to FR1 HST. To reduce the test effort, the test applicability rule can be introduced.

Observation 2: PUSCH requirements with both open space and tunnel scenario were introduced

As for whether to define PUSCH requirement for Uni-directional and Bi-directional separately, similar as FR2 HST open space scenario, If PUSCH requirement is introduced, we think a single requirement could fulfill the test purpose, where the channel model is based on Bi-directional scenario,

Proposal 2: if PUSCH requirement is introduced, single set requirement for PUSCH in tunnel scenario based on Bi-directional scenario in the tunnel scenario could be considered

# 3	Conclusion

In this contribution, the view on the test scope of demodulation requirement for tunnel scenario is provided

Observation 1: The maximum Doppler shift experienced by UE in tunnel scenario is equal to 9722Hz, high than that in Open space scenario.

Proposal 1: If PDSCH requirements with DPS transmission scheme is introduced, the DPS 1a and 1b can be considered for Uni-directional and Bi-directional scenario, separately

Observation 2: PUSCH requirements with both open space and tunnel scenario were introduced

Proposal 2: if PUSCH requirement is introduced, single set requirement for PUSCH in tunnel scenario based on Bi-directional scenario in the tunnel scenario could be considered

# References

R4-2305892, WF on demodulation requirements for Rel-18 FR2 HST