3GPP TSG-RAN WG4 Meeting #108              	R4- 2312212

Toulouse, France, 21th Aug-25th Aug, 2023

Agenda item:	8.4.3.2

Source:	Samsung

Title:	Simulation results for PUSCH with UL 4-layer transmission

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the remaining issue for the test setup for BS demodulation requirement with UL 4-layer transmission was under discussion. The simulation assumption was agreed with capturing into the WF [1] as

- PUSCH requirement - Bandwidth and SCS - For 15KHz - Introduce requirement for 5MHz and 50MHz, - Additional CBW: 20MHz for MCS 17 only - For 30KHz: - Introduce requirement for 10MHz and 100MHz - Additional CBW: 40MHz for MCS 17 only - MCS (table1) - For TDLB100-400: MCS2 - For TDLA30-10: MCS 17 - For TDLC300-100: MCS12 - Test applicability rule - Unless otherwise stated, for a BS supporting different numbers of antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) (see D.37 in table 4.6-1), the 4 Tx antenna tests with low MIMO correlation level shall apply only for the highest numbers of supported connectors which is larger or equal to 4, and the specific connectors used for testing are based on manufacturer declaration. - Measurement uncertainty and test tolerance - Take 1 dB for MU and TT with fading profile power uncertainty ±1dB

In this contribution, the updated simulation results are provided for alignment and requirement derivation.

# 2	Simulation results

In this section, the simulation results are provided based on agreed simulation assumption as following table

Table 2-1：  Simulation assumption for 4Tx

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |
|  | DM-RS port | {0, 1, 2, 3} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A and B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| TPMI index for 4Tx four-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

Table 2-2：  Ideal and impairment simulation results  for 4Tx with TDLA30-10 channel, 15KHz SC (To be updated)

| Case | Antenna configuration | SCS | BW | Mapping type | Channel | Number of DMRS | MCS | SNR@70%TP ideal | SNR@70%TP Impairment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4T4R | 15KHz | 5 | Type A | TDLA30-10 | 1+1(2,11) | MCS 17 |  |  |
| 2 | 4T4R | 15KHz | 20 | Type A | TDLA30-10 | 1+1(2,11) | MCS 17 |  |  |
| 3 | 4T4R | 15KHz | 50 | Type A | TDLA30-10 | 1+1(2,11) | MCS 17 |  |  |
| 4 | 4T8R | 15KHz | 5 | Type A | TDLA30-10 | 1+1(2,11) | MCS 17 |  |  |
| 5 | 4T8R | 15KHz | 20 | Type A | TDLA30-10 | 1+1(2,11) | MCS 17 |  |  |
| 6 | 4T8R | 15KHz | 50 | Type A | TDLA30-10 | 1+1(2,11) | MCS 17 |  |  |
| 7 | 4T4R | 15KHz | 5 | Type B | TDLA30-10 | 1+1(2,11) | MCS 17 |  |  |
| 8 | 4T4R | 15KHz | 20 | Type B | TDLA30-10 | 1+1(2,11) | MCS 17 |  |  |
| 9 | 4T4R | 15KHz | 50 | Type B | TDLA30-10 | 1+1(2,11) | MCS 17 |  |  |
| 10 | 4T8R | 15KHz | 5 | Type B | TDLA30-10 | 1+1(2,11) | MCS 17 |  |  |
| 11 | 4T8R | 15KHz | 20 | Type B | TDLA30-10 | 1+1(2,11) | MCS 17 |  |  |
| 12 | 4T8R | 15KHz | 50 | Type B | TDLA30-10 | 1+1(2,11) | MCS 17 |  |  |

Table 2-3：  Ideal and impairment simulation results  for 4Tx with TDLA30-10 channel, 30KHz SCS (To be updated)

| Case | Antenna configuration | SCS | BW | Mapping type | Channel | Number of DMRS | MCS | SNR@70%TP ideal | SNR@70%TP Impairment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4T4R | 30KHz | 10 | Type A | TDLA30-10 | 1+1(2,11) | MCS 17 | 17.8 | 19.8 |
| 2 | 4T4R | 30KHz | 40 | Type A | TDLA30-10 | 1+1(2,11) | MCS 17 | 18.6 | 20.6 |
| 3 | 4T4R | 30KHz | 100 | Type A | TDLA30-10 | 1+1(2,11) | MCS 17 | 19.1 | 21.6 |
| 4 | 4T8R | 30KHz | 10 | Type A | TDLA30-10 | 1+1(2,11) | MCS 17 | 10.3 | 12.3 |
| 5 | 4T8R | 30KHz | 40 | Type A | TDLA30-10 | 1+1(2,11) | MCS 17 | 10.6 | 12.6 |
| 6 | 4T8R | 30KHz | 100 | Type A | TDLA30-10 | 1+1(2,11) | MCS 17 | 10.7 | 12.7 |
| 7 | 4T4R | 30KHz | 10 | Type B | TDLA30-10 | 1+1(2,11) | MCS 17 |  |  |
| 8 | 4T4R | 30KHz | 40 | Type B | TDLA30-10 | 1+1(2,11) | MCS 17 |  |  |
| 9 | 4T4R | 30KHz | 100 | Type B | TDLA30-10 | 1+1(2,11) | MCS 17 |  |  |
| 10 | 4T8R | 30KHz | 10 | Type B | TDLA30-10 | 1+1(2,11) | MCS 17 |  |  |
| 11 | 4T8R | 30KHz | 40 | Type B | TDLA30-10 | 1+1(2,11) | MCS 17 |  |  |
| 12 | 4T8R | 30KHz | 100 | Type B | TDLA30-10 | 1+1(2,11) | MCS 17 |  |  |

Table 2-4：  Ideal and impairment simulation results  for 4Tx with TDLB100-400 channel, 15KHz SCS (To be updated)

| Case | Antenna configuration | SCS | BW | Mapping type | Channel | Number of DMRS | MCS | SNR@70%TP ideal | SNR@70%TP Impairment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4T4R | 15KHz | 5 | Type A | TDLB100-400 | 1+1(2,11) | MCS 2 |  |  |
| 2 | 4T4R | 15KHz | 50 | Type A | TDLB100-400 | 1+1(2,11) | MCS 2 |  |  |
| 3 | 4T8R | 15KHz | 5 | Type A | TDLB100-400 | 1+1(2,11) | MCS 2 |  |  |
| 4 | 4T8R | 15KHz | 50 | Type A | TDLB100-400 | 1+1(2,11) | MCS 2 |  |  |
| 5 | 4T4R | 15KHz | 5 | Type B | TDLB100-400 | 1+1(2,11) | MCS 2 |  |  |
| 6 | 4T4R | 15KHz | 50 | Type B | TDLB100-400 | 1+1(2,11) | MCS 2 |  |  |
| 7 | 4T8R | 15KHz | 5 | Type B | TDLB100-400 | 1+1(2,11) | MCS 2 |  |  |
| 8 | 4T8R | 15KHz | 50 | Type B | TDLB100-400 | 1+1(2,11) | MCS 2 |  |  |

Table 2-5：  Ideal and impairment simulation results  for 4Tx with TDLB100-400 channel, 30KHz SCS (To be updated)

| Case | Antenna configuration | SCS | BW | Mapping type | Channel | Number of DMRS | MCS | SNR@70%TP ideal | SNR@70%TP Impairment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4T4R | 30KHz | 10 | Type A | TDLB100-400 | 1+1(2,11) | MCS 2 | 0.1 | 2.1 |
| 2 | 4T4R | 30KHz | 100 | Type A | TDLB100-400 | 1+1(2,11) | MCS 2 | 0.7 | 2.7 |
| 3 | 4T8R | 30KHz | 10 | Type A | TDLB100-400 | 1+1(2,11) | MCS 2 | -2.8 | -0.8 |
| 4 | 4T8R | 30KHz | 100 | Type A | TDLB100-400 | 1+1(2,11) | MCS 2 | -2.0 | 0.0 |
| 5 | 4T4R | 30KHz | 10 | Type B | TDLB100-400 | 1+1(2,11) | MCS 2 |  |  |
| 6 | 4T4R | 30KHz | 100 | Type B | TDLB100-400 | 1+1(2,11) | MCS 2 |  |  |
| 7 | 4T8R | 30KHz | 10 | Type B | TDLB100-400 | 1+1(2,11) | MCS 2 |  |  |
| 8 | 4T8R | 30KHz | 100 | Type B | TDLB100-400 | 1+1(2,11) | MCS 2 |  |  |

Table 2-6：  Ideal and impairment simulation results  for 4Tx with TDLC300-100 channel, 15KHz SCS (To be updated)

| Case | Antenna configuration | SCS | BW | Mapping type | Channel | Number of DMRS | MCS | SNR@70%TP ideal | SNR@70%TP Impairment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4T4R | 15KHz | 5 | Type A | TDLC300-100 | 1+1(2,11) | MCS 12 |  |  |
| 2 | 4T4R | 15KHz | 50 | Type A | TDLC300-100 | 1+1(2,11) | MCS 12 |  |  |
| 3 | 4T8R | 15KHz | 5 | Type A | TDLC300-100 | 1+1(2,11) | MCS 12 |  |  |
| 4 | 4T8R | 15KHz | 50 | Type A | TDLC300-100 | 1+1(2,11) | MCS 12 |  |  |
| 5 | 4T4R | 15KHz | 5 | Type B | TDLC300-100 | 1+1(2,11) | MCS 12 |  |  |
| 6 | 4T4R | 15KHz | 50 | Type B | TDLC300-100 | 1+1(2,11) | MCS 12 |  |  |
| 7 | 4T8R | 15KHz | 5 | Type B | TDLC300-100 | 1+1(2,11) | MCS 12 |  |  |
| 8 | 4T8R | 15KHz | 50 | Type B | TDLC300-100 | 1+1(2,11) | MCS 12 |  |  |

Table 2-7：  Ideal and impairment simulation results  for 4Tx with TDLC300-100 channel, 30KHz SCS (To be updated)

| Case | Antenna configuration | SCS | BW | Mapping type | Channel | Number of DMRS | MCS | SNR@70%TP ideal | SNR@70%TP Impairment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4T4R | 30KHz | 10 | Type A | TDLC300-100 | 1+1(2,11) | MCS 12 | 12.9 | 14.9 |
| 2 | 4T4R | 30KHz | 100 | Type A | TDLC300-100 | 1+1(2,11) | MCS 12 | 13.6 | 15.6 |
| 3 | 4T8R | 30KHz | 10 | Type A | TDLC300-100 | 1+1(2,11) | MCS 12 | 6.4 | 8.4 |
| 4 | 4T8R | 30KHz | 100 | Type A | TDLC300-100 | 1+1(2,11) | MCS 12 | 6.7 | 8.7 |
| 5 | 4T4R | 30KHz | 10 | Type B | TDLC300-100 | 1+1(2,11) | MCS 12 |  |  |
| 6 | 4T4R | 30KHz | 100 | Type B | TDLC300-100 | 1+1(2,11) | MCS 12 |  |  |
| 7 | 4T8R | 30KHz | 10 | Type B | TDLC300-100 | 1+1(2,11) | MCS 12 |  |  |
| 8 | 4T8R | 30KHz | 100 | Type B | TDLC300-100 | 1+1(2,11) | MCS 12 |  |  |

# 3	Conclusion

In this contribution, the updated simulation results with ideal and impairment results are provided for alignment and requirement derivation.

# References

R4-2309798: WF on 4Tx demodulation