3GPP TSG-RAN WG4 Meeting #108              	R4- 2312217

Toulouse, France, 21th Aug-25th Aug, 2023

Agenda item:	5.2.4

Source:	Samsung

Title:	FRC and simulation assumption correction for Rel-17 FR2 HST

Document for:	Discussion

- Introduction

The Rel-17 FR2 HST WI has completed in RAN#97 meeting.

In this contribution, the view on CSI-RS configuration and FRC table for FR2 HST is provided for correction

# 2	Discussion

CSI-RS configuration

In Rel-17 FR2 HST, PDSCH was scheduled in the special slot for TDD. The agreed TDD patten is DDDSU (S= 10D: 2GP:2U) where 10 symbols are allocated for downlink. One will be used for PDCCH, hence PDSCH mapping starts at symbol 1 and has a duration of 9. Based on Rel-17 FR2 HST simulation assumption, 3 DMRS symbols will be configured. For normal DL slot, the DMRS symbols positioning should be (2,7, 11), and for special DL slot, the DMRS symbols positioning should be (2, 6, 9), based on specification TS 38.211 in Table 7.4.1.1.2-3

The following is the current one CSI-RS configuration in TS.101-4, where l0=9 is configured for CSI-RS in slot 3, the TDD special slot. The relation RE mapping can be illustrated in figure 1 [1]

Table 1: CSI-RS configuration

| Resource set #1 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 for CSI-RS resource 1,2,3,4 |
| --- | --- | --- | --- |
|  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 |
|  |  |  | l0 = 9 for CSI-RS resource 2 and 4 |
|  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset | Slots | 2 for CSI-RS resource 1 and 2 |
|  |  |  | 3 for CSI-RS resource 3 and 4 |
|  | QCL info |  | TCI state #4 |
|  | Frequency Occupation |  | Start PRB 0 |
|  |  |  | Number of PRB =ceil(BWP size/4)*4 |

Fig1:  3 DMRS symbol in Special slots (2,6, 9) in Bi-directional scenario, where 8 CSI resource set configured

Observation 1:  CSI-RS and DM-RS are configured on the same resource elements.

For FR2, the time index of CSI-RS can be configured as following

-	, , or for frequency range 1 and frequency range 2,

-	, , , , ,  or  for frequency range 2.

To avoid the colliding of DMRS symbol and CSI-RS RE in both special DL slot and normal DL slots, we suggestion to update the CSI-RS configuration for the first symbol as {1, 5} as

| Resource set #1 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 for CSI-RS resource 1,2,3,4 |
| --- | --- | --- | --- |
|  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 1 for CSI-RS resource 1 and 3 |
|  |  |  | l0 = 5 for CSI-RS resource 2 and 4 |
|  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset | Slots | 2 for CSI-RS resource 1 and 2 |
|  |  |  | 3 for CSI-RS resource 3 and 4 |
|  | QCL info |  | TCI state #4 |
|  | Frequency Occupation |  | Start PRB 0 |
|  |  |  | Number of PRB =ceil(BWP size/4)*4 |

Proposal 1:  Change the first OFDM symbol in the PRB used for CSI-RS from {5, 9} to {1,5}

FRC table

Since 3 DMRS symbols are available for special DL slots, the FRC should be updated to align 3 DMRS configuration. Meanwhile, the number of CSI-RS source number is 8 for bi-directional scenario in each slot, there will be 4RE CSI-RS REs overlapping with PR-RS RE.  Then, the corresponding overhead should be counted into the binary Channel Bits Per Slot calculation

| Parameter | Unit |  | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  |  |  | R.PDSCH.5-12.1 TDD | R.PDSCH.5-12.2 TDD |  |  |  |  |
| Channel bandwidth |  | MHz |  | 200 | 200 |  |  |  |  |
| Subcarrier spacing |  | kHz |  | 120 | 120 |  |  |  |  |
| Allocated resource blocks |  | PRBs |  | 132 | 132 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  |  |  | N/A | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  |  |  | 9 | 9 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,159} |  |  |  | 13 | 13 |  |  |  |  |
| For Slot i=1 |  |  |  | 13 | N/A (Note 4) |  |  |  |  |
| Allocated slots per 2 frames |  |  |  | 127 | 126 |  |  |  |  |
| MCS table |  |  |  | 64QAM | 64QAM |  |  |  |  |
| MCS index |  |  |  | 17 | 17 |  |  |  |  |
| Modulation |  |  |  | 64QAM | 64QAM |  |  |  |  |
| Target Coding Rate |  |  |  | 0.43 | 0.43 |  |  |  |  |
| Number of MIMO layers |  |  |  | 2 | 2 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  |  |  | N/A | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  |  |  | 18 | 18 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,159} |  |  |  | 18 | 18 |  |  |  |  |
| For Slot i = 1 |  |  |  | 18 | N/A (Note 4) |  |  |  |  |
| Overhead for TBS determination |  |  |  | 6 | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | Bits |  | N/A | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | Bits |  | 57376 | 57376 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,159} |  | Bits |  | 90176 | 90176 |  |  |  |  |
| For Slot i = 1 |  |  |  | 90176 | N/A (Note 4) |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | Bits |  | N/A | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | Bits |  | 24 | 24 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,159} |  | Bits |  | 24 | 24 |  |  |  |  |
| For Slot i = 1 |  |  |  | 24 | N/A (Note 4) |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | CBs |  | N/A | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | CBs |  | 7 | 7 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,159} |  | CBs |  | 11 | 11 |  |  |  |  |
| For Slot i = 1 |  |  |  | 11 | N/A (Note 4) |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | Bits |  | N/A | N/A |  |  |  |  |
| For Slots i = 2 and 82 (Note 3) |  | Bits |  | 195696 | 181968 |  |  |  |  |
| For Slots i = 3 and 83 (Note 3) |  | Bits |  | 122832 | 109104 |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…,79, 84,…,159} |  | Bits |  | 137808 | 137808 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {4,…,79,84,…,159} |  | Bits |  | 210672 | 210672 |  |  |  |  |
| For Slot i = 1 |  | Bits |  | 210672 | N/A (Note 4) |  |  |  |  |
| Max. Throughput averaged over 2 frames |  | Mbps |  | 520.138 | 515629 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. Note 4: SS/PBCH block is transmitted in slot #1 with periodicity 20ms |  |  |  |  |  |  |  |  |  |

Proposal 2: Update the FRC for Rel-17 FR2 HST UE demodulation in Table A.3.2.2.5-12

# 3	Conclusion

In this contribution, the view on the correction FR2 HST test setup for Rel-17, including CSI-RS configuration and FRC table

Observation 1:  CSI-RS and DM-RS are configured on the same resource elements.

Proposal 1:  Change the first OFDM symbol in the PRB used for CSI-RS from {5, 9} to {1,5}

Proposal 2: Update the FRC for Rel-17 FR2 HST UE demodulation in Table A.3.2.2.5-12

| Parameter | Unit |  | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  |  |  | R.PDSCH.5-12.1 TDD | R.PDSCH.5-12.2 TDD |  |  |  |  |
| Channel bandwidth |  | MHz |  | 200 | 200 |  |  |  |  |
| Subcarrier spacing |  | kHz |  | 120 | 120 |  |  |  |  |
| Allocated resource blocks |  | PRBs |  | 132 | 132 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  |  |  | N/A | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  |  |  | 9 | 9 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,159} |  |  |  | 13 | 13 |  |  |  |  |
| For Slot i=1 |  |  |  | 13 | N/A (Note 4) |  |  |  |  |
| Allocated slots per 2 frames |  |  |  | 127 | 126 |  |  |  |  |
| MCS table |  |  |  | 64QAM | 64QAM |  |  |  |  |
| MCS index |  |  |  | 17 | 17 |  |  |  |  |
| Modulation |  |  |  | 64QAM | 64QAM |  |  |  |  |
| Target Coding Rate |  |  |  | 0.43 | 0.43 |  |  |  |  |
| Number of MIMO layers |  |  |  | 2 | 2 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  |  |  | N/A | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  |  |  | 18 | 18 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,159} |  |  |  | 18 | 18 |  |  |  |  |
| For Slot i = 1 |  |  |  | 18 | N/A (Note 4) |  |  |  |  |
| Overhead for TBS determination |  |  |  | 6 | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | Bits |  | N/A | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | Bits |  | 57376 | 57376 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,159} |  | Bits |  | 90176 | 90176 |  |  |  |  |
| For Slot i = 1 |  |  |  | 90176 | N/A (Note 4) |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | Bits |  | N/A | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | Bits |  | 24 | 24 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,159} |  | Bits |  | 24 | 24 |  |  |  |  |
| For Slot i = 1 |  |  |  | 24 | N/A (Note 4) |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | CBs |  | N/A | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | CBs |  | 7 | 7 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,159} |  | CBs |  | 11 | 11 |  |  |  |  |
| For Slot i = 1 |  |  |  | 11 | N/A (Note 4) |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | Bits |  | N/A | N/A |  |  |  |  |
| For Slots i = 2 and 82 (Note 3) |  | Bits |  | 195696 | 181968 |  |  |  |  |
| For Slots i = 3 and 83 (Note 3) |  | Bits |  | 122832 | 109104 |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…,79, 84,…,159} |  | Bits |  | 137808 | 137808 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {4,…,79,84,…,159} |  | Bits |  | 210672 | 210672 |  |  |  |  |
| For Slot i = 1 |  | Bits |  | 210672 | N/A (Note 4) |  |  |  |  |
| Max. Throughput averaged over 2 frames |  | Mbps |  | 520.138 | 515629 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. Note 4: SS/PBCH block is transmitted in slot #1 with periodicity 20ms |  |  |  |  |  |  |  |  |  |

# References

RP-231354	Correction for HST FR2 test setup from Rel-17 (TS 38.101-4, Rel-17)