# 3GPP TSG-RAN WG4 Meeting # 108                                   R4-2312230

# Toulouse, France, August 21 – August 25, 2023

Title:	Reply LS on on beam application time, contents of cell switch command, TCI state activation and UE based TA measurement for LTM

Response to:	R1-2306259/R4-2311038

Release:	Rel-18

Work Item:	NR_mob_enh2-Core

Source:	RAN4

To:	RAN1

Cc:	RAN2, RAN3

Contact person:	Wu Yan

wu.yan7@zte.com.cn

Send any reply LS to:    3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

Overall description

RAN4 would like to thank RAN1 for their LS on beam application time, contents of cell switch command, TCI state activation and UE based TA measurement for LTM in R1-2306259/R4-2311038. Based on the LS sent by RAN1, RAN4 understand that the feasibility of this working assumption needs further discussion. The specific replication is as follows.

Confirm the following Working Assumption, and sent LS to RAN4 to clarify the feasibility of supporting this mechanism Working Assumption From RAN 1 perspective, UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported. - Corresponding UE capability is to be introduced to support UE-based TA measurement - For a UE reports support of this capability, configuration of UE-based TA measurement is supported - FFS: other impacts on RAN1 spec

In RAN4’s view, whether UE based TA measurement is feasible is up to gNB sync error. The feasible range of UE based TA measurement is shown in Table 1.

Table1. Limitation of gNB sync error for UE based TA measurement

| SCS (kHz) | CP length (us) | Limitation of gNB sync error (us) |
| --- | --- | --- |
| 15 | 4.7 | 1.55 |
| 30 | 2.3 | 0.35 |
| 60 | 1.2 | / |
| 120 | 0.6 | 0.05 |

# 2	Actions

To RAN1

ACTION: 	RAN4 would like RAN1 to take into account the above RAN4 replies/feedback in their future work on beam application time, contents of cell switch command, TCI state activation and UE based TA measurement for LTM.

# 3	Dates of next TSG-RAN WG RAN4 meetings

RAN4#108bis, 9th - 13th October 2023, China.

RAN4#109, 13th-17th November 2023, America.