3GPP TSG-RAN WG4 Meeting # 108	R4-2312237

# Toulouse, FR, Aug. 21 – 25, 2023

Source:	LG Electronics

Title:	Discussion for FR2-1 UL256QAM MPR

Agenda item:	8.6.2

Document for:	Discussion

Introduction

During the FR2-1 UL256QAM meetings, we provided MPR simulation results for FR2-1 UL256QAM, and we agreed that the MPR vaule should be a reasonable value for UL256QAM EVM test. In this paper, we performed MPR simulation with the agreed assumption[1] and low AM-PM distortion PA model based on our mmWave transceiver system to obtain the reasonable MPR value.

Discussion

MPR simulation results for FR2-1 UL256QAM PC2

Table 1 phase noise profiles

For 29 GHz phase noise profile - option 1 (TR 38.803 Example 1) % fz = [3e3 550e3 280e6]; % fp = [1 1.6e6 30e6]; % alphaz = [2.37 2.7 2.53]; % alphap = [3.3 3.3 1]; % PSD0 = 32; - option 2 (QC’s) % fz = [3e3 7e5 9e5]; % fp = [1 1e6 1.1e6]; % alphaz = [2.37 4.7 2]; % alphap = [3.3 5.3 2.5]; % PSD0 = 33; - option 3 (MTK’s) % fz = [3e3 550e3 280e6]; % fp = [1 1.6e6 30e6]; % alphaz = [2.37 2.7 2.53]; % alphap = [3.3 3.3 1]; % PSD0 = 29.35; For 39 GHz phase noise profi - option 4 (MTK’s) % fz = [3e3 620e3 240e6]; % fp = [1 1.6e6 30e6]; % alphaz = [2.37 2.7 2.53]; % alphap = [3.3 3.3 1]; % PSD0 = 31.76;

We recognize the AM-PM distortion is the key factor for the MPR value based on our mmWave transceiver system. So, we performed MPR simulation with low AM-PM distortion PA. The low AM-PM distortion PA has only few degrees distortion until the AM-AM drops. The MPR result comparison between general PA and Low AM-PM distortion PA are showed in table 2 for 29 GHz and table 3 for 39 GHz.

-DFT-s-OFDM for 100 MHz CBW

MPR value with Option2 phase noise profile and low AM-PM distortion PA is improved about 1~1.5 dB over MPR with Option2 phase noise profile only.

-CP-OFDM for 100 MHz CBW

MPR value with Option2 phase noise profile and low AM-PM distortion PA is improved about 0.5~1 dB over MPR using only Option2 phase noise profile.The CP-OFDM waveform has a high PAPR, so the AM-AM distortion is more critical to the MPR value. That’s why the MPR improvement is less than DFT-s-OFDM when the lower AM-PM distortion PA is applied.

-DFT-s-OFDM for 400 MHz CBW

The MPR value with Option2 phase noise profile and low AM-PM distortion PA is improved about 0~0.5 dB. The limiting factor for 400 MHz DFT-s-OFDM is IBE&EVM, so it has low improvement.

-CP-OFDM for 400 MHz CBW

The MPR value with Option2 phase noise profile and low AM-PM distortion PA is improved about 0~1 dB over MPR using only Option2 phase noise profile.

Observation 1: The lower MPR value with low AM-PM distortion PA than MPR value with general PA is observed.

Proposal 1: Consider the low AM-PM distortion PA to obtain the reasonable MPR value for FR2-1 256QAM.

Table 2 Simulated MPR results for 29 GHz PC2 UL256QAM

|  | PA model | PN Profile | Region 1 [22, 20] | Outer1 [0, 64] | Outer 2 [0, 1] | Outer 3 [0, 20] |
| --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM (CBW: 100 MHz) | General | Option 1 | 7.54 | 7.54 | 6.21 | 6.50 |
|  | General | Option 2 | 7.22 | 7.34 | 5.96 | 6.18 |
|  | General | Option 3 | 7.22 | 7.22 | 6.00 | 6.19 |
|  | Low AM-PM distortion | Option 2 | 5.86 | 5.97 | 5.60 | 5.78 |
| CP-OFDM (CBW: 100 MHz) | General | Option 1 | 9.48 | 9.50 | 8.27 | 9.05 |
|  | General | Option 2 | 9.26 | 9.27 | 8.04 | 8.72 |
|  | General | Option 3 | 9.26 | 9.26 | 7.74 | 8.83 |
|  | Low AM-PM distortion | Option 2 | 8.39 | 8.29 | 7.51 | 8.18 |
|  | PN profile | Region 1 [88, 80] | Outer1 [0, 240] | Outer 2 [0, 1] | Outer 3 [0, 80] | Outer 3 [0, 20] |
| DFT-s-OFDM (CBW: 400 MHz) | General | Option 1 | 8.91 | 8.91 | 7.19 | 7.84 |
|  | General | Option 2 | 8.48 | 8.06 | 7.10 | 7.33 |
|  | General | Option 3 | 8.48 | 8.06 | 7.11 | 7.44 |
|  | Low AM-PM distortion | Option 2 | 8.26 | 7.54 | 7.06 | 7.23 |
| CP-OFDM (CBW: 400 MHz) | General | Option 1 | 10.96 | 10.84 | 9.04 | 10.09 |
|  | General | Option 2 | 10.30 | 10.30 | 9.10 | 9.76 |
|  | General | Option 3 | 10.19 | 10.30 | 8.96 | 9.75 |
|  | Low AM-PM distortion | Option 2 | 9.44 | 9.86 | 8.76 | 9.44 |

[start RB position, allocated RB number]

Table 3 Simulated MPR results for 39 GHz PC2 UL256QAM

|  | PA model | PN Profile | Region 1 [22, 20] | Outer1 [0, 64] | Outer 2 [0, 1] | Outer 3 [0, 20] |
| --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM (CBW: 100 MHz) | General | Option 4 | 7.44 | 7.33 | 6.01 | 6.37 |
|  | Low AM-PM distortion | Option 4 | 5.97 | 6.17 | 5.54 | 5.86 |
| CP-OFDM (CBW: 100 MHz) | General | Option 4 | 9.27 | 9.48 | 8.00 | 8.83 |
|  | Low AM-PM distortion | Option 4 | 8.37 | 8.39 | 7.49 | 8.31 |
|  | PN profile | Region 1 [88, 80] | Outer1 [0, 240] | Outer 2 [0, 1] | Outer 3 [0, 80] | Outer 3 [0, 20] |
| DFT-s-OFDM (CBW: 400 MHz) | General | Option 4 | 8.47 | 8.48 | 7.00 | 7.54 |
|  | Low AM-PM distortion | Option 4 | 8.48 | 7.64 | 6.84 | 7.33 |
| CP-OFDM (CBW: 400 MHz) | General | Option 4 | 10.73 | 10.85 | 9.13 | 9.82 |
|  | Low AM-PM distortion | Option 4 | 10.19 | 10.30 | 8.78 | 9.54 |

[start RB position, allocated RB number]

Table 4 MPR results for 29 GHz PC2 UL256QAM with margin

| CBW (100 MHz) | PA model | P/N profile | Inner RB allocations, Region 1 | Edge RB allocations |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM | General | Option 1 | 9 | 9 |
|  | General | Option 2 | 8.5 | 9 |
|  | General | Option 3 | 8.5 | 8.5 |
|  | Low AM-PM distortion | Option 2 | 7.5 | 7.5 |
| CP-OFDM | General | Option 1 | 11 | 11 |
|  | General | Option 2 | 11 | 11 |
|  | General | Option 3 | 11 | 11 |
|  | Low AM-PM distortion | Option 2 | 10 | 10 |
| CBW (400 MHz) | PA model | P/N profile | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | General | Option 1 | 10.5 | 10.5 |
|  | General | Option 2 | 9.5 | 9.5 |
|  | General | Option 3 | 10 | 9.5 |
|  | Low AM-PM distortion | Option 2 | 10 | 9 |
| CP-OFDM | General | Option 1 | 12.5 | 12.5 |
|  | General | Option 2 | 12 | 12 |
|  | General | Option 3 | 11.5 | 12 |
|  | Low AM-PM distortion | Option 2 | 11 | 11.5 |

[start RB position, allocated RB number]

Table 5 MPR results for 39 GHz PC2 UL256QAM with margin

| CBW (100 MHz) | PA model | P/N profile | Inner RB allocations, Region 1 | Edge RB allocations |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM | General | Option 4 | 9 | 9 |
|  | Low AM-PM distortion | Option 4 | 7.5 | 7.5 |
| CP-OFDM | General | Option 4 | 11 | 11 |
|  | Low AM-PM distortion | Option 4 | 10 | 10 |
| CBW (400 MHz) | PA model | P/N profile | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | General | Option 4 | 10 | 10 |
|  | Low AM-PM distortion | Option 4 | 10 | 9 |
| CP-OFDM | General | Option 4 | 12 | 12.5 |
|  | Low AM-PM distortion | Option 4 | 11.5 | 12 |

[start RB position, allocated RB number]

Table 4 and table 5 show the MPR value with the margin. We observed the MPR values using Low AM-PM distortion PA are lower than MPR value with general PA model. But, the characteristic of low AM-PM distortion PA is from our mmWave products, so we need to more margin for the MPR valu with low AM-PM distortion. So, we propose the MPR value as the average of the MPR results using the general PA and the MPR results using the low AM-PM distortion PA.

Proposal 2: Consider the MPR value as the average of the MPR results using the general PA and the MPR results using the low AM-PM distortion PA. (29 GHz: table 6, 39 GHz: table 7)

Table 6 The proposed MPR values for 29 GHz PC2 UL256QAM

| CBW (100 MHz) | Inner RB allocations, Region 1 | Edge RB allocations |
| --- | --- | --- |
| DFT-s-OFDM | 8 | 8 |
| CP-OFDM | 10.5 | 10.5 |
| CBW (400 MHz) | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | 10 | 9.5 |
| CP-OFDM | 11.5 | 11.5 |

Table 7 The MPR values for 39 GHz PC2 UL256QAM

| CBW (100 MHz) | Inner RB allocations, Region 1 | Edge RB allocations |
| --- | --- | --- |
| DFT-s-OFDM | 8 | 8.5 |
| CP-OFDM | 10.5 | 10.5 |
| CBW (400 MHz) | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | 10 | 9.5 |
| CP-OFDM | 12 | 12 |

Conclusion

Observation 1: The lower MPR value with low AM-PM distortion PA than MPR value with general PA is observed.

Proposal 1: Consider the low AM-PM distortion PA to obtain the reasonable MPR value for FR2-1 256QAM.

Proposal 2: Consider the MPR value as the average of the MPR results using the general PA and the MPR results using the low AM-PM distortion PA. (29 GHz: table 6, 39 GHz: table 7)

Table 6 The proposed MPR values for 29 GHz PC2 UL256QAM

| CBW (100 MHz) | Inner RB allocations, Region 1 | Edge RB allocations |
| --- | --- | --- |
| DFT-s-OFDM | 8 | 8 |
| CP-OFDM | 10.5 | 10.5 |
| CBW (400 MHz) | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | 10 | 9.5 |
| CP-OFDM | 11.5 | 11.5 |

Table 7 The proposed MPR values for 39 GHz PC2 UL256QAM

| CBW (100 MHz) | Inner RB allocations, Region 1 | Edge RB allocations |
| --- | --- | --- |
| DFT-s-OFDM | 8 | 8.5 |
| CP-OFDM | 10.5 | 10.5 |
| CBW (400 MHz) | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | 10 | 9.5 |
| CP-OFDM | 12 | 12 |

- References

[1]R4-2310260, WF on UL 256QAM requirements (Xiaomi)