3GPP TSG-RAN WG4 Meeting # 108 				            	R4-2312242

Toulouse, France, August 21 – August 25, 2023

Source:	LG Electronics

Title:	UE RF requirements of con-current operation on Uu and sidelink

Agenda item:	8.30.2.2

Document for:	Discussion

Introduction

In this paper, we provide our views on the remaining UE RF requirements of con-current operation on Uu and sidelink based on the WF [1].

Discussion

Regarding the WF [1], the current status can be summarized in Table 2.1.

Table 2.1: Status of issues for con-current operation on Uu and sidelink

| Issue | Status |
| --- | --- |
| 1-1-1: Limited number of example band combination for Uu@Licensed and SL@Un-licensed | Agreed |
| 1-1-2: Example band combination for Uu@Licensed and SL@Un-licensed | Agreed |
| 1-2-1: Con-current operation of Uu@Un-licensed and SL@Un-licensed | Agreed |
| 1-2-1: Con-current operation of Uu@Un-licensed and SL@licensed | Agreed |
| 1-2-3: Con-current operation of intra-band SL@Un-licensed | Agreed |
| 1-3-1: Basis of SL-U for con-current operation | Agreed |
| 1-3-2: Applicable channel bandwidth for con-current operation of Uu@Licensed and SL@Un-licensed | Agreed |
| 1-3-3: RF architecture reference of Uu@Licensed and SL@Un-licensed | Agreed |
| 1-4-0: MOP for Uu@Licensed and SL@Un-licensed | FFS |
| 1-4-1: Targeted power class for Uu@Licensed and SL@Un-licensed | Agreed |
| 1-4-2: MPR/A-MPR for Uu@Licensed and SL@Un-licensed | Agreed |
| 1-4-3: Configured transmitted power for Uu@Licensed and SL@Un-licensed | Initial agreement |
| 1-4-4: UE coexistence requirements for Uu@Licensed and SL@Un-licensed | Agreed |
| 1-4-5: General spurious emission for Uu@Licensed and SL@Un-licensed | Agreed |
| 1-4-6: Additional spurious emission for Uu@Licensed and SL@Un-license | Agreed |
| 1-4-7: Other Tx requirements for Uu@Licensed and SL@Un-licensed | Agreed |
| 1-5-1: MSD for Uu@Licensed and SL@Un-licensed | Initial agreement |
| 1-5-2: Other Rx requirements for Uu@Licensed and SL@Un-licensed | Agreed |

The issues with FFS and the initial agreement are focused on in this contribution.

1-4-0: MOP for Uu@Licensed and SL@Un-licensed

In the last meeting, the following options are discussed.

- Option 1: Define MOP as the total transmitted power from each operating band

- Option 2: Define MOP per each operating band in Rel-18

RAN4 agreed with n78@licensed band + n46@unlicensed band as the example band combination.

- Power class 3 (23dBm) for n78 + Power class 5 (20dBm) for n46

Total maximum output power can be larger than 23 dBm. It can make SAR issue. To avoid SAR issue, Option1 is preferred. If option 2 is considered, how to solve SAR issue should be considered.

Proposal 1: Define MOP as the total transmitted power from each operating band of Uu and SL.

1-4-3: Configured transmitted power for Uu@Licensed and SL@Un-licensed

In the last meeting, the following options are discussed.

- Option 1: Apply the principle of the configured transmitted power of NR inter-band CA for the total UE configured maximum output power PCMAX (p,q) in a slot p of NR uplink carrier and a slot q of NR  sidelink

  - PCMAX_L (p,q) =  MIN {10log10 [pCMAX_L,c,NR (p)+ pCMAX_L,c,SL (q)], PPowerClass_CA, PEMAX,CA}

  - PCMAX_H (p,q) = MIN {10 log10 [pCMAX_H,c,NR (p) + pCMAX_H,c,SL (q)], PPowerClass_CA, PEMAX,CA}

- Option 2: Consider the maximum total transmit power to be used by the UE across all carriers in Uu and SL-U in FR1 which is indicated by NW (e.g., PEMAX,con-current)

- Option 3: Follow the same approach as Rel-17 inter-band concurrent operation

For the configured transmitted power, each PEMAX,c can be indicated to the corresponding Uu link and SL link. In addition, total PEMAX(e.g., PEMAX,con-current ) needs to be indicated to limit the total configured maximum transmitted power as EN-DC in which PEMAX, EN-DC is indicated as total PEMAX.

Option 1 seems not be different from option2 in aspect of concept. For the configured maximum output power, PPowerClass,con-current  and  PEMAX, con-current  need to be considered instead of PPowerClass_CA, PEMAX,CA in option 1.

Proposal 2: Consider the total UE configured maximum output power (e.g., PEMAX,con-current).

1-5-1: MSD for Uu@Licensed and SL@Un-licensed

In the last meeting, the following was agreed.

- Reuse the existing CA_n46-n78 MSD requirements for MSD of the inter-band concurrent operation, if power class 5 was assumed for the aggressor band n46 in the existing MSD requirement

In addition, CR [2] for CA_n46-n78 MSD requirements was agreed with power class 5 aggressor NR-U UL band as follows.

Table 7.3A.4-4d: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a power class 5 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n46 | n7 | 5 | 15 | 12 (RBstart=0) | 5 | 5.3 | NOTE 7 | UL1/DL2 |
| n46 | n7 | 5 | 15 | 24 (RBstart=0) | 30 | 0.8 | NOTE 7 | UL1/DL2 |
| n46 | n48 | 5 | 15 | 12 (RBstart=0) | 5 | 20.3 | NOTE 2 | UL2/DL3 |
| n46 | n48 | 20 | 15 | 100 (RBstart=0) | 100 | 9.7 | NOTE 2 | UL2/DL3 |
| n46 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 17.2 | NOTE 2 | UL2/DL3 |
| n46 | n78 | 20 | 15 | 100 (RBstart=0) | 100 | 9.7 | NOTE 2 | UL2/DL3 |
| n96 | n48 | 5 | 15 | 25 (RBstart=0) | 5 | 3.5 | NOTE 2 | UL2/DL3 |
| n96 | n48 | 20 | 15 | 100 (RBstart=0) | 15 | 0.4 | NOTE 2 | UL2/DL3 |
| NOTE 1: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (high) band (superscript HB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. NOTE 3: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 5: The requirements should be verified for DL NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 6: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 7: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

Table 7.3A.6-1c: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a power class 5 aggressor NR single UL band for DL NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n46 | n48 | 5190 | 80 | 30 | 216 (RBstart=0) | 3697.5 | 5 | 11 | >ACLR2 |
| n46 | n48 | 5190 | 80 | 30 | 216 (RBstart=0) | 3650 | 100 | 3.9 | >ACLR2 |
| n46 | n78 | 5190 | 80 | 30 | 216 (RBstart=0) | 3795 | 10 | 18.1 | >ACLR2 |
| n46 | n78 | 5190 | 80 | 30 | 216 (RBstart=0) | 3750 | 100 | 2.8 | >ACLR2 |
| n48 | n411 | 3570 | 40 | 15 | 216 (RBstart=0) | 2685 | 10 | 2.2 | >ACLR2 |
| n96 | n48 | 5965 | 80 | 30 | 216 (RBstart=0) | 3697.5 | 5 | 11 | >ACLR2 |
| n96 | n48 | 5965 | 80 | 30 | 216 (RBstart=0) | 3650 | 100 | 3.9 | >ACLR2 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. |  |  |  |  |  |  |  |  |  |

The MSD of Table 7.3A.4-4d and Table Table 7.3A.6-1c  can be reused for the band combination of Uu@n78 + SL@n46 with changing from UL to SL Tx, and from DL to SL Rx for n46.

Proposal 3: Reuse MSD requirements of inter band CA_n46-n78 for con-current operation with Uu@78 + SL@n46.

Conclusion

In this contribution, we provided our view on the remaining UE RF requirements of con-current operation on Uu and SL-U. Based on those, the followings are proposed.

Proposal 1: Define MOP as the total transmitted power from each operating band of Uu and SL.

Proposal 2: Consider the total UE configured maximum output power (e.g., PEMAX,con-current).

Proposal 3: Reuse MSD requirements of inter band CA_n46-n78 for con-current operation with Uu@78 + SL@n46.

- References

[1] R4-2310313, “WF on NR_SL_enh2_UERF_part2”, LG Electronics

[2] R4-2310422, “CR on MSD for shared spectrum band”, LG Electronics