3GPP TSG-RAN WG4 Meeting # 108	R4-2312260

# Toulouse, FR, Aug. 21 – 25, 2023

Source:	LG Electronics

Title:	Discussion for Lower MSD

Agenda item:	8.4.1.3.1

Document for:	Discussion

Introduction

In the last meeting, there were detailed discussions and agreements regarding the lower MSD capability signaling. In this paper, we would like to share our point of view for lower MSD threshold and .

Discussion

Lower MSD threshold

Table 1 WF[1] for low MSD thresholds

- Sub-topic 1-4: Candidate MSD thresholds - Candidate options: - Option 1: 3-bit solution with maximum threshold around 20dB (Samsung, Meta, QC, HW, ZTE, MTK) Index; Maximum allowed actual MSD (i.e. Thresholds); Lower MSD Capability classes; Note. 0; 0dB; Ⅰ; No degradation. 1; 3 dB; Ⅱ; Actual MSD ≤ 3dB. 2; 6 dB; Ⅲ; Actual MSD ≤ 6dB. 3; 9 dB; IV; Actual MSD ≤ 9dB. 4; 12 dB; Ⅴ; Actual MSD ≤ 12dB. 5; 15 dB; Ⅵ; Actual MSD ≤ 15dB. 6; 18 dB; Ⅶ; Actual MSD ≤ 18dB. 7; 21dB; Ⅷ; Actual MSD ≤ 21dB. - Option 2: Others - Agreement on Tuesday online session - The maximum threshold is around 20dB - FFS on the concrete values for thresholds - FFS on whether 2 or 3 bits will be used for threshold range.

HPUE RF requirements were specified in Rel-17, and the CA/DC band combinations for HPUE were also included in the spec. In Rel-18, the band combinations for HPUE are being added by HPUE basket WI. The MSD values for HPUE are higher than MSD value for PC 3 and Table 2 shows the example of MSD difference between the PC2 and PC3. In the future, there will be an increasing number of CA/DC band combinations with MSD values above 30 dB. If the maximum threshold for lower MSD is around 20 dB, the number of CA/DC band combinations that cannot be applied, even with an MSD improvement of 10 dB or more, will increase. Therefore, a higher threshold for lower MSD than 20 dB should be considered for CA/DC band combinations that have or will have an MSD above 30 dB.

Table 2 example of MSD difference between the PC2 and PC3.

Observation 1: The CA/DC band combinations for HPUE are being added by HPUE basket WI during Rel-17 and Rel-18, and the MSD values for HPUE band combination are higher than the MSD values for PC3.

Observation 2: It is necessary to consider the higher Lower MSD threshold for the CA/DC band combinations that have high MSD value.

Proposal: Consider maximum threshold of lower MSD to be 25 dB

Conclusion

Observation 1: The CA/DC band combinations for HPUE are being added by HPUE basket WI during Rel-17 and Rel-18, and the MSD values for HPUE band combination are higher than the MSD values for PC3.

Observation 2: It is necessary to consider the higher Lower MSD threshold for the CA/DC band combinations that have high MSD value.

Proposal: Consider maximum threshold of lower MSD to be 25 dB

- References

# [1] R4-2310499, WF on lower MSD (Huawei, HiSilicon)

# [2] R4-2304663, Discussion for lower MSD threshold (LGE)