3GPP TSG-RAN WG4 Meeting # 108	                      				R4-2312264

# Toulouse, France, August 21 – August 25, 2023

Source:	LG Electronics

Title:	UE RF requirements of SL CA

Agenda item:	8.30.2.3

Document for:	Discussion

Introduction

In this paper, we provide our views on the remaining issues of UE RF requirements for intra-band SL CA in Band n47 based on the WF[1].

Discussion

Regarding the WFs [1][2], the current status can be summarized in Table 2.1.

Table 2.1: Status of issues for intra-band SL CA in Band n47

| Issue | Status |
| --- | --- |
| 1-1: Spec suffix | Agreed |
| 1-2: Notation for NR SL CA | Agreed |
| 1-3: SCS for NR SL CA | Agreed |
| 2-1: Channel bandwidth class and aggregated channel BW for sidelink CA | Agreed |
| 2-2: The maximum aggregated CBW for sidelink CA | Agreed |
| 2-3: Configurations for sidelink CA | Agreed |
| 3-1: UE maximum output power for SL CA | Agreed |
| 3-2: MPR/A-MPR simulation assumptions for SL CA | Initial agreement |
| 3-3: Configured transmitted power for SL intra-band contiguous CA | Agreed |
| 3-4: Methodology on Tx requirements for SL intra-band contiguous CA | FFS |
| 4: Rx RF Requirements for SL CA | Agreed |

Issues to be clarified

Issue 1: Sidelink CA configuration / Bandwidth combination set

In the last meeting, the following bandwidth combinations were captured in [3].

Table 5.2.3-1: Intra-band contiguous CA operating bands for SL CA in FR1

| Sidelink CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sidelink CA configuration | Sidelink CA configuration for TX | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| SL_n47B | SL_n47B | [10] | [20,30] |  |  | 70 | 0 |
|  |  | [20] | [20,30] |  |  |  |  |
|  |  | 30 | [30],40 |  |  |  |  |

However, the bandwidth combination of 10MHz + 10MHz which was included in the agreement [2] is missed in [3].

  - Use Option 4 as baseline

| NR V2X intra-band con-current operating configuration | NR Band | Interface | Channel bandwidth (MHz) (NOTE 1) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| SL_n47B | n47 | PC5 | 10, 20, 30, 40 | 70 | 0 |
|  | n47 |  | 10, 20, 30 |  |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1 in TS38.101-1. |  |  |  |  |  |

Some Car OEM companies have big interest in the channel bandwidth combination of 10MHz + 10MHz. Therefore, this combination should be considered.

Proposal 1: Add the channel bandwidth combination of 10MHz + 10MHz for SL CA in Rel-18.

Issue-2 : MPR Simulation assumption for SL CA Agreement

The following was agreed in [2].

- For basic parameters, reuse the simulation assumptions in TR38.785 (Rel-17 enhanced NR sidelink). Other constraints for PSCCH/PSSCH/PSFCH/S-SSB can be assumed based on the current RAN1’s agreement.

| Center frequency | 5.9GHz |
| --- | --- |
| Bandwidth | per CC: 10/20/30/40MHz aggregated CBW: 20+40MHz/ 30+40MHz |
| Maximum output power for aggregated CBW | 23dBm |
| Numerology | 15 kHz/30kHz/60kHz |
| Modulation per CC | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| Carrier leakage | 25dBc |
| IQ image | 25dBc |
| CIM3 | FFS: 45dBc or, 45dBc and 60dBc |
| PA calibration | PA calibrated to deliver [31dBc] ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

Here, Carrier leakage and IQ image needs to be updated depending on the configured Modulation Order to meet the corresponding EVM requirements.

Proposal 2: Update carrier leakage and IQ image considering Modulation Order to meet the corresponding EVM requirements in simulation assumption.

- -28dB for QPSK, 16QAM, 64QAM

- -34dB for 256QAM

Issue 3: Methodology on Tx requirements for SL intra-band contiguous CA

- Open issue needs further discussion

  - Option 1: Reusing the existing requirements of NR intra-band contiguous CA

  - Option 2: LTE intra-band contiguous CA requirements are considered as baseline

  - Option 3: For further study

Option 1 is preferred.

Proposal 3: Maximize reusing the existing requirements of NR intra-band contiguous CA if needed.

MPR for SLCA PSSCH/PSCCH

Regarding that RAN1 is expected to start SL CA from August meeting, RAN4 needs to start simulation work for possible scenarios which are not impacted by RAN1 conclusion.

The existing RB allocations which are defined in the NR contiguous CA MPR can be reused for SL CA PSSCH/PSCCH MPR.

- Inner/Outer RB allocation in NR C-CA with contiguous RB allocations

- Inner/Outer1/Outer2 RB allocation in NR C-CA with non-contiguous RB allocations

Proposal 4: Reuse Inner/Outer RB allocation and Inner/Outer1/Outer2 RB allocation of NR C-CA for SL C-CA PSSCH/PSCCH MPR.

For Contiguous RB allocation of SL contiguous CA

The following simulation scenarios are considered as Table 2.2-1 for Contiguous RB allocations.

Table 2.2-1: SLCA PSSCH/PSCCH MPR simulation scenarios (Contiguous RB allocation)

| Aggregated CBW | Scenario | CC1 | CC2 | Inner/Outer RB allocation | SCS |
| --- | --- | --- | --- | --- | --- |
| 10MHz + 10MHz | 1 | 10RB42 | 10RB0 | Inner | 15 |
|  | 2 | 10RB42 | 12RB0 | Inner | 15 |
|  | 3 | 10RB42 | 15RB0 | Inner | 15 |
|  | 4 | 10RB42 | 25RB0 | Inner | 15 |
|  | 5 | 10RB42 | 30RB0 | Inner | 15 |
|  | 6 | 10RB42 | 36RB0 | Outer | 15 |
|  | 7 | 10RB42 | 50RB0 | Outer | 15 |
|  | 8 | 12RB40 | 36RB0 | Outer | 15 |
|  | 9 | 25RB27 | 36RB0 | Outer | 15 |
|  | 10 | 40RB12 | 40RB0 | Outer | 15 |
|  | 11 | 50RB2 | 50RB0 | Outer | 15 |
| 20MHz + 30MHz | 12 | 10RB41 | 10RB0 | Inner | 30 |
|  | 13 | 10RB41 | 12RB0 | Inner | 30 |
|  | 14 | 10RB41 | 25RB0 | Inner | 30 |
|  | 15 | 10RB41 | 30RB0 | Inner | 30 |
|  | 16 | 10RB41 | 48RB0 | Inner | 30 |
|  | 17 | 10RB41 | 50RB0 | Outer | 30 |
|  | 18 | 25RB26 | 36RB0 | Outer | 30 |
|  | 19 | 36RB15 | 36RB0 | Outer | 30 |
|  | 20 | 40RB11 | 40RB0 | Outer | 30 |
|  | 21 | 50RB1 | 75RB0 | Outer | 30 |
| 20MHz + 40MHz | 22 | 10RB41 | 10RB0 | Inner | 30 |
|  | 23 | 10RB41 | 25RB0 | Inner | 30 |
|  | 24 | 10RB41 | 36RB0 | Inner | 30 |
|  | 25 | 10RB41 | 48RB0 | Inner | 30 |
|  | 26 | 10RB41 | 60RB0 | Inner | 30 |
|  | 27 | 10RB41 | 70RB0 | Outer | 30 |
|  | 28 | 25RB26 | 36RB0 | Outer | 30 |
|  | 29 | 36RB15 | 70RB0 | Outer | 30 |
|  | 30 | 40RB11 | 90RB0 | Outer | 30 |
|  | 31 | 50RB1 | 105RB0 | Outer | 30 |

Table 2.2-2 and Figure 2.2-1 show the MPR simulation results.

Table 2.2-2: PSSCH/PSCCH MPR simulation results for SL Contiguous CA with Contiguous RB allocations

| '10MHz+10MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 0.13 | 0.13 | 0.14 | 0.13 | 0.13 | 0.95 | 2.70 | 1.27 | 1.60 | 2.32 | 3.09 |  |  |  |  |  |  |  |
|  | '16QAM' | 1.27 | 0.96 | 1.27 | 0.95 | 0.95 | 0.96 | 2.69 | 1.27 | 1.60 | 2.32 | 3.09 |  |  |  |  |  |  |  |
|  | '64QAM' | 2.31 | 2.31 | 2.31 | 2.31 | 2.30 | 2.31 | 2.69 | 2.31 | 2.31 | 2.32 | 3.09 |  |  |  |  |  |  |  |
|  | '256QAM' | 4.36 | 3.92 | 3.93 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 |  |  |  |  |  |  |  |
| '20MHz+30MHz' | Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 0.37 | 0.65 | 1.26 | 0.96 | 0.67 | 0.95 | 0.96 | 1.95 | 2.31 | 3.09 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 0.66 | 0.94 | 0.95 | 0.95 | 1.27 | 1.27 | 1.27 | 1.95 | 2.31 | 3.09 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 1.93 | 1.93 | 2.31 | 2.31 | 2.31 | 2.31 | 2.31 | 2.31 | 2.31 | 3.09 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 3.91 | 3.91 | 3.92 | 3.92 | 3.92 | 3.92 | 3.91 | 3.92 | 3.92 | 3.92 |  |  |  |  |  |  |  |  |
| '20MHz+40MHz' | Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 0.00 | 0.00 | 0.00 | 0.13 | 0.13 | 0.39 | 0.66 | 1.95 | 2.31 | 3.09 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 0.65 | 0.95 | 0.96 | 0.95 | 0.96 | 0.96 | 0.95 | 1.94 | 2.31 | 3.09 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 1.93 | 1.94 | 2.30 | 2.31 | 2.31 | 2.31 | 2.30 | 2.31 | 2.31 | 3.09 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 3.49 | 3.91 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 |  |  |  |  |  |  |  |  |

- 10MHz + 10MHz

- 20MHz + 30MHz

- 20MHz + 40MHz

Figure 2.2-1: PSSCH/PSCCH MPR simulation results for SL Contiguous CA with Contiguous RB allocations

Table 2.2-3 shows the maximum value of simulation results for SL Contiguous CA with Contiguous RB allocations considering Inner RB allocation and Outer RB allocation as NR uplink Contiguous CA.

Table 2.2-3: PSSCH/PSCCH MPR simulation results for SL Contiguous CA with Contiguous RB allocations

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | 1.26 | 3.09 |
|  | 16QAM | 1.27 | 3.09 |
|  | 64QAM | 2.31 | 3.09 |
|  | 256QAM | 4.36 | 3.92 |

The MPR can be proposed as Table 2.2-4 based on the simulation results when considering implementation margin.

Table 2.2-4: PSSCH/PSCCH MPR simulation results for SL Contiguous CA with Contiguous RB allocations

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 5.0 |
|  | 16QAM | ≤ 3.0 | ≤ 5.0 |
|  | 64QAM | ≤ 4.5 | ≤ 5.0 |
|  | 256QAM | ≤ 7.0 | ≤ 7.0 |

Proposal 5: Consider Table 2.2-4 for PSSCH/PSCCH MPR of SL C-CA with contiguous RB allocation.

For Non-contiguous RB allocation of SL contiguous CA

The following test scenarios are considered as Table 2.2-5 for Non-contiguous RB allocations.

Table 2.2-5: SL contiguous CA MPR test scenarios (Non-contiguous RB allocation)

| Aggregated CBW | Scenario | CC1 | CC2 | Inner/Outer1/Outer2 RB allocation | SCS |
| --- | --- | --- | --- | --- | --- |
| 10MHz + 10MHz | 1 | 10RB0 | 10RB42 | Outer2 | 15 |
|  | 2 | 10RB18 | 10RB22 | Outer2 | 15 |
|  | 3 | 10RB19 | 10RB22 | Outer2 | 15 |
|  | 4 | 10RB27 | 10RB14 | Outer2 | 15 |
|  | 5 | 10RB33 | 10RB7 | Outer1 | 15 |
|  | 6 | 10RB34 | 10RB7 | Outer1 | 15 |
|  | 7 | 10RB42 | 10RB1 | Inner | 15 |
|  | 8 | 25RB0 | 25RB27 | Outer2 | 15 |
|  | 9 | 25RB19 | 25RB8 | Outer2 | 15 |
|  | 10 | 25RB20 | 25RB8 | Outer2 | 15 |
|  | 11 | 25RB27 | 25RB1 | Outer1 | 15 |
|  | 12 | 30RB0 | 30RB22 | Outer2 | 15 |
|  | 13 | 30RB19 | 30RB3 | Outer2 | 15 |
|  | 14 | 30RB19 | 30RB2 | Outer2 | 15 |
|  | 15 | 30RB22 | 30RB1 | Outer1 | 15 |
|  | 16 | 50RB1 | 50RB0 | Outer2 | 15 |
| 20MHz + 30MHz | 17 | 10RB0 | 10RB68 | Outer2 | 30 |
|  | 18 | 10RB15 | 10RB32 | Outer2 | 30 |
|  | 19 | 10RB15 | 10RB31 | Outer2 | 30 |
|  | 20 | 10RB32 | 10RB10 | Outer1 | 30 |
|  | 21 | 10RB34 | 10RB9 | Outer1 | 30 |
|  | 22 | 10RB34 | 10RB8 | Outer1 | 30 |
|  | 23 | 10RB41 | 10RB1 | Inner | 30 |
|  | 24 | 25RB0 | 25RB53 | Outer2 | 30 |
|  | 25 | 25RB20 | 25RB25 | Outer2 | 30 |
|  | 26 | 25RB21 | 25RB25 | Outer2 | 30 |
|  | 27 | 25RB26 | 25RB1 | Outer1 | 30 |
|  | 28 | 30RB0 | 30RB48 | Outer2 | 30 |
|  | 29 | 30RB15 | 30RB12 | Outer2 | 30 |
|  | 30 | 30RB15 | 30RB11 | Outer2 | 30 |
|  | 31 | 30RB21 | 30RB1 | Outer1 | 30 |
|  | 32 | 50RB0 | 50RB28 | Outer2 | 30 |
|  | 33 | 50RB0 | 50RB0 | Outer2 | 30 |
|  | 34 | 50RB0 | 78RB0 | Outer2 | 30 |
| 20MHz + 40MHz | 35 | 10RB0 | 10RB96 | Outer2 | 30 |
|  | 36 | 10RB4 | 10RB30 | Outer2 | 30 |
|  | 37 | 10RB5 | 10RB30 | Outer2 | 30 |
|  | 38 | 10RB37 | 10RB15 | Outer1 | 30 |
|  | 39 | 10RB38 | 10RB15 | Inner | 30 |
|  | 40 | 10RB41 | 10RB1 | Inner | 30 |
|  | 41 | 25RB0 | 25RB81 | Outer2 | 30 |
|  | 42 | 25RB20 | 25RB38 | Outer2 | 30 |
|  | 43 | 25RB20 | 25RB37 | Outer2 | 30 |
|  | 44 | 25RB26 | 25RB1 | Outer1 | 30 |
|  | 45 | 30RB0 | 30RB76 | Outer2 | 30 |
|  | 46 | 30RB15 | 30RB26 | Outer2 | 30 |
|  | 47 | 30RB15 | 30RB25 | Outer2 | 30 |
|  | 48 | 30RB21 | 30RB1 | Outer1 | 30 |
|  | 49 | 50RB0 | 50RB56 | Outer2 | 30 |
|  | 50 | 50RB0 | 50RB0 | Outer2 | 30 |
|  | 51 | 50RB0 | 105RB0 | Outer2 | 30 |

Table 2.2-6 and Figure 2.2-2 show the MPR simulation results.

Table 2.2-6: PSSCH/PSCCH MPR simulation results for SL Contiguous CA with Non-contiguous RB allocations

| '10MHz+10MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 6.67 | 3.09 | 2.69 | 2.33 | 0.00 | 0.00 | 0.14 | 4.81 | 2.32 | 2.32 | 0.14 | 4.81 | 1.95 | 1.95 | 1.28 | 3.09 |  |  |
|  | '16QAM' | 6.67 | 2.70 | 3.10 | 2.70 | 0.95 | 0.97 | 0.96 | 5.26 | 2.32 | 2.32 | 0.96 | 4.81 | 1.95 | 1.95 | 1.28 | 3.09 |  |  |
|  | '64QAM' | 6.67 | 2.69 | 3.09 | 2.32 | 1.94 | 2.31 | 2.31 | 5.26 | 2.31 | 2.32 | 2.31 | 4.81 | 1.95 | 1.95 | 2.32 | 3.09 |  |  |
|  | '256QAM' | 7.15 | 3.92 | 3.93 | 3.93 | 3.92 | 3.92 | 3.93 | 4.81 | 3.92 | 3.92 | 3.92 | 4.81 | 3.92 | 3.92 | 3.92 | 3.92 |  |  |
| '20MHz+30MHz' | Scenario # | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 |
|  | 'QPSK' | 5.27 | 2.69 | 2.69 | 0.00 | 0.00 | 0.00 | 0.00 | 3.50 | 2.69 | 2.69 | 0.38 | 3.10 | 2.69 | 2.69 | 1.27 | 3.10 | 3.09 | 3.09 |
|  | '16QAM' | 5.27 | 2.70 | 2.69 | 0.66 | 0.66 | 0.65 | 0.66 | 3.50 | 2.70 | 2.69 | 0.96 | 3.09 | 2.69 | 2.69 | 1.27 | 3.09 | 3.09 | 3.09 |
|  | '64QAM' | 5.27 | 2.69 | 2.70 | 1.93 | 1.94 | 1.93 | 1.93 | 3.51 | 2.69 | 2.69 | 2.31 | 3.09 | 2.69 | 2.69 | 2.31 | 3.09 | 3.09 | 3.09 |
|  | '256QAM' | 5.26 | 3.50 | 3.49 | 3.48 | 3.91 | 3.49 | 3.49 | 3.50 | 3.50 | 3.49 | 3.91 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 |
| '20MHz+40MHz' | Scenario # | #35 | #36 | #37 | #38 | #39 | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 |  |
|  | 'QPSK' | 5.28 | 3.09 | 3.08 | 0.00 | 0.00 | 0.00 | 3.09 | 2.69 | 2.69 | 0.66 | 3.09 | 2.69 | 2.69 | 1.60 | 3.10 | 3.09 | 3.09 |  |
|  | '16QAM' | 5.26 | 3.08 | 3.08 | 0.65 | 0.65 | 0.65 | 3.09 | 2.69 | 2.69 | 0.95 | 3.10 | 2.69 | 2.69 | 1.60 | 3.09 | 3.09 | 3.09 |  |
|  | '64QAM' | 5.27 | 3.09 | 3.09 | 1.94 | 1.94 | 1.94 | 3.09 | 2.69 | 2.69 | 1.94 | 3.09 | 2.69 | 2.69 | 2.31 | 3.10 | 3.09 | 3.09 |  |
|  | '256QAM' | 5.27 | 3.49 | 3.49 | 3.49 | 3.48 | 3.49 | 3.50 | 3.49 | 3.49 | 3.91 | 3.92 | 3.50 | 3.92 | 3.92 | 3.92 | 3.92 | 3.92 |  |

- 10MHz + 10MHz

- 20MHz + 30MHz

- 20MHz + 40MHz

Figure 2.2-2: PSSCH/PSCCH MPR simulation results for SL Contiguous CA with Non-contiguous RB allocations

Table 2.2-7 shows the maximum value of simulation results for SL Contiguous CA with Non-contiguous RB allocations considering Inner RB allocation and Outer RB allocation as NR uplink Contiguous CA.

Table 2.2-7 : PSSCH/PSCCH MPR simulation results for SL Contiguous CA with Non-contiguous RB allocations

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | 0.14 | 3.08 | 6.67 |
|  | 16QAM | 0.97 | 3.10 | 6.67 |
|  | 64QAM | 2.31 | 3.09 | 6.67 |
|  | 256QAM | 3.93 | 3.93 | 7.15 |

The MPR can be proposed as Table 2.2-8 based on the simulation results when considering implementation margin.

Table 2.2-8 PSSCH/PSCCH MPR simulation results for SL Contiguous CA with Non-contiguous RB allocations

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 5.0 | ≤ 9.5 |
|  | 16QAM | ≤ 3.0 | ≤ 5.0 | ≤ 9.5 |
|  | 64QAM | ≤ 4.5 | ≤ 5.0 | ≤ 9.5 |
|  | 256QAM | ≤ 7.0 | ≤ 7.0 | ≤ 9.5 |

Proposal 6: Consider Table 2.2-8 for PSSCH/PSCCH MPR of SL C-CA with Non-contiguous RB allocation.

MPR for SLCA PSFCH

The following simulation scenarios are considered as Table 2.3-1 for SL contiguous CA.

Table 2.3-1: SL contiguous CA MPR test scenarios

| Aggregated CBW | Scenario | CC1 | CC2 | Ratio (R) | SCS |
| --- | --- | --- | --- | --- | --- |
| 10MHz + 10MHz | 1 | 1RB0 | - |  | 15 |
|  | 2 | 1RB51 | - |  | 15 |
|  | 3 | 1RB0 | 1RB0 | 0.5255 | 15 |
|  | 4 | 1RB0 | 1RB10 | 0.6185 | 15 |
|  | 5 | 1RB0 | 1RB20 | 0.7116 | 15 |
|  | 6 | 1RB0 | 1RB30 | 0.8046 | 15 |
|  | 7 | 1RB0 | 1RB40 | 0.8976 | 15 |
|  | 8 | 1RB0 | 1RB51 | 1.0 | 15 |
|  | 9 | 1RB10 | 1RB0 | 0.4324 | 15 |
|  | 10 | 1RB20 | 1RB0 | 0.3394 | 15 |
|  | 11 | 1RB30 | 1RB0 | 0.2463 | 15 |
|  | 12 | 1RB40 | 1RB0 | 0.1533 |  |
|  | 13 | 1RB51 | 1RB0 | 0.0509 | 15 |
| 20MHz + 30MHz | 14 | 1RB0 | - |  | 30 |
|  | 15 | 1RB50 | - |  | 30 |
|  | 16 | 1RB0 | 1RB0 | 0.4248 | 30 |
|  | 17 | 1RB0 | 1RB10 | 0.4995 | 30 |
|  | 18 | 1RB0 | 1RB20 | 0.5742 | 30 |
|  | 19 | 1RB0 | 1RB30 | 0.6489 | 30 |
|  | 20 | 1RB0 | 1RB40 | 0.7236 | 30 |
|  | 21 | 1RB0 | 1RB50 | 0.7983 | 30 |
|  | 22 | 1RB0 | 1RB60 | 0.8730 | 30 |
|  | 23 | 1RB0 | 1RB70 | 0.9477 | 30 |
|  | 24 | 1RB0 | 1RB77 | 1.0 | 30 |
|  | 25 | 1RB10 | 1RB0 | 0.3501 | 30 |
|  | 26 | 1RB20 | 1RB0 | 0.2754 | 30 |
|  | 27 | 1RB30 | 1RB0 | 0.2007 | 30 |
|  | 28 | 1RB40 | 1RB0 | 0.1260 | 30 |
|  | 29 | 1RB50 | 1RB0 | 0.0513 | 30 |
| 20MHz + 40MHz | 30 | 1RB0 | - |  | 30 |
|  | 31 | 1RB50 | - |  | 30 |
|  | 32 | 1RB0 | 1RB0 | 0.3509 | 30 |
|  | 33 | 1RB0 | 1RB10 | 0.4127 | 30 |
|  | 34 | 1RB0 | 1RB20 | 0.4745 | 30 |
|  | 35 | 1RB0 | 1RB30 | 0.5363 | 30 |
|  | 36 | 1RB0 | 1RB40 | 0.5981 | 30 |
|  | 37 | 1RB0 | 1RB50 | 0.6600 | 30 |
|  | 38 | 1RB0 | 1RB60 | 0.7218 | 30 |
|  | 39 | 1RB0 | 1RB70 | 0.7836 | 30 |
|  | 40 | 1RB0 | 1RB80 | 0.8454 | 30 |
|  | 41 | 1RB0 | 1RB90 | 0.9073 | 30 |
|  | 42 | 1RB0 | 1RB100 | 0.9691 | 30 |
|  | 43 | 1RB0 | 1RB105 | 1.0 | 30 |
|  | 44 | 1RB10 | 1RB0 | 0.2890 | 30 |
|  | 45 | 1RB20 | 1RB0 | 0.2272 | 30 |
|  | 46 | 1RB30 | 1RB0 | 0.1654 | 30 |
|  | 47 | 1RB40 | 1RB0 | 0.1036 | 30 |
|  | 48 | 1RB50 | 1RB0 | 0.0417 | 30 |

Here, the aggregated CA bandwidth is CA bandwidth class B which is composed of 2 CCs (CC1 + CC2). The ratio of NGap/(NRB1+NRB2+ NGBchannel_CC1+ NGBchannel_CC2) is considered.

- R = NGap/(NRB1+NRB2+ NGBchannel_CC1+ NGBchannel_CC2)

  - NGap is the gap RB amount from CC1 RBstart  to CC2 RBend for SL contiguous CA when a single PSFCH or contiguous and non-contiguous allocation simultaneous PSFCHs is transmitted in each CC.

  - NGBchannel_CC1 and NGBchannel_CC2 is the number of RB which corresponds to the minimum guardbands for CC1 and CC2, respectively, which have been calculated using the following equation:

NGBchannel =  GBchannel / (SCS x 12)

GBchannel = (BWChannel - NRB x SCS x 12) / 2 - SCS/2,

where NRB are from Table 5.3.2-1 in TS38.101-1, and BWChannel is the channel bandwidth for each CC.

Figure 2.3-0 shows NGap, NRB1, NRB2, NGBchannel_CC1, and NGBchannel_CC2.

Figure 2.3-0: Parameters related to Ratio of R

Table 2.3-2 and Figure 2.3-1 show the PSFCH MPR simulation results.

Table 2.3-2: PSFCH MPR simulation results for SL Contiguous CA

| '10MHz+10MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 | #12 | #13 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2.66 | 0.00 | 6.91 | 6.91 | 11.14 | 11.14 | 10.68 | 10.70 | 2.04 | 0.84 | 0.00 | 0.00 | 0.00 |  |  |  |  |  |
| '20MHz+30MHz' | Scenario # | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 |  |  |
|  |  | 1.59 | 0.00 | 6.37 | 5.88 | 5.40 | 10.63 | 11.08 | 10.63 | 10.63 | 10.17 | 10.18 | 2.04 | 0.84 | 0.00 | 0.00 | 0.00 |  |  |
| '20MHz+40MHz' | Scenario # | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |  |  |  |  |  |  |
|  |  | 2.58 | 0.00 | 1.68 | 6.39 | 5.90 | 5.90 | 11.10 | 10.64 | 10.18 | 9.71 |  |  |  |  |  |  |  |  |
|  | Scenario # | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 |  |  |  |  |  |  |  |  |
|  |  | 10.64 | 11.10 | 10.64 | 10.65 | 1.22 | 0.84 | 0.00 | 0.00 | 0.00 | 10.64 |  |  |  |  |  |  |  |  |

- 10MHz + 10MHz

- 20MHz + 30MHz

- 20MHz + 40MHz

Figure 2.3-1: PSFCH MPR simulation results for SL Contiguous CA

Table 2.3-3 shows the maximum value of simulation results for SL Contiguous CA considering the ratio of R.

R = NGap/(NRB1+NRB2+ NGBchannel_CC1+ NGBchannel_CC2)

Table 2.3-3 : PSFCH MPR simulation results for SL Contiguous CA

| MPR for ratio (R) in bandwidth class B(dB) |  |  |
| --- | --- | --- |
| R ≤ 0. 33 | 0.33 < R ≤ 0. 55 | 0.55 < R ≤ 1.0 |
| 1.22 | 6.91 | 11.14 |

The MPR can be proposed as Table 2.3-4 based on the simulation results when considering implementation margin.

Table 2.3-4 PSFCH MPR simulation results for SL Contiguous CA

| MPR for ratio (R) in bandwidth class B(dB) |  |  |
| --- | --- | --- |
| R ≤ 0. 33 | 0.33 < R ≤ 0. 55 | 0.55 < R ≤ 1.0 |
| 3.5 | 9.0 | 13.0 |
| Here, R = NGap/(NRB1+NRB2+ NGBchannel_CC1+ NGBchannel_CC2) |  |  |

Proposal 7: Consider Table 2.3-4 for PSFCH MPR of SL C-CA.

MPR for SLCA S-SSB

The following simulation scenarios are considered as Table 2.4-1 for SL contiguous CA.

Table 2.4-1: SL contiguous CA MPR test scenarios

| Aggregated CBW | Scenario | CC1 | CC2 | Inner/Outer1/Outer2 RB allocation | SCS |
| --- | --- | --- | --- | --- | --- |
| 10MHz + 10MHz | 1 | 11RB0 | 11RB41 | Outer2 | 15 |
|  | 2 | 11RB18 | 11RB22 | Outer2 | 15 |
|  | 3 | 11RB20 | 11RB22 | Outer1 | 15 |
|  | 4 | 11RB33 | 11RB17 | Outer1 | 15 |
|  | 5 | 11RB35 | 11RB7 | Inner | 15 |
|  | 6 | 11RB41 | 11RB0 | Inner | 15 |
| 20MHz + 30MHz | 7 | 11RB0 | 11RB67 | Outer2 | 30 |
|  | 8 | 11RB15 | 11RB32 | Outer2 | 30 |
|  | 9 | 11RB15 | 11RB30 | Outer1 | 30 |
|  | 10 | 11RB33 | 11RB7 | Outer1 | 30 |
|  | 11 | 11RB34 | 11RB7 | Inner | 30 |
|  | 12 | 11RB40 | 11RB0 | Inner | 30 |
| 20MHz + 40MHz | 13 | 11RB0 | 11RB95 | Outer2 | 30 |
|  | 14 | 11RB4 | 11RB30 | Outer2 | 30 |
|  | 15 | 11RB15 | 11RB30 | Outer1 | 30 |
|  | 16 | 11RB33 | 11RB7 | Outer1 | 30 |
|  | 17 | 11RB34 | 11RB7 | Inner | 30 |
|  | 18 | 11RB40 | 11RB0 | Inner | 30 |

Table 2.4-2 and Figure 2.4-1 show the S-SSB MPR simulation results.

Table 2.4-2: S-SSB MPR simulation results for SL Contiguous CA

| '10MHz+10MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10.21 | 6.30 | 6.20 | 1.25 | 1.08 | 1.50 |
| '20MHz+30MHz' | Scenario # | #7 | #8 | #9 | #10 | #11 | #12 |
|  |  | 8.20 | 4.52 | 4.38 | 1.14 | 1.09 | 1.46 |
| '20MHz+40MHz' | Scenario # | #13 | #14 | #15 | #16 | #17 | #18 |
|  |  | 8.08 | 4.50 | 4.62 | 1.23 | 1.06 | 1.25 |

Figure 2.4-1: S-SSB MPR simulation results for SL Contiguous CA

Table 2.4-3 shows the maximum value of simulation results for SL Contiguous CA considering Inner, Outer1, and Outer 2 RB allocation ranges.

Table 2.4-3: S-SSB MPR simulation results for SL Contiguous CA

| MPR for ratio (R) in bandwidth class B(dB) |  |  |
| --- | --- | --- |
| Inner | Outer1 | Outer2 |
| 1.50 | 6.20 | 10.21 |

The MPR can be proposed as Table 2.4-4 based on the simulation results when considering implementation margin.

Table 2.4-4 S-SSB MPR simulation results for SL Contiguous CA

| MPR for ratio (R) in bandwidth class B(dB) |  |  |
| --- | --- | --- |
| Inner | Outer1 | Outer2 |
| 3.5 | 9.0 | 13.0 |

Proposal 8: Reuse Inner/Outer RB allocation and Inner/Outer1/Outer2 RB allocation of NR C-CA for SL C-CA S-SSB MPR.

Proposal 9: Consider Table 2.4-4 for S-SSB MPR of SL C-CA.

Conclusion

In this contribution, we provided our view on the remained issues and MPR simulation results for SL intra-band contiguous CA. Based on those, the followings are proposed.

Proposal 1: Add the channel bandwidth combination of 10MHz + 10MHz for SL CA in Rel-18.

Proposal 2: Update carrier leakage and IQ image considering Modulation Order to meet the corresponding EVM requirements in simulation assumption.

- -28dB for QPSK, 16QAM, 64QAM

- -34dB for 256QAM

Proposal 3: Maximize reusing the existing requirements of NR intra-band contiguous CA if needed.

MPR

Proposal 4: Reuse Inner/Outer RB allocation and Inner/Outer1/Outer2 RB allocation of NR C-CA for SL C-CA PSSCH/PSCCH MPR.

Proposal 5: Consider Table 2.2-4 for PSSCH/PSCCH MPR of SL C-CA with contiguous RB allocation.

Table 2.2-4: PSSCH/PSCCH MPR simulation results for SL Contiguous CA with Contiguous RB allocations

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 5.0 |
|  | 16QAM | ≤ 3.0 | ≤ 5.0 |
|  | 64QAM | ≤ 4.5 | ≤ 5.0 |
|  | 256QAM | ≤ 7.0 | ≤ 7.0 |

Proposal 6: Consider Table 2.2-8 for PSSCH/PSCCH MPR of SL C-CA with Non-contiguous RB allocation.

Table 2.2-8 PSSCH/PSCCH MPR simulation results for SL Contiguous CA with Non-contiguous RB allocations

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 5.0 | ≤ 9.5 |
|  | 16QAM | ≤ 3.0 | ≤ 5.0 | ≤ 9.5 |
|  | 64QAM | ≤ 4.5 | ≤ 5.0 | ≤ 9.5 |
|  | 256QAM | ≤ 7.0 | ≤ 7.0 | ≤ 9.5 |

Proposal 7: Consider Table 2.3-4 for PSFCH MPR of SL C-CA.

Table 2.3-4 PSFCH MPR simulation results for SL Contiguous CA

| MPR for ratio (R) in bandwidth class B(dB) |  |  |
| --- | --- | --- |
| R ≤ 0. 33 | 0.33 < R ≤ 0. 55 | 0.55 < R ≤ 1.0 |
| 3.5 | 9.0 | 13.0 |
| Here, R = NGap/(NRB1+NRB2+ NGBchannel_CC1+ NGBchannel_CC2) |  |  |

Proposal 8: Reuse Inner/Outer RB allocation and Inner/Outer1/Outer2 RB allocation of NR C-CA for SL C-CA S-SSB MPR.

Proposal 9: Consider Table 2.4-4 for S-SSB MPR of SL C-CA.

Table 2.4-4 S-SSB MPR simulation results for SL Contiguous CA

| MPR for ratio (R) in bandwidth class B(dB) |  |  |
| --- | --- | --- |
| Inner | Outer1 | Outer2 |
| 3.5 | 9.0 | 13.0 |

- References

[1] R4-2310314, “WF on NR_SL_enh2_UERF_part3”, Huawei, Hisilicon

[2] R4-2306637, “WF on SL CA”, Huawei, Hisilicon

[3] R4-2310312, “TP for TR 38.786 on the updated TR structure for NR SL CA operation”, Meta Ireland