3GPP TSG-RAN WG4 Meeting # 108															R4-2312266

Toulouse, France, August 21 – August 25, 2023

Agenda item:			8.30.2.3 Sidelink CA

Source:					LG Electronics

Title:	TP for TR 38.786 on the SLCA channel bandwidth and MPR

Document for:	Approval

- Background

This contribution provides text proposal on the channel bandwidth and MPR.

- Text Proposal

<<<<<<<<<<< Start of changes in section 1 >>>>>>>>>>

5.2.3	Channel bandwidth for NR SL CA operation

For NR SL CA operation, the SL CA channel bandwidths for each operating band is specified in Table 5.2.3-1.

Table 5.2.3-1: Intra-band contiguous CA operating bands for SL CA in FR1

| Sidelink CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sidelink CA configuration | Sidelink CA configuration for TX | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| SL_n47B | SL_n47B | [10] | [10, 20,30] |  |  | 70 | 0 |
|  |  | [20] | [20,30] |  |  |  |  |
|  |  | 30 | [30],40 |  |  |  |  |

<<<<<<<<<<< End of changes in section 1 >>>>>>>>>>

<<<<<<<<<<< Start of changes in section 2 >>>>>>>>>>

- 6.3	Tx requirements for NR SL CA operation

- 6.3.1	Maximum output power for NR SL CA operation

For the intra-band SL CA operation, the following NR SL CA UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least one sub frame (1ms).

Table 6.3.1-1: NR SL CA UE Power Class

| NR SL CA band Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X_n47B |  |  |  |  | 23 | +2/-33 |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: For intra-band SL CA UE, the maximum power requirement apply to the total transmitted power over all component carriers (per UE). NOTE 3: 3 refers to the transmission bandwidths (Figure 5.6-1 in TS38.101-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |  |  |  |  |  |  |

- 6.3.2	UE maximum output power reduction for NR SL CA operation

For basic parameters, reuse the simulation assumptions in TR38.785 (Rel-17 enhanced NR sidelink). Other constraints for PSCCH/PSSCH/PSFCH/S-SSB can be assumed based on current RAN1’s agreement.

| Center frequency | 5.9GHz |
| --- | --- |
| Bandwidth | per CC: 10/20/30/40MHz Aggregated CBW: Table 5.2.3-1 (up to 70MHz CBW) |
| Maximum output power for aggregated CBW | 23dBm |
| Numerology | 15 kHz/30kHz/60kHz |
| Modulation per CC | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| Carrier leakage | 28dBc, 34dBc |
| IQ image | 28dBc, 34dBc |
| CIM3 | FFS: 45dBc or, 45dBc and 60dBc |
| PA calibration | PA calibrated to deliver 30 dB ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

6.3.2.1 MPR for NR SL CA operation

6.3.2.1.1	MPR for simultaneous PSSCH/PSCCH transmission

For intra-band contiguous NR SL carrier aggregation, the allowed Maximum Power Reduction (MPR) for the maximum output power in 6.3.1-1 with contiguous RB allocation is specified in Table 6.3.2.1.1-1 for UE power class 3 CA bandwidth classes B. The Inner/Outer RB allocation is specified in 6.2A.2.1 in TS 38.101-1.

For intra-band contiguous carrier aggregation, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table Table 6.3.1.1-1 with non-contiguous RB allocation is specified in Table 6.3.2.1.1-2 for UE power class 3 CA bandwidth classes B. The Inner/Outer1/Outer2 RB allocation is specified in 6.2A.2.1 in TS 38.101-1.

Table 6.3.2.1.1-1: PSSCH/PSCCH MPR simulation results for SL Contiguous CA with Contiguous RB allocations

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 5.0 |
|  | 16QAM | ≤ 3.0 | ≤ 5.0 |
|  | 64QAM | ≤ 4.5 | ≤ 5.0 |
|  | 256QAM | ≤ 7.0 | ≤ 7.0 |

Table 6.3.2.1.1-2: PSSCH/PSCCH MPR simulation results for SL Contiguous CA with Non-contiguous RB allocations

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | 0.14 | 3.08 | 6.67 |
|  | 16QAM | 0.97 | 3.10 | 6.67 |
|  | 64QAM | 2.31 | 3.09 | 6.67 |
|  | 256QAM | 3.93 | 3.93 | 7.15 |

6.3.2.1.2	MPR for PSFCH transmission

For intra-band contiguous NR SL carrier aggregation, the allowed Maximum Power Reduction (MPR) for the maximum output power in 6.3.1-1 is specified in Table 6.3.2.1.2-1 for UE power class 3 CA bandwidth classes B.

Table 6.3.2.1.2-1 PSFCH MPR simulation results for SL Contiguous CA

| MPR for ratio (R) in bandwidth class B(dB) |  |  |
| --- | --- | --- |
| R ≤ 0. 33 | 0.33 < R ≤ 0. 55 | 0.55 < R ≤ 1.0 |
| 3.5 | 9.0 | 13.0 |
| Here, R = NGap/(NRB1+NRB2+ NGBchannel_CC1+ NGBchannel_CC2) |  |  |

Here, the aggregated CA bandwidth is CA bandwidth class B which is composed of 2 CCs (CC1 + CC2). The ratio of NGap/(NRB1+NRB2+ NGBchannel_CC1+ NGBchannel_CC2) is considered.

- R = NGap/(NRB1+NRB2+ NGBchannel_CC1+ NGBchannel_CC2)

  - NGap is the gap RB amount from CC1 RBstart  to CC2 RBend for SL contiguous CA when a single PSFCH or contiguous and non-contiguous allocation simultaneous PSFCHs is transmitted in each CC.

  - NGBchannel_CC1 and NGBchannel_CC2 is the number of RB which corresponds to the minimum guardbands for CC1 and CC2, respectively, which have been calculated using the following equation:

NGBchannel =  GBchannel / (SCS x 12)

GBchannel = (BWChannel - NRB x SCS x 12) / 2 - SCS/2,

where NRB are from Table 5.3.2-1 in TS38.101-1, and BWChannel is the channel bandwidth for each CC.

6.3.2.1.3	MPR for S-SSB transmission

For intra-band contiguous NR SL carrier aggregation, the allowed Maximum Power Reduction (MPR) for the maximum output power in 6.3.1-1 is specified in Table 6.3.2.1.3-1 for UE power class 3 CA bandwidth classes B. The Inner/Outer1/Outer2 RB allocation is specified in 6.2A.2.1 in TS 38.101-1.

Table 6.3.2.1.3-1 S-SSB MPR simulation results for SL Contiguous CA

| MPR for ratio (R) in bandwidth class B(dB) |  |  |
| --- | --- | --- |
| Inner | Outer1 | Outer2 |
| 3.5 | 9.0 | 13.0 |

<<<<<<<<<<< End of changes in section 2 >>>>>>>>>>

- References

[1] R4-2312264, “UE RF requirements of SL CA”, LG Electronics

[2] TR 38.786 v0.2.0