3GPP TSG-RAN WG4 Meeting #108		R4-2312272

# Toulouse, France, August 21 – August 25, 2023

Source:	LG Electronics

Title:	4Tx UE RF requirements

Agenda item:	8.4.1.1

Document for:	Discussion

Introduction

In this paper, we provide our views on 4Tx UE RF requirements based on the approved WF [1].

Discussion

Based on the WF[1] which is captured below, we would like to further discuss the following issues.

- Power class and Reference architecture for Stage-2 configurations - In Rel-18, focus on power class 1.5. - In Stage-2 of Rel-18 WI, only consider 4x23, 4x26 and 2x23 + 2x26. - PCMAX tolerance for 4Tx based UL-MIMO/TxDiversity - Use the following table as a as a starting point of PCMAX tolerance for 4Tx-based UL-MIMO / TxDiversity. and further discuss the values to be specified - Table 4: PCMAX,c tolerance for 4 Tx UL-MIMO PCMAX,c (dBm); Tolerance TLOW(PCMAX_L,c) (dB); Tolerance THIGH(PCMAX_H,c) (dB). 26 ≤ PCMAX,c ≤ 29; 3.0; 2.0. 25 ≤ PCMAX,c < 26; 5.0; 2.0. 24 ≤ PCMAX,c < 25; 5.0; 3.0. 23 ≤ PCMAX,c < 24; 5.0; 4.0. 19 ≤ PCMAX,c < 23; 5.0. 14 ≤ PCMAX,c < 19; 6.0. -40 ≤ PCMAX,c < 14; 7.0.

Power class and Reference architecture for Stage-2 configurations

  - In Rel-18, focus on power class 1.5.

  - In Stage-2 of Rel-18 WI, only consider 4x23, 4x26 and 2x23 + 2x26.

For 4x23dBm, the maximum output power can be,

  - PC1.5 with 4 antenna ports

  - PC2 with 2 antenna ports

  - PC3 with 1 antenna port

For 2x23dBm+2x26dBm, the maximum output power can be,

  - PC1.5 with 4 antenna ports

  - PC1.5 with 2 antenna ports

  - PC3, or PC2 with 1 antenna port

For 4x26dBm, the maximum output power can be,

  - PC1.5 with 4 antenna ports

  - PC1.5 with 2 antenna ports

  - PC2 with 1 antenna port

To reuse the existing UE power classes as much as possible, when configured with 2 antenna ports, a same PA pair is assumed as baseline, e.g., (2x23dBm) or (2x26dBm).

For the reference architecture with same PA pair for 2 Tx, MPR can be applied with Table 2.1 for no TxDiversity.

Table 2.1: MPR applicability for 4 Tx PC1.5 without Tx diversity

| PA configuration | # of antenna ports | Applicable Power class | MPR | Related Spec. |
| --- | --- | --- | --- | --- |
| 4x23dBm | 4 | PC1.5 | MPR for PC1.5 with 4 Tx | Table 6.2D.2-4, Table 6.2D.2-5 (R4-2307144) |
|  | 2 | PC2 | MPR for PC2 with dual Tx | Table 6.2D.2-1 |
|  | 1 | PC3 | MPR for PC3 | Table 6.2.2-1 |
| 2x23dBm + 2x26dBm | 4 | PC1.5 | MPR for PC1.5 with 4 Tx | Table 6.2D.2-4, Table 6.2D.2-5 (R4-2307144) |
|  | 2 | PC1.5 | MPR for PC1.5 with dual Tx | Table 6.2D.2-2 |
|  | 2 | PC2 | MPR for PC2 with dual Tx | Table 6.2D.2-1 |
|  | 1 | PC2 | MPR for PC2 | Table 6.2.2-2 |
|  | 1 | PC3 | MPR for PC3 | Table 6.2.2-1 |
| 4x26dBm | 4 | PC1.5 | MPR for PC1.5 with 4 Tx | Table 6.2D.2-4, Table 6.2D.2-5 (R4-2307144) |
|  | 2 | PC1.5 | MPR for PC1.5 with dual Tx | Table 6.2D.2-2 |
|  | 1 | PC2 | MPR for PC2 | Table 6.2.2-2 |

For the reference architecture with same PA pair for 2 Tx, MPR can be applied with Table 2.2 for TxDiversity.

Table 2.2: MPR applicability for 4 Tx PC1.5 with Tx diversity

| PA configuration | # of Tx diversity | Applicable Power class | MPR | Related Spec. |
| --- | --- | --- | --- | --- |
| 4x23dBm | 4 | PC1.5 | MPR for PC1.5 with 4 Tx | Table 6.2D.2-4, Table 6.2D.2-5 (R4-2307144) |
|  | 2 | PC2 | MPR for PC2 with dual Tx | Table 6.2D.2-1 |
|  | 1 | PC3 | MPR for PC3 | Table 6.2.2-1 |
| 2x23dBm + 2x26dBm | 4 | PC1.5 | MPR for PC1.5 with 4 Tx | Table 6.2D.2-4, Table 6.2D.2-5 (R4-2307144) |
|  | 2 | PC1.5 | MPR for PC1.5 with dual Tx | Table 6.2D.2-2 |
|  | 2 | PC2 | MPR for PC2 with dual Tx | Table 6.2D.2-1 |
|  | 1 | PC2 | MPR for PC2 | Table 6.2.2-2 |
|  | 1 | PC3 | MPR for PC3 | Table 6.2.2-1 |
| 4x26dBm | 4 | PC1.5 | MPR for PC1.5 with 4 Tx | Table 6.2D.2-4, Table 6.2D.2-5 (R4-2307144) |
|  | 2 | PC1.5 | MPR for PC1.5 with dual Tx | Table 6.2D.2-2 |
|  | 1 | PC2 | MPR for PC2 | Table 6.2.2-2 |

For PA configurations of 2x23dBm + 2x26dBm, and 4x23dBm, the same MPR of PC1.5 with 4Tx (4x23dBm) can be applied.

Proposal 1: Apply the same MPR of PC1.5 with 4Tx (4x23dBm) to both 2x23dBm+2x26dBm and 4x26dBm.

PCMAX tolerance for 4Tx based UL-MIMO/TxDiversity

If there is no technical issue, Table 4 in [1] is proposed as PCMAX tolerance for 4Tx UL-MIMO.

Proposal 2: Define PCMAX tolerance for 4Tx UL-MIMO/TxDiversity with Table 4 in [1].

Table 4: PCMAX,c tolerance for 4 Tx UL-MIMO/TxDiversity

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| 26 ≤ PCMAX,c ≤ 29 | 3.0 | 2.0 |
| 25 ≤ PCMAX,c < 26 | 5.0 | 2.0 |
| 24 ≤ PCMAX,c < 25 | 5.0 | 3.0 |
| 23 ≤ PCMAX,c < 24 | 5.0 | 4.0 |
| 19 ≤ PCMAX,c < 23 | 5.0 |  |
| 14 ≤ PCMAX,c < 19 | 6.0 |  |
| -40 ≤ PCMAX,c < 14 | 7.0 |  |

Conclusion

In this contribution, we provided our view on 4Tx UE RF requirements. Based on those, the followings are proposed.

Proposal 1: Apply the same MPR of PC1.5 with 4Tx (4x23dBm) to both 2x23dBm+2x26dBm and 4x26dBm.

Proposal 2: Define PCMAX tolerance for 4Tx UL-MIMO/TxDiversity with Table 4 in [1].

Table 4: PCMAX,c tolerance for 4 Tx UL-MIMO/TxDiversity

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| 26 ≤ PCMAX,c ≤ 29 | 3.0 | 2.0 |
| 25 ≤ PCMAX,c < 26 | 5.0 | 2.0 |
| 24 ≤ PCMAX,c < 25 | 5.0 | 3.0 |
| 23 ≤ PCMAX,c < 24 | 5.0 | 4.0 |
| 19 ≤ PCMAX,c < 23 | 5.0 |  |
| 14 ≤ PCMAX,c < 19 | 6.0 |  |
| -40 ≤ PCMAX,c < 14 | 7.0 |  |

- References

[1] R4-2310278, “WF on 4Tx UE RF requirements”, vivo

[2] R4-2213725, “Work plan for Rel-18 FR1 UE RF enhancement”, Huawei, HiSilicon, NTT DOCOMO, INC

[3] RP-230753, “Revised WID on Further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1)”, Huawei, HiSilicon