3GPP TSG-RAN WG4 Meeting #108		R4-2312274

# Toulouse, France, August 21 – August 25, 2023

Source:	LG Electronics

Title:	UE Tx requirement for inter-band CA/EN-DC PC1.5 with 3Tx

Agenda item:	7.29.3.1

Document for:	Discussion

Introduction

In this paper, we provide our views on SAR issue for inter-band CA/EN-DC PC1.5 with 3Tx based on the WF[1].

Discussion

Based on the WF[1],  the SAR issue needs to be solved.

- 2-1-1: PC1.5 inter-band CA SAR compliance - Option 1 is agreed, i.e. 0.5* maxUplinkDutyCycle-interBandCA-PC2-r17 is applied as the max Uplink duty cycle that PC1.5 inter-band UL CA capability. - If the averaged percentage of uplink symbols is below 0.5* maxUplinkDutyCycle-interBandCA-PC2-r17 the requirements for PC1.5 applies; - If the averaged percentage of uplink symbols is larger than 0.5* maxUplinkDutyCycle-interBandCA-PC2 but less than maxUplinkDutyCycle-interBandCA-PC2, the requirements for PC2 applies; - otherwise the requirements of default power class apply. - If maxUplinkDutyCycle-interBandCA-PC2 is absent, UE shall work on power class PC1.5 regardless of UL duty cycle and may use P-MPRc as defined in 6.2.4 in TS 38.101-1 or other means if necessary. - 2-1-2: Text proposal for SAR solutions in 38.101-1 - WF: FFS whether below are agreeable in next meeting: - the averaged percentage of UL symbols can be defined as: 50% * (DutyNR,x / maxDutyNR,x + DutyNR,y / maxDutyNR,x). - the maxDutyNR,x is 100% for the band which is PC3 - the maxDutyNR,y is the max Uplink duty cycle capability for the PC1.5 band which is the smaller of maxUplinkDutyCycle-PC1dot5-MPE-FR1 IE and 0.5*maxUplinkDutyCycle-PC2-FR1 IE

The last sub-bullet in the issue 2-1-2 is not clear as Table 2-1. For example, when maxUplinkDutyCycle-PC1dot5-MPE-FR1-r1 is not absent but maxUplinkDutyCycle-PC2-FR1 is absent, the maxDutyNR,y value is not clear.

Table 2-1 summarizes the difference of maxDutyNR,y  between the proposal 3[3] and option1 of issue 2-1-1.

Table 2-1:  Comparison of maxDutyNR,y  ( proposal 3 [3] vs Option1 of issue2-1-1)

| NR Band y Power Class | NR Band y |  |  |  |
| --- | --- | --- | --- | --- |
|  | maxUplinkDutyCycle-PC2-FR1 | maxUplinkDutyCycle-PC1dot5-MPE-FR1-r1 | maxDutyNR,y |  |
|  |  |  | Proposal 3 [3] | Option1 of issue 2-1-1 |
| PC1.5 | present | present | maxUplinkDutyCycle-PC1dot5-MPE-FR1-r1 | Min(maxUplinkDutyCycle-PC1dot5-MPE-FR1 , 0.5*maxUplinkDutyCycle-PC2-FR1) |
|  | present | absent | 25% | ? |
|  | absent | present | maxUplinkDutyCycle-PC1dot5-MPE-FR1-r1 | ? |
|  | absent | absent | 25% | ? |
| PC2 | present | present | maxUplinkDutyCycle-PC2-FR1 | ? |
|  | present | absent | maxUplinkDutyCycle-PC2-FR1 | ? |
|  | absent | present | 50% | ? |
|  | absent | absent | 50% | ? |
| PC3 | present/absent | present/absent | 100% | 100% |

Based on Table 2-1, Proposal 3[3] is more clear solution than option 1 of issue 2-1-1. Therefore, RAN4 needs to consider maxUplinkDutyCycle-PC1dot5-MPE-FR1-r1 as maxDutyNR,y.

Based on this, the following is proposed.

Proposal 1: Update the corresponding maxDutyNR,x or maxDutyNR,y

- maxDutyNR,x, maxDutyNR,y represent the field of UE capability maxUplinkDutyCycle-PC2-FR1 per band or   maxUplinkDutyCycle-PC1dot5-MPE-FR1-r1 per band as defined in TS 38.331.  For NR Band x or NR Band y,

  - if power class of one or both of the bands within the band combination is power class 1.5 and the corresponding UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1-r1 is absent;

    - the corresponding maxDutyNR,x or maxDutyNR,y is equal to 25%;

  - else if power class of one or both of the bands within the band combination is power class 2 and the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent;

    - the corresponding maxDutyNR,x or maxDutyNR,y is equal to 50%;

  - else if the band is configured with power class 3;

    - the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

Conclusion

In this contribution, we provided our view on the SAR issue for inter-band CA PC1.5 with 3Tx. The following is proposed.

Proposal 1: Update the corresponding maxDutyNR,x or maxDutyNR,y

- maxDutyNR,x, maxDutyNR,y represent the field of UE capability maxUplinkDutyCycle-PC2-FR1 per band or   maxUplinkDutyCycle-PC1dot5-MPE-FR1-r1 per band as defined in TS 38.331.  For NR Band x or NR Band y,

  - if power class of one or both of the bands within the band combination is power class 1.5 and the corresponding UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1-r1 is absent;

    - the corresponding maxDutyNR,x or maxDutyNR,y is equal to 25%;

  - else if power class of one or both of the bands within the band combination is power class 2 and the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent;

    - the corresponding maxDutyNR,x or maxDutyNR,y is equal to 50%;

  - else if the band is configured with power class 3;

    - the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

- References

[1] R4-2310409, “WF on UE RF requirements for 3Tx and 4Rx”, Oppo

[2] RP-230161, “Revised WID for Low NR band 4Rx and 3Tx”, Oppo

[3] R4-2307486,” UE Tx requirement for inter-band CA/EN-DC PC1.5 with 3Tx”, LG Electronics