3GPP TSG-RAN4 Meeting # 108	R4-2312310

Toulouse, France, August 21 – August 25, 2023

Source: 	Huawei, HiSilicon

Title: 	TP to TR 38.858: Co-site inter-sector interference analysis for FR1 Wide Area BS

Agenda Item:	8.19.2.2.1

Document for:	Approval

- Introduction

SBFD feasibility study and RF impact from BS aspects were discussed in last meeting and WF for the feasibility from BS aspect was approved in [1].

Agreement:

- The TR section “10.2 Feasibility of FR1 Wide Area BS aspects” shall be further broken-down to harmonize the common understating from RAN4 if possible and summarize the input from companies.

In this contribution, we provide some text proposal for the section 10.2 Co-site inter-sector interference analysis based on our input in [2] and [3].

- Text Proposal

### 10.2.2	Co-channel inter-sub-band co-site inter-sector interference analysis

Editor's note: This section captures the typical assumption of RF requirements and analysis results.

### 10.2.2.1	Summary table for co-channel inter-sub-band co-site inter-sector interference analysis

Editor's note: This section captures the summary table which is based on co-channel inter-sub-band co-site inter-sector interference analysis framework.

| FR1 (or FR2-1) |  |  |  | Company-A |
| --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 53 dBm |
| Number of co-site co-channel sectors considered |  |  |  | 2 |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαCSSI-frequency, TX = ② dBc |  | 45 dBc |
|  |  | Frequency isolation techniques used |  | DPD |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαCSSI-spatial = ③ dBc |  | 85 dBc |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used |  | Spatial separation between TX panel with absorbing material and choke structure. |
|  | TX Beam nulling /isolation of inter-sector interference in TX sub-band dBαCSSI-beam = ④ dBc |  |  | 10 dB |
|  | DL EIRP impact due to beam nulling in TX sub-band (considering all nulling for self- and inter-sector interference) |  |  | Less than 0.5 dB loss |
|  | Interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPCSSI_leakage due to inter-sector interference (Note 1) |  |  | -74 dBm |
|  | Interference signal in gNB TX subband, measured at the input of LNA (Note 1) due to inter-sector interference |  |  | -39 dBm |
|  | Blocker Suppression at RX | Frequency isolation capability dBαCSSI-frequency, RX= ⑥ dBc |  | sub-band analog filter: 10 dB digital filter: 60-80 dB |
|  |  | Frequency isolation techniques |  | sub-band analog filter put after LNA; digitla filter |
|  |  | RX IMD | Rx IIP3 capability (dBm) | 0 dBm |
|  |  |  | Rx IM3 contribution (dBm) | -127 |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | Negligible |
|  | Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized due to co-site inter-sector co-channel interference only dBPCSSI_Selectivity(Note 1, 2) |  |  | Note: Each company to explain if/how they have separated CSSI and SI when considering IM3 |
|  | RX Beam nulling /isolation in RX sub-band dBαCSSI-beam = ⑨ dBc |  |  | 10 dB |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | Less than 0.5 dB loss |
|  | Digital processing interference supression capability |  |  | 12 |
| Total interference in RX SB (dBm) (Note 2) |  |  |  | -102 dBm/20 MHz |
| Noise floor ⑩dBm |  |  |  | -96 dBm/CBW |
| Calculated Desensitization (dB) |  |  |  | 1 dB |
| SBFD configuration |  |  |  |  | DUD [40, 20, 40] |  |
| Guardband assumption (if exist) |  |  |  |  | Existing SU |  |
| bandwidth over which suppression is achieved |  |  |  | Several hundred MHz |
| Others |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. Note 4: The abbreviation CSSI refers to co-site co-channel inter-sector interference in this table |  |  |  |  |

### 10.2.2.2	Feasibility study on co-channel inter-sub-band co-site inter-sector interference

Editor's note: This section captures the feasibility study on co-channel inter-sub-band co-site inter-sector interference based on individual companies’ analysis.

### 10.2.2.2.1	Huawei

Editor's note: Individual company may provide the analysis assumption/configuration used for the corresponding analysis summarized in 10.2.2.1. Additionally, the views on the preference/views on component technology and corresponding trade-off can be provided and analysed.

On digital IC aspect, in our view, since the information of non-linear product is already got in the digital domain, and it can be exchanged between sectors within a BBU, hence digital IC can be applied for this case.

On the achievable coupling loss in the case of co-site inter-sector gNB-gNB, RF measurements for the spatial isolation has been conducted. Figure 10.2.2.2.1-1 shows a measurements results for two 120° sectors which is 2.5 m distance in horizontal. The curve represents the spatial isolation between a TX full power transmission of one sector to one receiver chain of the other sector. Form the measurements it can be found that the larger angle beam steering, the isolation becomes smaller. The isolation at worst case is ~76 dB. It is a test on legacy AAS BS. There are some method can be adopted to improve the isolation for a BS capable of SBFD operation, e.g. mounting EM absorber materials next to the antenna array in the base station. ~10 dB improvement is foreseen based on our evaluations. For co-site inter-sector case better spatial isolation than RSI case is achievable.

Figure 2.3-1: spatial isolation measurements

Figure 2.3-2: measurements results at different beam directions

### 10.2.2.2.2	[Company Name]

### 10.2.2.2.3	[Company Name]

### 10.2.2.3	Conclusion

- Conclusion

In this contribution, we provide some text proposal on co-site inter-sector interference analysis for FR1 WA BS.

- Reference

[1] R4-2306005, “WF on implementation feasibility of SBFD”, Samsung

[2] R4-2301426,	“Feasibility study from BS aspect”, Huawei, HiSilicon

[3] R4-2305302, “Discussion on the feasibility of FR1 BS aspects”, Huawei, HiSilicon