3GPP TSG-RAN4 Meeting # 108	R4-2312312

Toulouse, France, August 21 – August 25, 2023

Source: 	Huawei, HiSilicon

Title: 	TP to TR 38.858: Co-site inter-sector interference analysis for FR2 BS

Agenda Item:	8.19.2.2.2

Document for:	Approval

- Introduction

SBFD feasibility study and RF impact from BS aspects were discussed in last meeting and WF for the feasibility from BS aspect was approved in [1].

Agreement:

- The TR section “10.2 Feasibility of FR1 Wide Area BS aspects” shall be further broken-down to harmonize the common understating from RAN4 if possible and summarize the input from companies.

- The same TR section break-down as FR1 WA BS can be applied to FR2-1 BS.

In this contribution, we provide some text proposal for the section 10.5.2 based on our input in [2] and [3].

- Text Proposal

### 10.5.2	Co-channel inter-sub-band co-site inter-sector interference analysis

Editor's note: This section captures the typical assumption of RF requirements and the analysis results

### 10.5.2.1	Summary table for co-channel inter-sub-band co-site inter-sector interference analysis

Editor's note: This section captures the summary table which is based on co-channel inter-sub-band co-site inter-sector interference analysis framework.

The Co-channel inter-sub-band co-site inter-sector interference analysis from companies’ input for FR2 SBFD-capable gNB is summarized in Table 10.5.2.1-1. Both self-interference leakage in gNB RX subband due to non-ideal TX and Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity are studied in the analysis framework. The interference cancellation techniques including spatial isolation, frequency isolation, beam nulling and digital/RF cancellation are considered in the analysis framework.

Table 10.5.2.1-1: Co-channel inter-sub-band co-site inter-sector interference analysis

| FR2-1 |  |  |  | Huawei |
| --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 35 dBm |
| Number of co-site co-channel sectors considered |  |  |  | 2 |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαCSSI-frequency, TX = ② dBc |  | 28 dB |
|  |  | Frequency isolation techniques used |  | DPD |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαCSSI-spatial = ③ dBc |  | 100 dB |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used |  | Spatial separation between TX/RX panel and surface wave suppression techniques |
|  | TX Beam nulling /isolation of inter-sector interference in TX sub-band dBαCSSI-beam = ④ dBc |  |  | - |
|  | DL EIRP impact due to beam nulling in TX sub-band (considering all nulling for self- and inter-sector interference) |  |  | - |
|  | Interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPCSSI_leakage due to inter-sector interference (Note 1) |  |  | -96 |
|  | Interference signal in gNB TX subband, measured at the input of LNA (Note 1) due to inter-sector interference |  |  | -62 |
|  | Blocker Suppression at RX | Frequency isolation capability dBαCSSI-frequency, RX= ⑥ dBc |  | - |
|  |  | Frequency isolation techniques |  | - |
|  |  | RX IMD | Rx IIP3 capability (dBm) | - |
|  |  |  | Rx IM3 contribution (dBm) | - |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | - |
|  | Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized due to co-site inter-sector co-channel interference only dBPCSSI_Selectivity(Note 1, 2) |  |  | - |
|  | RX Beam nulling /isolation in RX sub-band dBαCSSI-beam = ⑨ dBc |  |  | - |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | - |
|  | Digital processing interference supression capability |  |  | - |
| Total interference in RX SB (dBm) (Note 2) |  |  |  | -96 dBm |
| Noise floor ⑩dBm |  |  |  | -88 dBm/40 MHz |
| Calculated Desensitization (dB) |  |  |  | 0.6 dB |
| SBFD configuration |  |  |  | DUD |
| Guardband assumption (if exist) |  |  |  | Existing SU |
| bandwidth over which suppression is achieved |  |  |  | Several GHz |

### 10.5.2.2	Feasibility study on co-channel inter-sub-band co-site inter-sector interference

Editor's note: This section captures the feasibility study on co-channel inter-sub-band co-site inter-sector interference based on individual companies’ analysis.

### 10.5.2.2.1	Huawei

Editor's note: Individual company may provide the analysis assumption/configuration used for the corresponding analysis summarized in 10.5.2.1. Additionally, the views on the preference/views on component technology and corresponding trade-off can be provided and analysed.

For FR2 inter-sector isolation, some measurements based on existing modules are proceed as below. The AAS mounting in the same mast would be worse cases for radiated isolation. The following two cases are the typical site deployments for the worse cases. The left one (Case 1) is up-down installation and the right one (case 2)is 120° installation.

For each case, 60*60=3600 beam combinations are measured. The results are shown in Table 10.5.2.2.1-1 for existing modules and Table 10.5.2.2.1-2 for improved modules. Some surface wave suppression measures can be used to improve the isolation, e.g. reflection and absorption structure. Using these measures, more than 10 dB improvement can be achieved for the case with poor isolation. As shown in in the measurements, 100 dB inter-sector isoaltion is achieveable for FR2

Table 10.5.2.2.1-1: Inter-sector isolation for existing modules

| Test cases | Existing modules | 95% CDF | 90% CDF | 50% CDF |
| --- | --- | --- | --- | --- |
| Case 1 | up-down installation | 96 | 99 | 103 |
| Case 2 | 120°installation | 85 | 90 | 100 |

Table 10.5.2.2.1-2: Inter-sector isolation for improved modules

| Test cases | Improved modules | 95% CDF | 90% CDF | 50% CDF |
| --- | --- | --- | --- | --- |
| Case 1 | up-down installation | 101 | 103 | 104 |
| Case 2 | 120°installation | 100 | 101 | 102 |

### 10.5.2.2.2	[Company Name]

### 10.5.2.2.3	[Company Name]

### 10.5.2.3	Conclusion

Editor's note: This section captures the conclusion for feasibility study on co-channel inter-sub-band co-site inter-sector interference based on RAN4 agreement.

Based on the provided studies in clause 10.5.2.1, it can be concluded that it is feasible to meet the 1 dB desensitization target at least for FR2 BS with TX output power <= 35 dBm.

- Reference

[1] R4-2306005, “WF on implementation feasibility of SBFD”, Samsung

[2] R4-2301426,	“Feasibility study from BS aspect”, Huawei, HiSilicon

[3] R4-2305303, “Discussion on the feasibility of FR2 BS aspects”, Huawei, HiSilicon