3GPP TSG-RAN WG4 Meeting #108	R4-2312337

# Toulouse, France,  21st - 25th August, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | <CR#> | rev | <Rev#> | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on interruption requirements for UE reporting NFG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-08-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New interruption requirements apply when a NeedforGapInfoNR capable UE measures inter-frequency MO-s without measurement gap but with interruptions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces the applicability and the interruption length/ratio allowed for a UE measuring inter-frequency MO-s without measurement gap but with interruptions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirement is applied. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 8.2.2.2.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <<Start of Change1>>

##### 8.2.2.2.X	Interruptions due to measurements without gap carried out by UE supporting NeedForGapInfoNR

When a UE supports NeedForGapInfoNR measurements on inter-frequency and inter-RAT Measurement Objects without measurement gap but with interruptions, if the network configures UE Measurement Objects and applicable measCycleNFG and there is no measurement gap configured for the MO-s, the UE is allowed to cause interruptions due to retuning of its vacant RF chain to the target MO where measurement without gap but with interruption is carried out.

When DRX is not configured, the UE is allowed to cause interruption per Tcycle period on a specific frequency layer that corresponds to the configured MO, where Tcycle is the interruption cycle specified in sub-clause [TBD]. When there are multiple MO-s configured to the UE where the UE is allowed to cause intrruptions to the serving cell due to measurements on these MO-s without measurement gap, the allowed the interruption from measurements on any of the MO-s is at longest X slots according to the SCS in the serving cell per smallest Tcycle length among all.

The requirements of X slots are speficied in the below table 8.2.2.2.X-1 for different numerologies.

Table 8.2.2.2.X-1: Interruption length X

|  | SCS (kHz) | NR Slot | Interruption length X (slots) |
| --- | --- | --- | --- |
|  |  | length (ms) |  |
| 0 | 15 | 1 | 1 |
| 1 | 30 | 0.5 | 1 |
| 2 | 60 | 0.25 | 3 |
| 3 | 120 | 0.125 | 5 |
| 5 | 480 | 0.03125 | 17 |
| 6 | 960 | 0.015625 | 33 |

Editors’ note: Discussion is ongoing on cases where DRX or measurement gap is configured. Further update to this sub-clause subjects to the conclusions of those discussions.

### <<End of Change1>>

### <<Start of Change2>>

### 9.1.Y	Measurements without measurement gap for UE supporting NeedForGapInfoNR

#### 9.1.Y.1	Introduction

The UE capable of NeedForGapInfoNR measurements without measurement gap can be configured with NR and inter-RAT measurement objectives without measurement gap configuration.

The measurement period and reporting requirements are specified in sub-clause [TBD] for the UE capable of NeedForGapInfoNR without measurement gap and with or without interruption caused. The allowed interruption requirements are specified correspdingly for UE capable of NeedForGapInfoNR measurements without gap but with interruption in sub-clause [TBD].

### <<End of Change2>>