3GPP TSG-RAN WG4 Meeting #108		R4- 2312338

Toulouse, France, 21st – 25th Aug, 2023

Agenda item:	8.34.4.2

Source:	Intel Corporation

Title:	NR network energy saving RRM aspects – SSB-less SCell

Document for:	Discussion

# Introduction

Following the SI of NR network energy saving, a WI is approved in RANP#98-e in Dec. 2022. The WID can be seen in [1]. The objectives are copied in the below box.

- Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2] - Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3] - Note: No change for SSB transmission due to cell DTX/DRX. - Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided. - Specify the following techniques in spatial and power domains - Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2] - Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2] - Note: Above objectives are only for UE specific channels/signals - Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements - Specify mechanism(s) to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, if necessary [RAN2] - Specify CHO procedure enhancement(s) in case source/target cell is in NES mode [RAN2] - Specify inter-node beam activation and enhancements on restricting paging in a limited area [RAN3]. - Specify the corresponding RRM/RF core requirements, if necessary, for the above features [RAN4]

In April WG meeting, agreements were made facilitating initial progress of this item. The agreements are captured in the approved WF R4-2306369 [2]. In May WG meeting, the agreements made are captured in the WF R4-2310086 [3].

In this paper we provide our views on RRM aspects that are related to the discussions within WI scope.

# Discussion

## SSB-less SCell in inter-band CA

Scenarios

In the last working group meeting we had an extensive discussion on how the scenarios of SSB-less SCell are identified in this work item in Rel-18. The group has identified 3 scenarios and tried to make prioritization among all. The previously identified scenarios are as follows.

- Scenario 1: No SSB but with TRS transmission configured on the SSB-less SCell

  - FFS whether to consider requirements based on TRS transmission during and/or after SCell activation

- Scenario 2: No SSB and no TRS transmission configured on the SSB-less SCell

- Scenario 2a: No DL transmission but with UL reception at the NW side on the SSB-less SCell.

The group decided to deprioritize scenario 2 in the last meeting. The agreements are captured in the below box.

- Agreements - Continue RAN4 work on the following SSB-less SCell scenarios - Scenario 1: SCell without SSB transmission and with TRS transmission - Scenario 2a: SCell without SSB transmission and without any other DL transmissions, but with UL reception at the NW side - Note: No RAN1 impacts are expected, and no RAN4 requirements will be defined if the scenario is not supported from RAN1 specification perspective. - Deprioritize RAN4 work on the following SSB-less SCell scenario - Scenario 2: SCell without SSB transmission and without TRS transmission - Send LS to RAN1/2 to check on support of Scenario 2a from RAN1/2 specifications perspective

Note that there was controversy on scenario 2a in the final discussion and the LS mentioned was not sent due to no consensus on the detailed content. RANP also touched this discussion, but no consensus was reached either in June.

## Scenario 1: without SSB but with TRS transmission

The FR1 SCell activation requirements for SSB-less target intra-band SCell which is contiguous in frequency with the PCell were at first introduced in the Rel-15 specification. The idea is that the UE is supposed to use the information on the PCell to derive needed information in order to activate the intra-band contiguous target SCell. Indeed, there are prerequisite conditions to be met to facilitate the applicability of the requirements.

If the SCell being activated belongs to FR1 and if there is at least one active serving cell contiguous to the SCell on that FR1 band, if the UE is not provided with SSB configuration (absoluteFrequencySSB) nor SMTC configuration for the target SCell, Tactivation_time is 3 ms for UE supporting scellWithoutSSB, provided - The RTD between the target SCell and the contiguous active serving cell is within within ±260ns, and - The difference of the reception power with the contiguous active serving cell is <= 6dB, and - The RS(s) of SCell being activated is (are) QCL-TypeA with TRS(s) of the SCell being activated, and the TRS(s) of the SCell being activated is (are) further QCL-TypeC with SSB(s) of any active serving cell that is contiguous to the SCell being activated on that FR1 band.

In the network energy saving work item, we have the below objective,

- Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2]

This is to extend the activation requirements of SSB-less SCell to also support inter-band CA. It is natural that the idea of this objective is the same as in the previous spec about intra-band contiguous CA: the UE is supposed to make use of reference signals transmitted in another carrier to derive needed info on the target SCell without SSB transmitted.

In order to extend the applicability to inter-band CA, we need to examine all the prerequisite conditions to see if there is need to modify or apply without change.

Reference timing difference

For intra-band contiguous cases, it is required that the reference timing difference is less than 260ns between the two intra-band contiguous cells. But in inter-band CA cases, it is difficult to have RTD between two cells less than 3us, even when they are collocated deployed. 260ns cannot be easily achieved when the frequency separation between the carriers is large. This can be seen in the MRTD and MTTD requirements specified for the inter-band CA deployments. Please see the below copy of the MRTD requirements specified in TS 38.133 where 33us considers 3us RTD and 31us propagation delay difference.

Table 7.6.4-2: Maximum receive timing difference requirement for inter-band NR carrier aggregation Frequency Range of the pair of carriers; Maximum receive timing difference (µs). FR1; 33.

Thus, when it comes to the inter-band CA cases, in order to apply the 3ms SCell activation delay on the SSB-less target cell, it is required as one of the prerequisite conditions that

- The RTD between the target inter-band SCell (without SSB) and the active serving cell with SSB is within ±3us

Of course, there are cases where potential enhancements on RTD can be considered, when the bands are close to each other, but in order to have a unified requirement RAN4 needs to consider the minimum requirements.

Proposal 1: In order to apply the 3ms SCell activation delay on the SSB-less target cell, it is required as one of the prerequisite conditions that the RTD between the target inter-band SCell (without SSB) and the active serving cell with SSB is within ±3us.

Power imbalance

The power difference between the target cell and the reference cell configured with SSB is also important for the UE to correctly carry out SCell activation on the SSB-less target. In order for the UE to use the measured results on the SSB cell to derive info to the SSB-less target cell, the power difference between the two cells has to be smaller than a certain threshold. In the intra-band contiguous case, <6dB is required.

Though it is often seen that between inter-band carriers, the power imbalance is much larger compared to intra-band cases, we need to guarantee the SCell activation performance under a correct deployment and configuration from the network. Since we assume that this kind of requirement only applies to collocated scenarios, we propose to keep the 6dB maximum imbalance value.

Proposal 2: The inter-band SSB-less SCell activation delay requirements apply then the difference of the received powers between target SCell and the active serving cell with SSB is <= 6dB.

QCL relations among RS-s

The primary condition to apply the 3ms activation requirement is that TRS (tracking reference signal, periodic non zero powered CSI-RS) has to be guaranteed on the target SCell. Even with intra-band contiguous cases, TRS is required to be transmitted on the target SCell and the TRS has to be QCLed with the SSB transmitted in the other serving cell.

Once TRS is transmitted on the target SCell, the UE is able to use the TRS to carry out frequency/time tracking and CSI measurements and reporting. The performance then is largely guaranteed and that is why in the existing activation requirements for intra-band contiguous CA, TRS is transmitted on the target cell.

Proposal 3: TRS which is QCLed with the SSB on an active serving cell is transmitted on the inter-band SSB-less target SCell to guarantee fair performance of the UE activation.

In the last meeting, companies had proposed a scenario where there is not any signal transmitted in DL but only UL. In general this scenario is a UL CA deployment without DL corresponding band in the SCell. This requires the UE to support the cross carrier scheduling capabilities to receive scheduling on PCell but to transmit UL signals on SCell. In this case, the SCell activation requirements are unique compared to what we had in the TS 38.133 spec since we never touched cross carrier scheduling and SCell without DL in the legacy releases.

With this said our view on this scenario is to carry out feasibility study on it to analyse whether we could specify requirements in Rel-18 to enable SSB-less SCell operations.

L1 measurements on the SSB-less SCell

As long as the L1 measurements are correctly configured, the UE is required to meet the L1-RSRP requirements as the same way as legacy. There is no need to specify any new requirement for this case.

L3 measurements on the SSB-less SCell

Since there is no SSB transmitted on the SCell, the network does not configure SSB based L3 measurements for the UE on the frequency layer. There is no need to specify any requirement.

Proposal 4: No new measurement requirement is specified for L1 or L3 measurements for SSB-less SCell operations.

## Scenario 2a: without SSB and without any other DL signal

Scenario 2a was proposed and discussed where there is no SSB transmission nor any other DL signal transmission on the target SCell before or after activation. It is quite controversial whether this scenario is feasible to be included in the discussions to specify RRM requirements in this work item.

This is a different SCell activation framework than the one that we talked about in this paper about scenario 1. Neither does the UE use QCLed TRS from the target SCell to help carry out fast activation nor does the network guarantee any power imbalance due to no DL transmission of any form at all.

In this case, the UE does not carry out any operation on the target frequency layer for SCell activation at all. In a RRM test, it is not clear how to test the UE against any activation delay requirement for an SCell that does not transmit DL signal at all. Even if we were to introduce activation delay core requirements, it made little sense if there was no meaningful test against them.

Plus, there is no need to specify any new requirement in the RRM spec for the UE to support this feature if RAN1/2 have already had the complete spec for this network configuration. As there is no change in the RAN1 or RAN2 spec, existing UE is certainly required to support this scenario 2a activation and a fair performance is guaranteed for the UE transmissions on the UL SCell. The functions such as power control and transmit timing in general should be guaranteed by the existing specs of 3GPP.

Proposal 5: No activation delay requirement is specified for scenario 2a.

Proposal 6: Existing interruption requirements still apply to UE configured with scenario 2a SCell activations.

## SSB-less SCell in intra-band non-contiguous CA

There is no reason to leave intra-band non-contiguous CA deployment out of scope in terms of discussions on the 3ms activation requirements for SSB-less SCell.

At least all the requirements specified for inter-band CA can be applied to intra-band non-contiguous CA. We propose that together with the specification of inter-band cases, RAN4 specifies requirements also for intra-band non-contiguous CA.

Proposal 7: Together with the specification of inter-band cases, RAN4 specifies requirements also for intra-band non-contiguous CA.

Proposal 8: Activation requirements specified for inter-band SSB-less SCell also apply to intra-band non-contiguous SSB-less SCell activation.

# Conclusion

In this paper we provide our initial views on RRM aspects that are related to the discussions within WI scope.

Proposal 1: In order to apply the 3ms SCell activation delay on the SSB-less target cell, it is required as one of the prerequisite conditions that the RTD between the target inter-band SCell (without SSB) and the active serving cell with SSB is within ±3us.

Proposal 2: The inter-band SSB-less SCell activation delay requirements apply then the difference of the received powers between target SCell and the active serving cell with SSB is <= 6dB.

Proposal 3: TRS which is QCLed with the SSB on an active serving cell is transmitted on the inter-band SSB-less target SCell to guarantee fair performance of the UE activation.

Proposal 4: No new measurement requirement is specified for L1 or L3 measurements for SSB-less SCell operations.

Proposal 5: No activation delay requirement is specified for scenario 2a.

Proposal 6: Existing interruption requirements still apply to UE configured with scenario 2a SCell activations.

Proposal 7: Together with the specification of inter-band cases, RAN4 specifies requirements also for intra-band non-contiguous CA.

Proposal 8: Activation requirements specified for inter-band SSB-less SCell also apply to intra-band non-contiguous SSB-less SCell activation.

# References

- RP-223540, New WID: Network energy savings for NR, Huawei

- R4-2306369, WF on RRM requirements for NR network energy saving, Huawei

- R4-2310086, WF on RRM requirements for NR network energy saving, Huawei