3GPP TSG-RAN4 Meeting #108	R4-2312352

Toulouse, France, August 21 – August 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR |  | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on 8Rx PDSCH demodulation requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_FR1_enh2-Perf |  |  |  |  |  | Date: |  |  | 2023-08-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Add requirements for FR1 8Rx PDSCH demodulation performance |  |  |  |  |  |  |  |  |
| Summary of change: |  | The summary of changes in this CR as below: - add requirments in Clause 5.2.4 5.2.4.1 5.2.4.1.1 5.2.4.2 5.2.4.2.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - No requirement for FR1 8Rx PDSCH demodulation performance |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.4 5.2.4.1 5.2.4.1.1 5.2.4.2 5.2.4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

5.2.4	8RX requirements

5.2.4.1	FDD

5.2.4.1.1	Minimum requirements for PDSCH Mapping Type A

The performance requirements are specified in Table 5.2.4.1.1-3, Table 5.2.4.1.1-4 and Table 5.2.4.1.1-5, with the addition of test parameters in Table 5.2.4.1.1-2 and the downlink physical channel setup according to Annex C.3.1.

The test purpose are specified in Table 5.2.4.1.1-1.

Table 5.2.4.1.1-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 8 receive antenna conditions and with different channel models, MCSs and number of MIMO layers | 1-1, 2-1, 3-1 |

Table 5.2.4.1.1-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 for rank <= 4 2 for rank > 4 |
|  | DMRS ports indexes |  | {1000,1001} for Rank2 {1000-1003} for Rank4 {1000-1007} for Rank8 |
| Codebook configuration | CodebookType |  | typeI-SinglePanel for 4Tx and 8Tx |
|  | CodebookMode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) for 4Tx (4,1) for 8Tx |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
|  | RI Restriction |  | 00000010 for rank 2 00001000 for rank 4 10000000 for rank 8 |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| NZP CSI-RS for CSI acquisition | Row index |  | 6 for 8 CSI-RS ports |
| Number of HARQ Processes |  |  | 4 |
| Maximum HARQ transmissions |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

Table 5.2.4.1.1-3: Minimum performance for Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | [R.PDSCH.1-3.1 FDD] | 10 / 15 | [64QAM, 0.50] | TDLC300-100 | 2x8, ULA Medium B | 70 | [TBD] |

Table 5.2.4.1.1-4: Minimum performance for Rank 4

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | [R.PDSCH.1-3.X FDD] | 10 / 15 | [64QAM, 0.43] | TDLA30-10 | 4x8, ULA Low | 70 | [TBD] |

Table 5.2.4.1.1-5: Minimum performance for Rank 8

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | [R.PDSCH.1-3.Y FDD] | 10 / 15 | [64QAM, 0.43] | TDLA30-10 | 8x8, ULA Low | 70 | [TBD] |

5.2.4.2	TDD

5.2.4.2.1	Minimum requirements for PDSCH Mapping Type A

The performance requirements are specified in Table 5.2.4.2.1-3, Table 5.2.4.2.1-4 and Table 5.2.4.2.1-5, with the addition of test parameters in Table 5.2.4.2.1-2 and the downlink physical channel setup according to Annex C.3.1.

The test purpose are specified in Table 5.2.4.2.1-1.

Table 5.2.4.2.1-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 8 receive antenna conditions and with different channel models, MCSs and number of MIMO layers | 1-1, 2-1, 3-1 |

Table 5.2.4.2.1-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 for rank <= 4 2 for rank > 4 |
|  | DMRS ports indexes |  | {1000,1001} for Rank2 {1000-1003} for Rank4 {1000-1007} for Rank8 |
| Codebook configuration | CodebookType |  | typeI-SinglePanel for 4Tx and 8Tx |
|  | CodebookMode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) for 4Tx (4,1) for 8Tx |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
|  | RI Restriction |  | 00000010 for rank 2 00001000 for rank 4 10000000 for rank 8 |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| NZP CSI-RS for CSI acquisition | Row index |  | 6 for 8 CSI-RS ports |
| Number of HARQ Processes |  |  | 8 |
| Maximum HARQ transmissions |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

Table 5.2.4.2.1-3: Minimum performance for Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | [R.PDSCH.2-3.1 TDD] | 40 / 30 | [64QAM, 0.50] | FR1.30-1 | TDLC300-100 | 2x8, ULA Medium B | 70 | [TBD] |

Table 5.2.4.2.1-4: Minimum performance for Rank 4

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | [R.PDSCH.2-3.X TDD] | 40 / 30 | [64QAM, 0.43] | FR1.30-1 | TDLA30-10 | 4x8, ULA Low | 70 | [TBD] |

Table 5.2.4.2.1-5: Minimum performance for Rank 8

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | [R.PDSCH.2-3.Y TDD] | 40 / 30 | [64QAM, 0.43] | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | [TBD] |

<End of changes>