3GPP TSG-RAN4 Meeting #108	R4-2312353

Toulouse, France, August 21 – August 25, 2023

Source:	Samsung

Title:	discussion on FR2 Multi-Rx PDSCH demodulation requirements

Agenda item:	8.7.4.2

Document for:	Discussion

- Introduction

In last RAN4 #107 meeting, the discussion of general test scope and the new correlation matrix for FR2 Multi-Rx demodulation performance requirements has got significant progress, simulation assumptions for FR2 Multi-Rx PDSCH requirements have some agreements in WF [1]. In this contribution, we provide our views on open issues.

Issue 1-1-2: On new correlation matrix for FR2-1 in a Multi-TRP and Multi-Rx context for OTA demodulation performance requirements. Agreement: Consider the baseline correlation model assuming cross-talk between the TRPs and UE Rx antenna as below R16×16=Eh16×1h16×1H=11+ρ∙diag1 ρ ρ 1⊗RMIMO =11+ρ∙RMIMOρRMIMOρRMIMORMIMO, where h16×1=vec(H4x4) is the re-arranged channel values and - ⊗ denotes the Kronecker product, diag denotes the diagonal matrix. - RMIMO is the 4×4 correlation matrix for each link between TRPs to UE panels generated with parameters α (Tx correlation coefficient), β (Rx correlation coefficient), and γ (cross-polarization correlation coefficient) as stated in 38.101-4 B2.3.2 - ρ is the cross-talk power - Candidate cross-talk power values for initial simulation purpose: - -15, -12, -9, -6 dB - Other options are not precluded. - Candidate cross-polarization correlation coefficient values for initial simulation purpose: - γ = 0.125 - Other options are not precluded Issue 1-1-3: Receiver assumption. Agreement: - Keep both options (independent and joint processing) open at this point of evaluation. - Companies are encouraged to bring results evaluating both independent and joint processing in August meeting. Issue 1-1-4: Number of receive antennas per Rx chain. Agreement: - 2 Rx per chain Issue 1-1-5: Duplex mode for demodulation requirements. Agreement: - TDD mode only - Sub-topic 2-1: Simulation assumptions for PDSCH demodulation requirements Issue 2-1-1: Channel model Agreement - At least consider TDLA30-75 for 100 MHz/120 kHz - FFS TDLA30-300 for 100 MHz/120 kHz. Issue 2-1-2: PTRS Port for sDCI schemes <Way forward > - Proposals - Option 1: One PTRS port per TRP for sDCI schemes - Option 2: One PTRS port for sDCI SDM based scheme. - Recommended WF: Encourage comments if any Issue 2-1-3: PTRS Port for mDCI schemes Agreement: - One PTRS port per TRP for mDCI schemes Issue 2-1-4: Time offset between TRPs Agreement: - Use the time offset between the two TRPs transmission points as {0.25us, -0.0625us}. - Further discuss which time offset to be used with certain configurations. Issue 2-1-5: Frequency offset between TRPs Agreement: - Assume 600Hz frequency offset between TRPs Issue 2-1-6: PDSCH rate matching in mTRP transmission <Way forward > - Proposals - Option 1: PT-RS allocation does not overlap with PDSCH allocation per TRP. - Option 2: PT-RS allocation does not overlap with any PDSCH allocation. - Recommended WF: Encourage comments if any Issue 2-1-7: Sending an LS to RAN1 for specification clarification on PTRS rate matching behaviour for mTRP transmission (if ambiguous) <Way forward > - Proposals - Option 1: Yes - Recommended WF: Encourage comments if any Issue 2-1-8: Test cases and simulation parameters for sDCI SDM <Way forward > - Proposals - Option 1: Consider PDSCH test cases and simulation parameter for sDCI based SDM scheme as presented in Tables 1 and 2. Tables can be found in R4-2308945. - Recommended WF: Encourage comments if any Issue 2-1-9: Test cases and simulation parameters for mDCI fully overlapping. <Way forward > - Proposals - Option 1: Consider PDSCH test cases and simulation parameter for mDCI based SDM scheme as presented in Tables 3 and 4. Tables can be found in R4-2308945. - Recommended WF: Encourage comments if any Issue 2-1-10: Test cases and simulation parameters for mDCI non-overlapping <Way forward > - Proposals - Option 1: Consider PDSCH test cases and simulation parameter for mDCI based non-overlapping scheme as presented in Tables 5 and 6. Tables can be found in R4-2308945. - Recommended WF: Encourage comments if any

- Discussion on PDSCH demodulation test assumptions

Channel model

For the channel model of PDSCH demodulation tests, both TDLA30-75 and TDLA30-300 for 100 MHz/120 kHz are proposed by companies in last RAN4 meeting. But considering current case 2-2 and 2-3 in 38.101-4 section 7.2.2.2.1 rank 2 has no obvious SNRBB difference, from our point of view, one channel model as TDLA30-75 is enough for Multi-Rx PDSCH demodulation requirements.

Proposal 1: For Multi-Rx PDSCH demodulation requirements, only use TDLA30-75 for 100 MHz/120 kHz.

PTRS Port for sDCI schemes

The main purpose of PTRS is tracking the variety of phase noise, while the phase noise comes from the random change in the input signal phase caused by various noises (such as random white noise and flicker noise) in RF device. The higher the frequency, the higher the phase noise, thus PTRS is mainly used in high frequency band. Due to the randomness of phase noise in time domain, different Rx chain has different phase noise and need calibration by using PTRS. Thus, we prefer to configure PTRS port per TRP for both sDCI scheme and mDCI scheme, although the configuration of PTRS ports will take some REs from PDSCH and need the optional capability “supportTwoPortDL-PTRS-r16”.

Proposal 2: use option 1 (One PTRS port per TRP for sDCI schemes) for PTRS port configuration.

Time and frequency offset between TRPs

In last RAN4#107 meeting, the time offset between two TRP transmission points as {0.25us, -0.0625us} are agreed, and frequency offset 600Hz is agreed as starting point, but which one should be specified to which case still need discussion. Considering proper value should correspond with acceptable performance loss with proper UE processing in receiver side and enough performance gap to differentiate UE behavior, we can take the experience of Rel-16 FR1 TDD multi-TRP test cases as starting point. Current FR1TDD multi-TRP test has four scenarios as below

Table 2-1 Rel-16 FR1 TDD multi-TRP test cases

| scheme | scenarios | (time offset, frequency offset) |
| --- | --- | --- |
| sDCI | SDM | (-0.25us, 300Hz) and (1us, 0Hz) |
|  | FDM | (-0.25us, 300Hz) |
|  | TDM | (1us, 300Hz) |
| mDCI | Non-overlapping | (-0.25us, 300Hz) |

We propose to set four test cases with time and frequency offset as

sDCI SDM (-0.0625us, 600Hz) and (0.25us, 0Hz)

mDCI non-overlapping (-0.0625us, 600Hz)

mDCI full-overlapping  (0.25us, 600Hz)

Proposal 3: use four test cases with time and frequency offset as starting point

sDCI SDM (-0.0625us, 600Hz) and (0.25us, 0Hz)

mDCI non-overlapping (-0.0625us, 600Hz)

mDCI full-overlapping  (0.25us, 600Hz)

PDSCH rate matching in mTRP transmission

PTRS allocation should be scheduled per TRP. It is not realistic for current TRP to avoid PTRS REs of other TRP considering the overhead and resource schedule complexity, especially for mDCI case.

Proposal 4: PT-RS allocation does not overlap with PDSCH allocation per TRP.

Sending an LS to RAN1 for specification clarification on PTRS rate matching behaviour for mTRP transmission (if ambiguous)

We don’t think it’s necessary to send LS for specification clarification on PTRS rate matching behaviour for mTRP transmission.

Proposal 5: No need to send LS to RAN1 for specification clarification on PTRS rate matching behaviour for mTRP transmission.

Test cases and simulation parameters for sDCI SDM

Considering the agreements in RAN4#107

- At least consider TDLA30-75 for 100 MHz/120 kHz

- the time offset between two TRP transmission points as{0.25us, -0.0625us}

- Assume 600Hz frequency offset between TRPs

And considering the agreements in RAN4#106bis-e

- Consider the following for initial evaluation

- Single-DCI based SDM scheme using 1+1- and 2+2-layer combination

- Multi-DCI based fully-overlapping resource allocation using 2+2-layer combination

- Multi-DCI with non-overlapping resource allocation using 2+2-layer combination.

- Consider the following MCS for initial evaluation

- Use MCS 13 or lower for defining rank 2+2, higher inter-TRP interference cases with SDM/fully-overlapping scheduling.

- Use MCS 17 for rank 1+1 cases or lower inter-TRP interference cases with SDM or non-overlapping case.

- Define the FR2 multi-Rx UE demodulation requirements for PDSCH using FR2.120-1 TDD UL-DL pattern, given by DDDSU and S: 10D+2G+2U.

Thus we propose to use test parameters in table 2-2 and test cases in table 2-3 for sDCI SDM simulation.

Proposal 6: use test parameters in Table 2-2 and test cases in Table 2-3 for sDCI SDM simulation assumption.

Table 2-2 test parameters for sDCI SDM

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 |  |
|  |  | CORESETPoolIndex |  | 0 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 1 |  |
|  | Length (L) |  |  | Specific to each Reference channel as defined in A.3.2.2 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | 1000 for test 1-1 1000-1001 for test 1-2 | 1002 for test 1-1 1002-1003 for test 1-2 |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Full-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  | us | -0.0625 for test 1-1 0.25 for test 1-2 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  | Hz | 600 for test 1-1 0 for test 1-2 |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) Note 2: PDSCH transmission is done from both TRxPs (PDSCH Layers 0 and 1 are transmitted from TRxP #1 and PDSCH layers 2 and 3 are transmitted from TRxP #2) |  |  |  |  |  |

Table 2-3 Minimum performance for sDCI SDM

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition (Note 1) | Correlation matrix and antenna configuration (Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) (Note 3) |
| 1-1 | R.PDSCH.5-6.1 TDD | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | TDLA30-75 | 2x2, ULA Low | 70 | TBD |
| 1-2 | R.PDSCH.5-2.2 TDD | 100 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-75 | 2x2, ULA Low | 70 | TBD |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 with scaling factor as 1/sqrt(2) for transmitted signal from each TRxP |  |  |  |  |  |  |  |  |

Test cases and simulation parameters for mDCI full-overlapping

Proposal 7: use test parameters in Table 2-4 and test cases in Table 2-5 for mDCI full-overlapping simulation assumption.

Table 2-4 test parameters for mDCI full-overlapping

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 1 |  |
|  | Length (L) |  |  | Specific to each Reference channel as defined in A.3.2.2 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | {1000,1001} | {1002,1003} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Full-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  | us | 0.25 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  | Hz | 600 |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

Table 2-5 Minimum performance for mDCI full-overlapping

| Test num. | Reference channel |  | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition (Note 1) | Correlation matrix and antenna configuration (Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) (Note 3) |
|  | TRxP #1 | TRxP #2 |  |  |  |  |  |  |  |
| 1-1 | R.PDSCH.5-2.2 TDD | R.PDSCH.5-2.2 TDD | 100 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-75 | 2x2, ULA Low | 70 | TBD |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 with scaling factor as 1/sqrt(2) for transmitted signal from each TRxP |  |  |  |  |  |  |  |  |  |

Test cases and simulation parameters for mDCI non-overlapping

Proposal 8: use test parameters in Table 2-6 and test cases in Table 2-7 for mDCI non-overlapping simulation assumption.

Table 2-6 test parameters for mDCI non-overlapping

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 1 |  |
|  | Length (L) |  |  | Specific to each Reference channel as defined in A.3.2.2 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | {1000,1001} | {1002,1003} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | non-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  | us | -0.0625 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  | Hz | 600 |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

Table 2-7 Minimum performance for mDCI non-overlapping

| Test num. | Reference channel |  | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition (Note 1) | Correlation matrix and antenna configuration (Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) (Note 3) |
|  | TRxP #1 | TRxP #2 |  |  |  |  |  |  |  |
| 1-1 | R.PDSCH.5-2.2X TDD | R.PDSCH.5-2.2X TDD | 100 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-75 | 2x2, ULA Low | 70 | TBD |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2. |  |  |  |  |  |  |  |  |  |

- Conclusion

In this contribution, we provide analysis and views on FR2 Multi-Rx PDSCH demodulation requirements for remaining open issues. The proposals could be summarized as:

Proposal 1: For Multi-Rx PDSCH demodulation requirements, only use TDLA30-75 for 100 MHz/120 kHz.

Proposal 2: use option 1 (One PTRS port per TRP for sDCI schemes) for PTRS port configuration.

Proposal 3: use four test cases with time and frequency offset as starting point

sDCI SDM (-0.0625us, 600Hz) and (0.25us, 0Hz)

mDCI non-overlapping (-0.0625us, 600Hz)

mDCI full-overlapping  (0.25us, 600Hz)

Proposal 4: PT-RS allocation does not overlap with PDSCH allocation per TRP.

Proposal 5: No need to send LS to RAN1 for specification clarification on PTRS rate matching behaviour for mTRP transmission.

Proposal 6: use test parameters in Table 2-2 and test cases in Table 2-3 for sDCI SDM simulation assumption.

Table 2-2 test parameters for sDCI SDM

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 |  |
|  |  | CORESETPoolIndex |  | 0 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 1 |  |
|  | Length (L) |  |  | Specific to each Reference channel as defined in A.3.2.2 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | 1000 for test 1-1 1000-1001 for test 1-2 | 1002 for test 1-1 1002-1003 for test 1-2 |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Full-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  | us | -0.0625 for test 1-1 0.25 for test 1-2 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  | Hz | 600 for test 1-1 0 for test 1-2 |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) Note 2: PDSCH transmission is done from both TRxPs (PDSCH Layers 0 and 1 are transmitted from TRxP #1 and PDSCH layers 2 and 3 are transmitted from TRxP #2) |  |  |  |  |  |

Table 2-3 Minimum performance for sDCI SDM

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition (Note 1) | Correlation matrix and antenna configuration (Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) (Note 3) |
| 1-1 | R.PDSCH.5-6.1 TDD | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | TDLA30-75 | 2x2, ULA Low | 70 | TBD |
| 1-2 | R.PDSCH.5-2.2 TDD | 100 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-75 | 2x2, ULA Low | 70 | TBD |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 with scaling factor as 1/sqrt(2) for transmitted signal from each TRxP |  |  |  |  |  |  |  |  |

Proposal 7: use test parameters in Table 2-4 and test cases in Table 2-5 for mDCI full-overlapping simulation assumption.

Table 2-4 test parameters for mDCI full-overlapping

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 1 |  |
|  | Length (L) |  |  | Specific to each Reference channel as defined in A.3.2.2 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | {1000,1001} | {1002,1003} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Full-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  | us | 0.25 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  | Hz | 600 |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

Table 2-5 Minimum performance for mDCI full-overlapping

| Test num. | Reference channel |  | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition (Note 1) | Correlation matrix and antenna configuration (Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) (Note 3) |
|  | TRxP #1 | TRxP #2 |  |  |  |  |  |  |  |
| 1-1 | R.PDSCH.5-2.2 TDD | R.PDSCH.5-2.2 TDD | 100 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-75 | 2x2, ULA Low | 70 | TBD |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 with scaling factor as 1/sqrt(2) for transmitted signal from each TRxP |  |  |  |  |  |  |  |  |  |

Proposal 8: use test parameters in Table 2-6 and test cases in Table 2-7 for mDCI non-overlapping simulation assumption.

Table 2-6 test parameters for mDCI non-overlapping

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 1 |  |
|  | Length (L) |  |  | Specific to each Reference channel as defined in A.3.2.2 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | {1000,1001} | {1002,1003} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | non-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  | us | -0.0625 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  | Hz | 600 |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

Table 2-7 Minimum performance for mDCI non-overlapping

| Test num. | Reference channel |  | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition (Note 1) | Correlation matrix and antenna configuration (Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) (Note 3) |
|  | TRxP #1 | TRxP #2 |  |  |  |  |  |  |  |
| 1-1 | R.PDSCH.5-2.2X TDD | R.PDSCH.5-2.2X TDD | 100 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-75 | 2x2, ULA Low | 70 | TBD |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2. |  |  |  |  |  |  |  |  |  |

- Reference

[1] R4-2309824, “WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception”, Qualcomm, 3GPP RAN4 #107 meeting

[2] R4-2308945, “PDSCH demodulation requirements for FR2 multi-Rx chain”, Ericsson, 3GPP RAN4 #107 meeting