TSG-RAN Working Group 4 (Radio) meeting #108	R4-2312376

Toulouse, EU, 21st to 25th August 2023

Source:	Ericsson

Title:	Additional simulation results related to SBFD adjacent channel coexistence evaluation

Agenda item:	8.19.2.1

Document for:	Discussion

# Introduction

At the last RAN4 meeting (RAN4#107 in Incheon) simulation results from several companies were collected in the moderator summary [1]. The majority of submitted results was produced for Scenario 1, 6 and Case 1, 3. In addition [2] we also presented results for Scenario 2 and Case 2, 4.

In this contribution we provide additional simulation results for Scenario 1, 2, 6 and 7.

# Discussion

In this contribution we follow the RAN4 coexistence evaluation methodology applied to the 4 coexistence cases identified for study during previous meetings. For each coexistence case we identify a “relative ACIR” value, which we sweep to achieve the RAN4 coexistence target of 5% degradation for mean user throughput and 5%-tile user throughput with respect to the baseline defined by the study in previous meetings. Specifically, for each identified coexistence case, the ACIR that impact the coexistence are derived from baseline assumptions for legacy TDD and SBFD BS and UE, as described in the following Table 2-1 and Table 2-2. The relative ACIR which is swept is highlighted, with the motivation explained in detail in the corresponding sections.

Table 2-1: FR1 Baseline ACIR derived from baseline assumptions for legacy TDD and SBFD BS and UE

| Coexistence Case | ACIR of interest | Baseline ACIR | ACLR | ACS |
| --- | --- | --- | --- | --- |
| 1 | ACIR_UE_UE | 28.23 dB | ACLR UE SBFD = 30 dB | ACS UE TDD = 33 dB |
|  | ACIR_BS_UE | 32.7 dB | ACLR BS SBFD = 45 dB | ACS UE TDD = 33 dB |
| 2 | ACIR_BS_BS | 43.8 dB | ACLR BS SBFD = 45 dB | ACS BS TDD = 50 dB |
|  | ACIR_UE_BS | 30 dB | ACLR UE SBFD = 30 dB | ACS BS TDD = 50 dB |
| 3 | ACIR_BS_BS | 43.8 dB | ACLR BS TDD = 45 dB | ACS BS SBFD = 50 dB |
|  | ACIR_BS_UE | 32.7 dB | ACLR BS TDD = 45 dB | ACS UE SBFD = 33 dB |
| 4 | ACIR_UE_UE | 28.23 dB | ACLR UE TDD = 30 dB | ACS UE SBFD = 33 dB |
|  | ACIR_UE_BS | 30 dB | ACLR UE TDD = 30 dB | ACS BS SBFD= 50 dB |

Table 2-2: FR2 Baseline ACIR derived from baseline assumptions for legacy TDD and SBFD BS and UE

| Coexistence Case | ACIR of interest | Baseline ACIR | ACLR | ACS |
| --- | --- | --- | --- | --- |
| 1 | ACIR_UE_UE | 20 dB | ACLR UE SBFD = 23 dB | ACS UE TDD = 23 dB |
|  | ACIR_BS_UE | 21.8 dB | ACLR BS SBFD = 28 dB | ACS UE TDD = 23 dB |
| 2 | ACIR_BS_BS | 22.5 dB | ACLR BS SBFD = 28 dB | ACS BS TDD = 24 dB |
|  | ACIR_UE_BS | 20.4 dB | ACLR UE SBFD = 23 dB | ACS BS TDD = 24 dB |
| 3 | ACIR_BS_BS | 22.5 dB | ACLR BS TDD = 28 dB | ACS BS SBFD = 24 dB |
|  | ACIR_BS_UE | 21.8 dB | ACLR BS TDD = 28 dB | ACS UE SBFD = 23 dB |
| 4 | ACIR_UE_UE | 20 dB | ACLR UE TDD = 23 dB | ACS UE SBFD = 23 dB |
|  | ACIR_UE_BS | 20.4 dB | ACLR UE TDD = 23 dB | ACS BS SBFD= 24 dB |

The simulation scope and corresponding priorities for deployment scenarios and coexistence evaluation cases are listed in Table 2-3 and Table 2-4.

Table 2-3: Network deployment scenarios

| Scenario | FR | Aggressor | Victim | Priority |
| --- | --- | --- | --- | --- |
| 1 | FR1 | Urban Macro | Urban Macro | High |
| 2 | FR1 | Urban Hotspot | Urban Hotspot | Low |
| 3 | FR1 | Indoor | Indoor | Low |
| 4 | FR1 | Urban Macro | Micro | Low |
| 5 | FR1 | Micro | Micro | Low |
| 6 | FR2-1 | Urban Macro | Urban Macro | High |
| 7 | FR2-1 | Urban Hotspot | Urban Hotspot | Low |
| 8 | FR2-1 | Urban Micro | Urban Micro | Low |
| 9 | FR2-1 | Indoor | Indoor | Low |

Table 2-4: Coexistence cases

| Case | Aggressor | Victim | Slot allocation Aggressor Victim | Priority |
| --- | --- | --- | --- | --- |
| 1 | SBFD | TDD DL |  | High |
| 2 | SBFD | TDD UL |  | Low |
| 3 | TDD DL | SBFD |  | High |
| 4 | TDD UL | SBFD |  | Low |

  - FR1

For FR1 deployment Scenario 1 and 2 will be considered for coexistence evaluation Cases 1, 2, 3 and 4. Differently from previous contribution, we will focus on updated assumptions considering the blocking model and an increased value for BS ACS (50 dB).

    - Case 1

Case 1 refers to the coexistence case where the aggressor is SBFD operator, and the victim is the DL (Down-Link) of legacy static TDD operator. Since we are interested in observing the impact of UE-to-UE CLI (Cross-Link Interference), performance of both Urban Macro and Urban Hotspot scenarios (deployment scenarios 1 and 2) are considered.

Notice that the aggressor SBFD transmits both UL (Up-Link) and DL, so the performance of legacy TDD DL is the result of the ACI impact from both SBFD UL and DL. The impact from DL SBFD, however, will not be noticeable because also the baseline of the comparison is affected by similar DL interference and the assumption taken on the PSD (Power Spectral Density) for DL SBFD is to be the same as TDD. This is why in the following we will focus on studying the impact on DL STDD from UL SBFD, by sweeping the ACIR_UE_UE value.

      - Scenario 1

Figure 2.1.1.1-1 shows the DL mean and 5%-tile user throughput of TDD network, in Urban Macro scenario, and as can be observed, there is not coexistence impact on the DL TDD performance. The reason is that in a wide area scenario, if users are dropped uniformly, it is highly unlikely that they happen to be at a distance where they are subject to UE-to-UE CLI. For this reason, for this coexistence case it is of interest to also study Urban Hotspot deployment scenario, with clustered UEs.

Figure 2.1.1.1-1: Case 1: DL mean and 5%-tile user throughput of TDD, FR1, scenario 1 (sweeping ACIR_UE_UE)

      - Scenario 2

Figure 2.1.1.2-1 shows the DL mean and 5%-tile user throughput of TDD network, in Urban Hotspot scenario, as a function of ACIR_UE_UE values. It can be observed that DL mean user throughput is not impacted by coexistence with SBFD operator, but 5%-tile user throughput is. In particular, considering the reference ACIR values based on current assumptions, coexistence of DL TDD with SBFD network causes 8.7% degradation in 5%-tile throughput performance, with respect to the baseline. In order to reduce the degradation to a level lower than 5%, it is required to increase the ACIR more than 6 dB, so up to 34 dB. Assuming that the ACS of the TDD UEs is fixed and equal to 33 dB, it is not possible to achieve the desired ACIR even assuming SBFD UE ACLR can be improved till 40 dB or beyond. Also, the ACS of the TDD UE should be improved.

Figure 2.1.1.2-1: Case 1: DL mean and 5%-tile user throughput of TDD, FR1, scenario 2 (sweeping ACIR_UE_UE)

Observation 1: Coexistence of an SBFD network with a DL legacy TDD network is possible in a scenario where users are uniformly distributed. In this case, the UE-to-UE CLI does not cause harmful impact against the DL of the legacy network. However, this scenario may hide coexistence issues, because when users are uniformly distributed over a wide area, the probability that two users active in UL and DL at the same time are dropped close enough to each other to generate UE-to-UE CLI is extremely low.

Observation 2: For coexistence Case 1, where DL TDD is the victim, it is of interest to study as well the Urban Hotspot scenario: where the users are clustered, and the distance among them is reduced, it is higher the probability that users can interfere among each other. In this case, we observe that the DL mean user throughput is not affected by the coexistence with SBFD, and the 5%-tile throughput is, with up to 8.7% degradation with respect to the baseline. This requires an ACIR increment up to at least 34 dB. To achieve 34 dB assumption on UE ACS for TDD users should be improved.

    - Case 2

Case 2 refers to the coexistence case where the aggressor is the SBFD DU operator, and the victim is the UL (Up-Link) of the legacy static TDD operator.

      - Scenario 1

Figure 2.1.2.1-1: Case 2: UL mean and 5%-tile user throughput of TDD, FR1, scenario 1 (sweeping ACIR_BS_BS)

Figure 2.1.2.1-1 shows the UL mean and 5%-tile user throughput of TDD. Considering the reference ACIR values based on the assumptions, it can be observed a degradation of 17.7% and 73.3% in terms of mean and 5%-tile user throughout, respectively. By sweeping the ACIR_BS_BS to high values, it is possible to reduce the impact of BS-to-BS interference. However, beyond a certain level, the 5%-tile throughput degradation cannot be further reduced. The reason is that in this case, the blocking of the UL TDD receiver, which happens with approximately 2% probability in our simulation due to the DL SBFD activity, and the increment of the resulting noise figure, introduce a degradation that cannot be eliminated, even if the ACIR is increased to very high and not realistic values.

      - Scenario 2

The behaviour in urban hotspot scenario is similar to the urban macro, as observed in Figure 2.1.2.2-1. This scenario is more challenging in terms of coverage, due to the fact that 80% of users are clustered indoor (section 2.2.3.9.1 in R4-2220247 [4]). About the mean user throughput, similarly to what observed in urban scenario, there is a reduction of UL mean user throughput performance of 18.2% when SBFD is the neighbour of a TDD network, compared to the case TDD coexists with TDD, and 67% for 5%-tile throughout. By increasing the ACIR we can get an improvement, but not enough to reduce the degradation to less than 5%. This is due to the additional source of performance degradation, besides the BS-to-BS CLI, introduced by the blocking of the receiver.

Figure 2.1.2.2-1: Case 2: UL mean and 5%-tile user throughput of TDD, FR1, scenario 2 (sweeping ACIR_BS_BS)

Observation 3: In FR1, the UL of a TDD network is highly impacted by the coexistence with an SBFD network. 17-18% degradation is observed (in different scenarios) in terms of mean user throughput, and 67-73% in terms of 5%-tile user throughput.

Observation 4: In FR1, when blocking is modelled at SBFD and TDD BS, approximately a 2% of blocking probability is observed in our simulation, due to the CLI generated by the DL of SBFD neighbour operator. This probability of blocking, together with the increase in noise figure, harmfully impacts the UL performance of the TDD network in such a way that even increasing the ACIR to very high values, it is not possible to reduce the degradation to an acceptable level, below 5%, with respect to the baseline.

    - Case 3

Case 3 refers to the coexistence case where the aggressor is the legacy TDD operator, and the victim are the UL and DL of the SBFD operator.

      - Scenario 1

Figure 2.1.3.1-1 shows the UL mean and 5%-tile user throughput of SBFD, when the aggressor is the DL of TDD. Considering the reference ACIR assumptions, it can be observed a degradation with respect to the baseline of 15.6% and 63%, in terms of mean and 5%-tile user throughout, respectively. In this case, due to the blocking of the UL of SBFD network, which happens with approximately 2% probability in our simulation, and increased noise figure, due to the DL TDD activity, it is not possible to reduce the degradation beyond a certain level, even if the ACIR is increased to very high and not realistic values.

Figure 2.1.3.1-2 shows the DL mean and 5%-tile user throughput of SBFD, when the aggressor is the DL of TDD. It can be observed that the DL of SBFD can coexist with the DL of TDD with the performance degradation below 5%.

Figure 2.1.3.1-1: Case 3: UL mean and 5%-tile user throughput of SBFD, FR1, scenario 1 (sweeping ACIR_BS_BS)

Figure 2.1.3.1-2: Case 3: DL mean and 5%-tile user throughput of SBFD, FR1, scenario 1 sweeping ACIR_BS_UE)

      - Scenario 2

The behaviour and conclusion in Urban Hotspot scenario are similar to the Urban Macro. Similarly, as in case 2, this scenario has more challenges in terms of coverage, due to the fact that 80% of users are clustered indoor (section 2.2.3.9.1 in R4-2220247 [4]). Similar to what observed in Urban Macro scenario, there is a reduction of approximately 14.5% and 60% of UL mean user throughput and UL 5%-tile user throughput, when STDD is the neighbour of a SBFD network, compared to the case SBFD network is isolated, as can be observed in Figure 2.1.3.2-1. By increasing the ACIR we can get an improvement, by removing the BS-to-BS interference, but not enough to reduce the degradation to less than 5%. 5%-tile cannot be recovered even pushing ACIR 36 dB higher; for mean user throughput, we should increase it by 16 dB, so up to 60 dB, which is not possible assuming ACLR of 45 dB at legacy TDD BSs.

Figure 2.1.3.2-2 shows the DL mean and 5%-tile user throughput of SBFD, when the aggressor is the DL of TDD. It can be observed that the DL of SBFD can coexist with the DL of TDD with the performance degradation below 5%.

Figure 2.1.3.2-1: Case 3: UL mean and 5%-tile user throughput of SBFD, FR1, scenario 2 (sweeping ACIR_BS_BS)

Figure 2.1.3.2-2: Case 3: DL mean and 5%-tile user throughput of SBFD, FR1, scenario 2 (sweeping ACIR_BS_UE)

Observation 5: In FR1, the UL of a SBFD network is highly impacted by the coexistence with a legacy TDD network. Approximately 15% degradation is observed in terms of mean user throughput, and 60% in terms of 5%-tile user throughput.

Observation 6 In FR1, when blocking is modelled at SBFD and TDD BS, approximately a 2% of blocking probability is observed in our simulation, due to the CLI generated by the DL of legacy TDD neighbour operator. This probability of blocking, together with the increment in resulting noise figure, as defined by the blocking model, harmfully impacts the UL performance of SBFD network in such a way that even increasing the BS-BS ACIR, it is not possible to reduce the degradation to an acceptable level, below 5%, with respect to the baseline.

Observation 7 It is important to understand the difference between the interference captured by the BS receiver chain, which degrades the signal SINR and consequently the throughput performance, and the total received signal in the operation band, which can potentially saturate the BS receiver. The increment of ACIR_BS_BS can reduce the impact of BS-to-BS interference, but cannot reduce the risk of blocking of the receiver, and this is what we observe in the UL results reported for cases 2 and 3, when the UL of TDD and SBFD are victim of the DL transmissions from the other operator.

    - Case 4

Case 4 refers to the coexistence case where the aggressor is the UL of legacy TDD operator, and the victim are the DL and UL of the SBFD operator.

      - Scenario 1

Figure 2.1.4.1-1 and Figure 2.1.4.1-2 show the DL and UL mean and 5%-tile user throughput of SBFD network, respectively, in Urban Macro scenario. As it can be observed, there is not coexistence issues on the DL and UL of SBFD performance in Urban Macro scenario. The reason is that in a wide area scenario, if users are dropped uniformly, it is highly unlikely that they happen to be at a distance where they are subject to UE-to-UE CLI.

Figure 2.1.4.1-1: Case 4: DL mean and 5%-tile user throughput of SBFD network, FR1, scenario 1 (sweeping ACIR_UE_UE)

Figure 2.1.4.1-2: Case 4: UL mean and 5%-tile user throughput of SBFD network, FR1, scenario 1 (sweeping ACIR_UE_BS)

      - Scenario 2

Figure 2.1.4.2-1 and Figure 2.1.4.2-2 show the DL and UL mean and 5%-tile user throughput, respectively, of SBFD network, in Urban Hotspot scenario. It can be observed that UL mean and 5%-tile throughput, and DL mean user throughput are not impacted by coexistence with the legacy TDD operator. DL 5%-tile user throughput instead is reduced by 5.5% compared to the baseline and so requires a slight increment of 2 dB of ACIR.

Figure 2.1.4.2-1: Case 4: DL mean and 5%-tile user throughput of SBFD network, FR1, scenario 2 (sweeping ACIR_UE_UE)

Figure 2.1.4.2-2: Case 4: UL mean and 5%-tile user throughput of SBFD network, FR1, scenario 2 (sweeping ACIR_UE_BS)

Observation 8: In FR1, and Urban Macro scenario, the TDD legacy UL does not generate disruptive interference against the DL or UL of SBFD. On the other hand, in urban hotspot scenario the DL 5%-tile can be slightly degraded due to the presence of UE-to-UE CLI generated by TDD legacy UL.

  - FR2-1

For FR2 deployment Scenario 6 and 7 will be considered for coexistence evaluation Cases 1, 2, 3 and 4. Differently from previous contribution, we will focus on the updated assumptions considering the blocking model and only Grid-Shift (GS) 100% and Line of Sight (LoS) UMa, as agreed in previous meetings for priority assumptions.

Similarly, to FR1, the discussion is organized based on coexistence cases. We will focus, as for FR1, primarily on all the priority assumptions that have been agreed. Then, for certain cases that we believe are sensitive to critical parameters, we also evaluate the coexistence performance for the optional assumptions. Specifically, we will focus on two optional parameters: 1) the TRP: 30 dBm (priority), 40 dBm (optional), 2) Grid-Shift (GS): 100% (priority), 10% (optional).

For case 1 and 4 where the DL of TDD and SBFD, respectively, are the victim, we will focus only on priority assumptions. The reason is that the DL is not expected to be sensitive to the GS between operators, and when increasing the BS Transmission power, the wanted DL will be stronger and so more resilient to UE-to-UE CLI. This is why we expect that the coexistence is more critical with the priority assumptions, and we will focus only on them.

Differently, cases 2 and 4 focus on the UL of TDD and SBFD, respectively, as a victim. The UL is extremely sensitive to the GS between operators and the BS transmission power, so for these cases we will also evaluate the optional assumptions.

    - Case 1

Case 1 refers to the coexistence case where the aggressor is SBFD operator, and the victim is the DL of legacy static TDD operator. Since we are interested in observing the impact of UE-to-UE CLI, performance of both Urban Macro and Urban Hotspot scenarios (deployment scenarios 6 and 7) are considered.

Notice that the aggressor SBFD transmits both UL and DL, so the performance of legacy TDD DL are the result of the ACI impact from both SBFD UL and DL. The impact from DL SBFD, however, will not be noticeable because also the baseline of the comparison (TDD coexistence with TDD) is affected by similar DL interference, and the assumption taken on the PSD (Power Spectral Density) from DL SBFD is to be the same as STDD. This is why in the following we will focus on studying the impact on DL STDD from UL SBFD, by sweeping the ACIR_UE_UE value.

As already mentioned, in this coexistence case we will focus only on the priority assumptions.

      - Scenario 6

Figure 2.2.1.1-1 shows the DL mean and 5%-tile user throughput of TDD network, in FR2 urban macro scenario, and as can be observed, there is not coexistence issue on the DL TDD performance. Similarly, to Case 1 Scenario 1 for FR1, the reason is that in a wide area scenario, if users are dropped uniformly, it is highly unlikely that they happen to be at a distance where they are subject to UE-to-UE CLI. In addition, FR2 is characterized by higher directionality of transmissions, which further reduces risks of UE-to-UE CLI.

Figure 2.2.1.1-1: Case 1: DL mean and 5%-tile user throughput of TDD, FR2, scenario 6 (sweeping ACIR_UE_UE)

      - Scenario 7

Figure 2.2.1.2-1 shows the DL mean and 5%-tile user throughput of TDD network, in Scenario 7 (Urban Hotspot). It can be observed that DL mean user throughput is not impacted by UE-to-UE CLI more than 5%, but 5%-tile user throughput slightly is, specifically, we can observe a degradation of 5.5 dB compared to the baseline, when ACI is introduced. It is sufficient to increase the ACIR_UE_UE by 2 dB in order to reduce the degradation below 5%.

Figure 2.2.1.2-1: Case 1: DL mean and 5%-tile user throughput of TDD, FR2, scenario 7 (sweeping ACIR_UE_UE)

Observation 9: In FR2 scenarios, when users are not clustered, the operation of SBFD does not impact the DL of the legacy TDD operator. When users are clustered, the chances for UE-to-UE interference increase and while the DL mean user throughput is not impacted by the operation of SBFD, the 5%-tile slightly is, which would require an increment of 2 dB in ACIR_UE_UE, and so the ACLR of SBFD users.

    - Case 2

Case 2 refers to the coexistence case where the aggressor is the SBFD DU operator, and the victim is the UL of the legacy TDD operator. In the following results we study the impact of the BS-to-BS interference, by sweeping the ACIR_BS_BS. To further investigate this coexistence case, we will also include results with some optional parameters, specifically BS transmission power of 40 dBm and grid-shift 10%, since the UL of TDD is victim of BS-to-BS interference and this is strongly sensitive to the considered transmission power and GS between operators, as listed in Table 2.2.2-1.

Table 2.2.2-1: Simulation assumptions

| Priority assumptions | TRP = 30 dBm | GS=100% |
| --- | --- | --- |
| Optional assumptions | TRP = 40 dBm | GS = 10% |

      - Scenario 6

Figure 2.2.2.1-1: Case 2 (priority assumptions): UL mean and 5%-tile user throughput of STDD, FR2, scenario 6 (sweeping ACIR_BS_BS)

Figure 2.2.2.1-1 shows the UL mean and 5%-tile user throughput of TDD, when SBFD is the aggressor and priority assumptions are considered. The dominant source of interference is the internal co-link, so that the impact of BS-to-BS ACI is not as high as in FR1 scenario. Considering the reference ACIR values based on assumptions, it can be observed a degradation of 9.5% in terms of 5%-tile user throughout. To reduce the degradation to below 5%, it is necessary to increase the ACIR_BS_BS by 6 dB, so up to 28.5 dB. This would require increasing the ACS of TDD BSs beyond the value that is currently assumed (24 dB).

Figure 2.2.2.1-2 shows the shows the UL mean and 5%-tile user throughput of TDD, when SBFD is the aggressor and optional assumptions are considered. In this case, the higher power of transmission of the BS causes a percentage of blocking and an increment of the noise figure if the receiver is not completely blocked. This makes that even increasing the ACIR_BS_BS and so reducing the BS-to-BS interference, it is not possible to reduce the degradation compared to the baseline below 5%.

Figure 2.2.2.1-2: Case 2 (optional assumptions): UL mean and 5%-tile user throughput of STDD, FR2, scenario 6 (sweeping ACIR_BS_BS)

      - Scenario 7

Figure 2.2.2.2-1 shows the UL mean and 5%-tile user throughput of TDD when it is victim of UL and DL of SBFD, and the scenario is clustered. It can be observed that the UL mean user throughput is not impacted by ACI more than 5%, while the 5%-tile throughput is by 12%. When increasing the ACIR_BS_BS to very high values, the degradation still cannot go below 5%. The reason is that other sources of interference impact the UL of TDD, besides the DL of SBFD. Specifically, the UL of SBFD also generates interference against UL TDD, and this UL to UL interference is particularly exposed in the clustered scenario due to beamforming behaviours. As mentioned, the aggressor SBFD transmits both UL and DL, so the performance of legacy TDD UL are the result of the ACI impact from both UL and DL SBFD. The baseline of this case (TDD coexistence with TDD) already considers co-link ACI UL to UL. However, the bandwidth in UL TDD is wider than that of UL SBFD, and the considered ACIR is determined assuming that victim and aggressor have the same bandwidth, and the emissions out of band are considered to be flat. As a result, the UL ACI from SBFD can be higher than that generated from another legacy TDD system operating over 100 MHz, and this UL to UL interference could only be removed by increasing ACIR_UE_BS.

Figure 2.2.2.2-2 shows the UL mean and 5%-tile user throughput of TDD when it is victim of UL and DL of SBFD, the scenario is clustered, and 40 dBm BS transmission power, and GS=10% are considered. In this case, the smaller GS and higher power cause that the receiver of TDD BS is blocked during approximately 4% of time in our simulation. This, together with the increment of noise figure modelled in the blocking model, makes that the degradation cannot be reduced to less than 5% when ACIR_BS_BS is increased.

Figure 2.2.2.2-1: Case 2 (priority assumptions): UL mean and 5%-tile user throughput of STDD, FR2, scenario 7 (sweeping ACIR_BS_BS)

Figure 2.2.2.2-2: Case 2 (optional assumptions): UL mean and 5%-tile user throughput of STDD, FR2, scenario 7 (sweeping ACIR_BS_BS)

Observation 10: In FR2, when SBFD DU is the aggressor, and TDD UL the victim, depending on the deployment of the users, not only DL, but also UL can generate harmful interference against the victim UL. The two UL have different bandwidths (20 MHz the aggressor and 100 MHz the victim), so it is important to define the scaling of ACIR_UE_BS and for all other similar cases.

Observation 11: The coexistence between SBFD DU and TDD UL is extremely sensitive to parameters like the GS and the BS transmission power. If those values are set to 10% and 40 dBm, respectively, there is a 4% percentage of complete blocking of the receiver in our simulations, which, together with the modelled increment of noise figure, makes that even increasing the ACIR_BS_BS, it is not possible to reduce the degradation due to ACI below 5%.

    - Case 3

Case 3 refers to the coexistence case where the aggressor is the legacy TDD operator, and the victims are the UL and DL of SBFD operator. Like in case 2, also in this case, when priority assumptions are considered and the transmission power is 30 dBm, the interference at the victim is dominated by internal interference. Specifically, a great source of internal interference, besides the co-link one, which was present also in case 2, is the inter-sector and self-interference. In general, the SBFD internal interference dominates and the impact from ACI results is lower than for FR1 and also than FR2 case 2. To further investigate this coexistence case and find situations where ACI plays a more interesting role, we will also include results with some optional parameters, specifically BS transmission power of 40 dBm and grid-shift 10%, as listed in Table 2.2.3-1.

Table 2.2.3-1: Simulation assumptions

| Priority assumptions | TRP = 30 dBm | GS=100% |
| --- | --- | --- |
| Optional assumptions | TRP = 40 dBm | GS = 10% |

      - Scenario 6

Figure 2.2.3.1-1 and Figure 2.2.3.1-2 refer to scenario 6 considering the priority assumptions, which specifically accounts for TRP equal to 30 dBm and for GS equal to 100%. Figure 2.2.3.1-3 and Figure 2.2.3.1-4 refer to the same scenario, but consider the optional simulation parameters of TRP equal to 40 dBm and GS equal to 10%

Figure 2.2.3.1-1 shows the UL mean and 5%-tile user throughput of SBFD, as a function of ACIR_BS_BS. Considering the reference ACIR values based on the assumptions, it can be observed a degradation of 9% in terms of 5%-tile user throughout. To reduce the degradation below 5%, it is necessary to increase the BS-BS ACIR by 6 dB, so up to 28 dB. So, this requires an increment of ACS at SBFD BS up to at least 28 dB

Figure 2.2.3.1-2 shows the DL mean and 5%-tile user throughput of SBFD, as a function of ACIR_BS_UE. Considering the reference ACIR value, based on assumptions. Important degradation is not observed.

When the optional parameters are considered and transmission power of BSs is increased to 40 dBm and the GS is changed to 10%, the coexistence becomes more challenging. In particular, degradation is observed in Figure 2.2.3.1-3 both in terms of mean (84%) and 5%tile (13.9%) user throughput. When increasing the ACIR_BS_BS, the degradation is reduced, as the BS-to-BS interference is removed, but the degradation cannot in any case go below 5%. This is due to the blocking that is observed with probability of 4 % in our simulations, and the additional increment of noise figure that is also modelled when the BS receiver is not completely blocked. On DL SBFD degradation is not observed in Figure 2.2.3.1-4.

Figure 2.2.3.1-1: Case 3 (priority assumptions): UL mean and 5%-tile user throughput of SBFD, FR2, scenario 6 (sweeping ACIR_BS_BS)

Figure 2.2.3.1-2: Case 3 (priority assumptions): DL mean and 5%-tile user throughput of SBFD, FR2, scenario 6 (sweeping ACIR_BS_UE)

Figure 2.2.3.1-3: Case 3 (optional assumptions): UL mean and 5%-tile user throughput of SBFD, FR2, scenario 6 (sweeping ACIR_BS_BS)

Figure 2.2.3.1-4: Case 3 (optional assumptions): DL mean and 5%-tile user throughput of SBFD, FR2, scenario 6 (sweeping ACIR_BS_UE)

      - Scenario 7

Similarly, to the previous section, Figure 2.2.3.2-1 and Figure 2.2.3.2-2 refer to the priority assumptions, while Figure 2.2.3.2-3 and Figure 2.2.3.2-4 to the optional assumptions in terms of TRP and GS.

Figure 2.2.3.2-1 shows the UL mean and 5%-tile user throughput of SBFD, as a function of ACIR_BS_BS. Considering the reference ACIR values based on assumptions, it can be observed a degradation of 6% in terms of 5%-tile user throughput. To reduce the degradation below 5%, it is necessary to increase the ACIR by 4 dB, so up to 26 dB. This requires an improvement at ACS SBFD BS.

Figure 2.2.3.2-2 shows the DL mean and 5%-tile user throughout of SBFD, as a function of ACIR_BS_US, it can be observed some degradation due to DL TDD to DL SBFD ACI, which is caused by the fact that UEs are clustered and so more sensitive to DL interference from other operator. Also, the DL wanted signal is transmitted with 30 dBm and this makes the wanted link more sensitive to the other operator DL interference. Similar degradation is however also observed for the reference TDD coexistence TDD case, as detailed later in the summary tables, and so this degradation is not of interest to the conclusion of the study, but just a result of a clustered scenario where the wanted signal is not so strong. This degradation is not observed when the BS TRP is increased to 40 dBm in Figure 2.2.3.2-4, where the BS transmission power is 40 dBm.

Figure 2.2.3.2-3 and Figure 2.2.3.2-4 show the UL and DL mean and 5%-tile user throughput of SBFD, when optional parameters for BS transmission power (40 dBm) and grid-shift are considered.

When the optional parameters are considered and transmission power of BSs is increased to 40 dBm and the GS is changed to 10%, the coexistence becomes more challenging. In particular, degradation is observed in Figure 2.2.3.2-3 both in terms of mean (80%) and 5%tile (13.8%) user throughput. When increasing the ACIR_BS_BS, the degradation is reduced, as the BS-to-BS interference is removed, but the degradation cannot in any case go below 5%. This is due by the blocking that is observed with probability of approximately 4 % in our simulations, and the additional increment of noise figure that is also modelled when the BS receiver is not completely blocked. On DL SBFD degradation is not observed in Figure 2.2.3.2-4.

Figure 2.2.3.2-1: Case 3 (priority assumptions): UL mean and 5%-tile user throughput of STDD, FR2, scenario 7 (sweeping ACIR_BS_BS)

Figure 2.2.3.2-2: Case 3 (priority assumptions): DL mean and 5%-tile user throughput of STDD, FR2, scenario 7 (sweeping ACIR_BS_UE)

Figure 2.2.3.2-3: Case 3 (optional assumptions): UL mean and 5%-tile user throughput of STDD, FR2, scenario 7 (sweeping ACIR_BS_BS)

Figure 2.2.3.2-4: Case 3 (optional assumptions): UL mean and 5%-tile user throughput of STDD, FR2, scenario 7 (sweeping ACIR_BS_UE)

Observation 12: In FR2, when SBFD is the victim of DL TDD, the transmission power of the BS is not high (30 dBm), and the grid-shift between operators is maximum (100%), the ACI is not so relevant for the UL performance, and the internal interference of SBFD dominates the interference pattern, so that the coexistence is possible while assuming certain improvements, compared to the current assumptions.

Observation 13: In FR2, when SBFD is the victim of DL TDD, if the transmission power of the BS is increased (40 dBm) and the grid-shift between operators is reduced (10%), the ACI becomes more relevant, the probability of blocking of the receiver BS increases to 4% in our simulation, and together with the increment of noise figure to model the blocking, makes that even when increasing the ACIR_BS_BS the degradation cannot be eliminated.

    - Case 4

Case 4 refers to the coexistence case where the aggressor is the UL of legacy TDD operator, and the victim is the DL and UL of the SBFD operator.

      - Scenario 6

Figure 2.2.4.1-1 shows the DL mean and 5%-tile user throughput of TDD network, in urban macro scenario, and as can be observed, there is not coexistence issue on the DL TDD performance in urban scenario. The reason is that in a wide area scenario, if users are dropped uniformly, it is highly unlikely that they happen to be at a distance where they are subject to UE-to-UE CLI, in addition, in an FR2 scenario directionality reduces interference among nodes.

Figure 2.2.4.1-2 shows UL mean and 5%-tile user throughput of TDD network, in urban scenario, and as expected it is not impacted by the coexistence with UL TDD.

Figure 2.2.4.1-1: Case 4: DL mean and 5%-tile user throughput of SBFD network, FR2, scenario 6 (sweeping ACIR UE UE)

Figure 2.2.4.1-2: Case 4: UL mean and 5%-tile user throughput of SBFD network, FR2, scenario 6 (sweeping ACIR UE BS)

      - Scenario 7

Figure 2.4.4.2-1 shows the DL mean and 5%-tile user throughput of SBFD network, in Urban Hotspot scenario. It can be observed that both DL mean and 5%-tile user throughput are not impacted by coexistence with the legacy TDD operator.

Figure 2.2.4.2-2 shows UL mean and 5%-tile user throughput of TDD network, in urban scenario, and as expected it is not impacted by the coexistence with UL TDD.

Figure 2.2.4.2-1: DL mean and 5%-tile user throughput of SBFD network, FR2, scenario 7 (sweeping ACIR UE UE)

Figure 2.2.4.2-2: DL mean and 5%-tile user throughput of SBFD network, FR2, scenario 7 (sweeping ACIR UE BS)

  - Simulation result overview

The simulation results are formatted in agreed templates from last RAN4 meeting captured in [3]. Where not explicitly indicated, the first priority assumptions are followed reported in the Annex and in [5]. For FR2, in order to study also the impact of some key optional assumptions, we also add for high priority case 3 the study of the optional parameters BS TRP = 40 dBm and Grid-Shift = 10% (while first priority assumptions consider TRP = 30 dBm and Grid-Shift=100%). For those results the optional parameter column is also used.

    - Scenario 1

The results for scenario 1 are listed in Table 2.3.1-1, Table 2.3.1-2, Table 2.3.1-3, Table 2.3.1-4, Table 2.3.1-5 and Table 2.3.1-6.

Table 2.3.1-1: Scenario 1, Case 1

| Company | Observation Point |  | Performance degradation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |
|  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 20 | 24 |
| Ericsson | 5% | SINR degradation (dB) | 0.02 | 0.02 | 0.02 |  |  |  |  |  |
|  |  | Throughput degradation (%) | 0.23 | 0.23 | 0.23 |  |  |  |  |  |
|  |  | 50% SINR degradation (dB) | 0.01 | 0.01 | 0.01 |  |  |  |  |  |
|  |  | Mean Throughput degradation (%) | 0.04 | 0.04 | 0.03 |  |  |  |  |  |

Table 2.3.1-2: Scenario 1, Case 2

| Company | Observation Point |  | Performance degradation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |
|  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 24 | 26 |
| Ericsson | 5% | SINR degradation (dB) | 8.4 | 6.7 | 5.4 | 4.3 | 2.7 | 2.4 | 1.8 | 1.6 |
|  |  | Throughput degradation (%) | 72.9 | 63.2 | 53.7 | 44.8 | 30.12 | 27.3 | 20.9 | 18.6 |
|  |  | 50% SINR degradation (dB) | 2.2 | 1.8 | 1.5 | 1.3 | 0.99 | 0.9 | 0.76 | 0.71 |
|  |  | Mean Throughput degradation (%) | 17.6 | 14.9 | 11.7 | 12.94 | 8 | 7.4 | 6.3 | 5.8 |

Table 2.3.1-3: Scenario 1, Case 3, UL

| Company | Victim | Observation Point |  | Performance degradation |  |  |  |  |  |  |  | Performance degradation reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |  |
|  |  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 20 | 26 |  |
| Ericsson | SBFD UL | 5% | SINR degradation (dB) | 5.6 | 4.4 | 3.6 | 2.9 | 1.7 | 1.5 | 1.2 | 1.2 | 0.12 |
|  |  |  | Throughput degradation (%) | 63.2 | 54.2 | 46 | 38.9 | 24.8 | 22 | 18.5 | 17.7 | 1.53 |
|  |  |  | 50% SINR degradation (dB) | 2 | 1.7 | 1.5 | 1.3 | 1 | 0.9 | 0.78 | 0.75 | 0.07 |
|  |  |  | Mean Throughput degradation (%) | 15.6 | 13.1 | 11.3 | 9.8 | 6.97 | 6.4 | 5.4 | 5.1 | 0.63 |

Table 2.3.1-4: Scenario 1, Case 3, DL

| Company | Victim | Observation Point |  | Performance degradation |  |  |  |  |  |  |  | Performance degradation reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |  |
|  |  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 20 | 26 |  |
| Ericsson | SBFD DL | 5% | SINR degradation (dB) | 0.34 | 0.27 |  |  |  |  |  |  | 0.28 |
|  |  |  | Throughput degradation (%) | 3.8 | 3 |  |  |  |  |  |  | 3.3 |
|  |  |  | 50% SINR degradation (dB) | 0.25 | 0.18 |  |  |  |  |  |  | 0.24 |
|  |  |  | Mean Throughput degradation (%) | 0.9 | 0.7 |  |  |  |  |  |  | 0.88 |

Table 2.3.1-5: Scenario 1, Case 4, UL

| Company | Victim | Observation Point |  | Performance degradation |  |  |  |  |  |  |  | Performance degradation reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |  |
|  |  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 20 | 26 |  |
| Ericsson | SBFD UL | 5% | SINR degradation (dB) | 0.03 | 0.03 | 0.01 |  |  |  |  |  | 0.12 |
|  |  |  | Throughput degradation (%) | 0.49 | 0.27 | 0.18 |  |  |  |  |  | 1.53 |
|  |  |  | 50% SINR degradation (dB) | 0.06 | 0.05 | 0.04 |  |  |  |  |  | 0.07 |
|  |  |  | Mean Throughput degradation (%) | 0.38 | 0.26 | 0.17 |  |  |  |  |  | 0.63 |

Table 2.3.1-6: Scenario 1, Case 4, DL

| Company | Victim | Observation Point |  | Performance degradation |  |  |  |  |  |  |  | Performance degradation reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |  |
|  |  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 20 | 26 |  |
| Ericsson | SBFD DL | 5% | SINR degradation (dB) | 0.008 | 0.008 | 0.008 |  |  |  |  |  | 0.28 |
|  |  |  | Throughput degradation (%) | 0.009 | 0.009 | 0.009 |  |  |  |  |  | 3.3 |
|  |  |  | 50% SINR degradation (dB) | 0.003 | 0.003 | 0.001 |  |  |  |  |  | 0.23 |
|  |  |  | Mean Throughput degradation (%) | 0.01 | 0.009 | 0.04 |  |  |  |  |  | 0.88 |

    - Scenario 2

The results for scenario 2 are listed in Table 2.3.2-1, Table 2.3.2-2, Table 2.3.2-3, Table 2.3.2-4, Table 2.3.2-5 and Table 2.3.2-6.

Table 2.3.2-1: Scenario 2, Case 1

| Company | Observation Point |  | Performance degradation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |
|  |  |  | 0 | 2 | 4 | 6 | 8 | 10 | 20 | 24 |
| Ericsson | 5% | SINR degradation (dB) | 0.74 | 0.64 | 0.52 | 0.39 | 0.31 | 0.24 | 0.03 | 0.008 |
|  |  | Throughput degradation (%) | 8.7 | 7.6 | 6.1 | 4.6 | 3.7 | 2.9 | 0.43 | 0.1 |
|  |  | 50% SINR degradation (dB) | 0.19 | 0.17 | 0.14 | 0.13 | 0.1 | 0.09 | 0.03 | 0.01 |
|  |  | Mean Throughput degradation (%) | 1 | 0.8 | 0.74 | 0.62 | 0.5 | 0.4 | 0.1 | 0.05 |

Table 2.3.2-2: Scenario 2, Case 2

| Company | Observation Point |  | Performance degradation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |
|  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 20 | 26 |
| Ericsson | 5% | SINR degradation (dB) | 6.2 | 4.6 | 3.7 | 2.98 | 1.6 | 1.3 | 1 | 1 |
|  |  | Throughput degradation (%) | 67 | 55.1 | 46.4 | 38.8 | 22.7 | 18.4 | 15.7 | 15 |
|  |  | 50% SINR degradation (dB) | 2.76 | 2.32 | 2 | 1.74 | 1.28 | 1.2 | 1 | 0.97 |
|  |  | Mean Throughput degradation (%) | 18 | 15.2 | 13.1 | 11.5 | 8.1 | 7.5 | 6.3 | 5.9 |

Table 2.3.2-3: Scenario 2, Case 3, UL

| Company | Victim | Observation Point |  | Performance degradation |  |  |  |  |  |  |  | Performance degradation reference (Mbps) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |  |
|  |  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 20 | 26 |  |
| Ericsson | SBFD UL | 5% | SINR degradation (dB) | 4.6 | 3.7 | 3.1 | 2.1 | 1 | 1 | 1 | 1 | 0.08 |
|  |  |  | Throughput degradation (%) | 60.1 | 51.9 | 45.1 | 36.9 | 27.6 | 23.6 | 20.6 | 18.9 | 1.24 |
|  |  |  | 50% SINR degradation (dB) | 1.98 | 1.63 | 1.43 | 1.15 | 0.94 | 0.88 | 0.81 | 0.77 | 0.14 |
|  |  |  | Mean Throughput degradation (%) | 14.5 | 12.1 | 10.3 | 8.9 | 6.2 | 5.7 | 4.8 | 4.5 | 0.87 |

Table 2.3.2-4: Scenario 2, Case 3, DL

| Company | Victim | Observation Point |  | Performance degradation |  |  |  |  |  |  |  | Performance degradation reference (Mbps) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |  |
|  |  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 20 | 26 |  |
| Ericsson | SBFD DL | 5% | SINR degradation (dB) | 0.16 | 0.05 |  |  |  |  |  |  | 0.25 |
|  |  |  | Throughput degradation (%) | 2.2 | 0.79 |  |  |  |  |  |  | 3 |
|  |  |  | 50% SINR degradation (dB) | 0.09 | 0.002 |  |  |  |  |  |  | 0.23 |
|  |  |  | Mean Throughput degradation (%) | 0.55 | 0.29 |  |  |  |  |  |  | 0.87 |

Table 2.3.2-5: Scenario 2, Case 4, UL

| Company | Victim | Observation Point |  | Performance degradation |  |  |  |  |  |  |  | Performance degradation reference (Mbps) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |  |
|  |  |  |  | 0 | 2 | 4 | 6 | 8 | 14 | 20 | 26 |  |
| Ericsson | SBFD UL | 5% | SINR degradation (dB) | 0.41 | 0.25 | 0.19 |  |  |  |  |  | 0.08 |
|  |  |  | Throughput degradation (%) | 5.5 | 3.3 | 2.5 |  |  |  |  |  | 1.24 |
|  |  |  | 50% SINR degradation (dB) | 0.1 | 0.08 | 0.02 |  |  |  |  |  | 0.14 |
|  |  |  | Mean Throughput degradation (%) | 0.45 | 0.25 | 0.17 |  |  |  |  |  | 0.87 |

Table 2.3.2-6: Scenario 2, Case 4, DL

| Company | Victim | Observation Point |  | Performance degradation |  |  |  |  |  |  |  | Performance degradation reference (Mbps) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |  |
|  |  |  |  | 0 | 2 | 4 | 6 | 8 | 14 | 20 | 26 |  |
| Ericsson | SBFD DL | 5% | SINR degradation (dB) | 0.03 | 0.004 | 0.004 |  |  |  |  |  | 0.25 |
|  |  |  | Throughput degradation (%) | 0.9 | 0.8 | 0.07 |  |  |  |  |  | 3 |
|  |  |  | 50% SINR degradation (dB) | 0.09 | 0.08 | 0.07 |  |  |  |  |  | 0.23 |
|  |  |  | Mean Throughput degradation (%) | 0.6 | 0.4 | 0.3 |  |  |  |  |  | 0.87 |

    - Scenario 6

The results for scenario 6 are listed in Table 2.3.3-1 to Table 2.3.3-9.

Table 2.3.3-1: Scenario 6, Case 1

| Company | Observation Point |  | Performance degradation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |
|  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 20 | 24 |
| Ericsson | 5% | SINR degradation (dB) | 0.35 | 0.22 | 0.22 |  |  |  |  |  |
|  |  | Throughput degradation (%) | 3.6 | 2.3 | 2.3 |  |  |  |  |  |
|  |  | 50% SINR degradation (dB) | 0.13 | 0.09 | 0.02 |  |  |  |  |  |
|  |  | Mean Throughput degradation (%) | 0.5 | 0.3 | 0.2 |  |  |  |  |  |

Table 2.3.3-2: Scenario 6, Case 2 (with priority assumptions)

| Company | Observation Point |  | Performance degradation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |
|  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 20 | 24 |
| Ericsson | 5% | SINR degradation (dB) | 0.91 | 0.79 | 0.6 | 0.44 | 0.21 |  |  |  |
|  |  | Throughput degradation (%) | 9 | 7.8 | 5.9 | 4.4 | 2 |  |  |  |
|  |  | 50% SINR degradation (dB) | 0.15 | 0.12 | 0.1 | 0.08 | 0.05 |  |  |  |
|  |  | Mean Throughput degradation (%) | 1.75 | 1.49 | 1.19 | 0.96 | 0.56 |  |  |  |

Table 2.3.3-3: Scenario 6, Case 2 (with optional TRP 40 dBm, and Grid-shift 10%)

| Company | Observation Point |  | Performance degradation |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |  |
|  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 20 | 24 | TRP= 40 dBm, GS = 10% |
| Ericsson | 5% | SINR degradation (dB) | 11.9 | 10.4 | 9.6 | 8.3 | 6.7 | 6.7 |  |  |  |
|  |  | Throughput degradation (%) | 82.5 | 76 | 73 | 66 | 56 | 56 |  |  |  |
|  |  | 50% SINR degradation (dB) | 1.2 | 1 | 0.9 | 0.75 | 0.58 | 0.55 |  |  |  |
|  |  | Mean Throughput degradation (%) | 15.3 | 13.3 | 12.7 | 11.3 | 9.5 | 9.5 |  |  |  |

Table 2.3.3-4: Scenario 6, Case 3 (with priority assumptions), UL

| Company | Victim | Observation Point |  | Performance degradation |  |  |  |  |  |  |  | Performance degradation reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |  |
|  |  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 20 | 26 |  |
| Ericsson | SBFD UL | 5% | SINR degradation (dB) | 0.89 | 0.77 | 0.51 | 0.38 | 0.16 |  |  |  | 0.3 |
|  |  |  | Throughput degradation (%) | 9.1 | 7.9 | 5.3 | 3.9 | 1.73 |  |  |  | 3 |
|  |  |  | 50% SINR degradation (dB) | 0.1 | 0.08 | 0.07 | 0.05 | 0.03 |  |  |  | 0.05 |
|  |  |  | Mean Throughput degradation (%) | 1.56 | 1.32 | 1 | 0.76 | 0.43 |  |  |  | 0.6 |

Table 2.3.3-5: Scenario 6, Case 3 (with priority assumptions), DL

| Company | Victim | Observation Point |  | Performance degradation |  |  |  |  |  |  |  | Performance degradation reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |  |
|  |  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 20 | 26 |  |
| Ericsson | SBFD DL | 5% | SINR degradation (dB) | 0.5 | 0.4 | 0.2 |  |  |  |  |  | 0.5 |
|  |  |  | Throughput degradation (%) | 5 | 4.5 | 3.1 |  |  |  |  |  | 3 |
|  |  |  | 50% SINR degradation (dB) | 0.4 | 0.4 | 0.2 |  |  |  |  |  | 0.6 |
|  |  |  | Mean Throughput degradation (%) | 1.2 | 0.9 | 0.6 |  |  |  |  |  | 0.6 |

Table 2.3.3-6: Scenario 6, Case 3 (with optional TRP 40 dBm, and Grid-shift 10%), UL

| Company | Victim | Observation Point |  | Performance degradation |  |  |  |  |  |  |  | Performance degradation reference |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 20 | 26 |  |  |
| Ericsson | SBFD UL | 5% | SINR degradation (dB) | 11.8 | 11.2 | 10.3 | 9.4 | 7.7 |  |  |  | 0.05 | TRP= 40 dBm, GS = 10% |
|  |  |  | Throughput degradation (%) | 85.4 | 83.4 | 80.1 | 76.6 | 68 |  |  |  | 0.5 |  |
|  |  |  | 50% SINR degradation (dB) | 1.15 | 1 | 0.96 | 0.87 | 0.77 |  |  |  | 0.03 |  |
|  |  |  | Mean Throughput degradation (%) | 13.9 | 13.2 | 12.2 | 11.2 | 9.9 |  |  |  | 0.22 |  |

Table 2.3.3-7: Scenario 6, Case 3 (with optional TRP 40 dBm, and Grid-shift 10%), DL

| Company | Victim | Observation Point |  | Performance degradation |  |  |  |  |  |  |  | Performance degradation reference |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 20 | 26 |  |  |
| Ericsson | SBFD DL | 5% | SINR degradation (dB) | 0.43 | 0.4 | 0.24 |  |  |  |  |  | 0.17 | TRP= 40 dBm, GS = 10% |
|  |  |  | Throughput degradation (%) | 3.7 | 3.5 | 2.1 |  |  |  |  |  | 1.5 |  |
|  |  |  | 50% SINR degradation (dB) | 1.4 | 1.1 | 0.8 |  |  |  |  |  | 1.6 |  |
|  |  |  | Mean Throughput degradation (%) | 1.7 | 1.1 | 0.7 |  |  |  |  |  | 1.7 |  |

Table 2.3.3-8: Scenario 6, Case 4, UL

| Company | Victim | Observation Point |  | Performance degradation |  |  |  |  |  |  |  | Performance degradation reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |  |
|  |  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 20 | 26 |  |
| Ericsson | SBFD UL | 5% | SINR degradation (dB) | 0.02 | 0.02 | 0.007 |  |  |  |  |  | 0.3 |
|  |  |  | Throughput degradation (%) | 0.19 | 0.7 | 0.2 |  |  |  |  |  | 3 |
|  |  |  | 50% SINR degradation (dB) | 0.007 | 0.002 | 0.001 |  |  |  |  |  | 0.05 |
|  |  |  | Mean Throughput degradation (%) | 0.08 | 0.04 | 0.04 |  |  |  |  |  | 0.6 |

Table 2.3.3-9: Scenario 6, Case 4, DL

| Company | Victim | Observation Point |  | Performance degradation |  |  |  |  |  |  |  | Performance degradation reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |  |
|  |  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 20 | 26 |  |
| Ericsson | SBFD DL | 5% | SINR degradation (dB) | 0.08 | 0.04 | 0.03 |  |  |  |  |  | 0.5 |
|  |  |  | Throughput degradation (%) | 0.8 | 0.4 | 0.4 |  |  |  |  |  | 3 |
|  |  |  | 50% SINR degradation (dB) | 013 | 0.1 | 0.1 |  |  |  |  |  | 0.6 |
|  |  |  | Mean Throughput degradation (%) | 0.2 | 01 | 0.1 |  |  |  |  |  | 0.6 |

    - Scenario 7

The results for scenario 7 are listed in Table 2.3.4-1 to Table 2.3.4-9.

Table 2.3.4-1: Scenario 7, Case 1

| Company | Observation Point |  | Performance degradation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |
|  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 20 | 24 |
| Ericsson | 5% | SINR degradation (dB) | 0.47 | 0.29 | 0.28 |  |  |  |  |  |
|  |  | Throughput degradation (%) | 5.5 | 3.4 | 3.3 |  |  |  |  |  |
|  |  | 50% SINR degradation (dB) | 0.23 | 0.17 | 0.11 |  |  |  |  |  |
|  |  | Mean Throughput degradation (%) | 0.79 | 0.6 | 0.4 |  |  |  |  |  |

Table 2.3.4-2: Scenario 7, Case 2 (priority assumptions)

| Company | Observation Point |  | Performance degradation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |
|  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 20 | 24 |
| Ericsson | 5% | SINR degradation (dB) | 1.19 | 1 | 0.97 | 0.86 | 0.68 | 0.66 | 0.6 | 0.6 |
|  |  | Throughput degradation (%) | 12.5 | 11.4 | 10.3 | 9.1 | 7.3 | 7.1 | 6.4 | 6.4 |
|  |  | 50% SINR degradation (dB) | 0.2 | 0.18 | 0.14 | 0.12 | 0.08 | 0.07 | 0.06 | 0.05 |
|  |  | Mean Throughput degradation (%) | 2.4 | 2.1 | 1.77 | 1.55 | 1.13 | 1 | 0.97 | 0.93 |

Table 2.3.4-3: Scenario 7, Case 2 (with optional TRP 40 dBm, and Grid-shift 10%)

| Company | Observation Point |  | Performance degradation |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |  |
|  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 20 | 24 | TRP= 40 dBm, GS = 10% |
| Ericsson | 5% | SINR degradation (dB) | 9.7 | 9.5 | 9.5 | 9.5 | 7.2 | 7 |  |  |  |
|  |  | Throughput degradation (%) | 75 | 75 | 75 | 75 | 58.8 | 62 |  |  |  |
|  |  | 50% SINR degradation (dB) | 1.3 | 1.2 | 1.1 | 0.98 | 0.76 | 0.74 |  |  |  |
|  |  | Mean Throughput degradation (%) | 15.1 | 14.5 | 13.1 | 12.4 | 10.1 | 10.1 |  |  |  |

Table 2.3.4-4: Scenario 7, Case 3 (priority assumptions), UL

| Company | Victim | Observation Point |  | Performance degradation |  |  |  |  |  |  |  | Performance degradation reference (Mbps) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |  |
|  |  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 20 | 26 |  |
| Ericsson | SBFD UL | 5% | SINR degradation (dB) | 0.499 | 0.5 | 0.28 | 0.27 |  |  |  |  | 0.3 |
|  |  |  | Throughput degradation (%) | 6 | 5.6 | 3.2 | 3.1 |  |  |  |  | 3.2 |
|  |  |  | 50% SINR degradation (dB) | 0.09 | 0.09 | 0.07 | 0.05 |  |  |  |  | 0.07 |
|  |  |  | Mean Throughput degradation (%) | 1.38 | 1.26 | 0.97 | 0.7 |  |  |  |  | 0.8 |

Table 2.3.4-5: Scenario 7, Case 3 (priority assumptions), DL

| Company | Victim | Observation Point |  | Performance degradation |  |  |  |  |  |  |  | Performance degradation reference (Mbps) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |  |
|  |  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 20 | 26 |  |
| Ericsson | SBFD DL | 5% | SINR degradation (dB) | 0.8 | 0.6 | 0.4 |  |  |  |  |  | 0.8 |
|  |  |  | Throughput degradation (%) | 9 | 7 | 5 |  |  |  |  |  | 9.5 |
|  |  |  | 50% SINR degradation (dB) | 0.7 | 0.6 | 0.4 |  |  |  |  |  | 0.8 |
|  |  |  | Mean Throughput degradation (%) | 1.9 | 1.5 | 1 |  |  |  |  |  | 2.1 |

Table 2.3.4-6: Scenario 7, Case 3-B (with optional TRP 40 dBm, and Grid-shift 10%), UL

| Company | Victim | Observation Point |  | Performance degradation |  |  |  |  |  |  |  | Performance degradation reference |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 20 | 26 |  |  |
| Ericsson | SBFD UL | 5% | SINR degradation (dB) | 9.7 | 8.3 | 7.8 | 7 | 6.6 | 6 | 5.6 |  | 0.04 | TRP= 40 dBm, GS = 10% |
|  |  |  | Throughput degradation (%) | 80.3 | 74.3 | 71.5 | 66.7 | 64.1 | 60.5 | 57 |  | 0.44 |  |
|  |  |  | 50% SINR degradation (dB) | 1.3 | 1.2 | 1.1 | 1 | 0.8 | 0.8 | 0.8 |  | 0.08 |  |
|  |  |  | Mean Throughput degradation (%) | 13.8 | 12.6 | 11.6 | 10.7 | 9.3 | 9 | 8.5 |  | 0.47 |  |

Table 2.3.4-7: Scenario 7, Case 3-B (with optional TRP 40 dBm, and Grid-shift 10%), DL

| Company | Victim | Observation Point |  | Performance degradation |  |  |  |  |  |  |  | Performance degradation reference |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 20 | 26 |  |  |
| Ericsson | SBFD DL | 5% | SINR degradation (dB) | 0.37 | 0.21 | 0.11 |  |  |  |  |  | 0.15 | TRP=40 dBm, GS= 10% |
|  |  |  | Throughput degradation (%) | 3.9 | 2.3 | 1.2 |  |  |  |  |  | 1.5 |  |
|  |  |  | 50% SINR degradation (dB) | 2.2 | 1.8 | 1.3 |  |  |  |  |  | 2.8 |  |
|  |  |  | Mean Throughput degradation (%) | 3 | 2.1 | 1.3 |  |  |  |  |  | 3.3 |  |

Table 2.3.4-8: Scenario 7, Case 4, UL

| Company | Victim | Observation Point |  | Performance degradation |  |  |  |  |  |  |  | Performance degradation reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |  |
|  |  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 20 | 26 |  |
| Ericsson | SBFD UL | 5% | SINR degradation (dB) | 0.28 | 0.14 | 0.1 |  |  |  |  |  | 0.3 |
|  |  |  | Throughput degradation (%) | 3.2 | 1.6 | 1.2 |  |  |  |  |  | 3.2 |
|  |  |  | 50% SINR degradation (dB) | 0.06 | 0.05 | 0.04 |  |  |  |  |  | 0.07 |
|  |  |  | Mean Throughput degradation (%) | 0.78 | 0.56 | 0.38 |  |  |  |  |  | 0.8 |

Table 2.3.4-9: Scenario 7, Case 4, DL

| Company | Victim | Observation Point |  | Performance degradation |  |  |  |  |  |  |  | Performance degradation reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |  |
|  |  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 20 | 26 |  |
| Ericsson | SBFD DL | 5% | SINR degradation (dB) | 0.12 | 0.05 | 0.03 |  |  |  |  |  | 0.8 |
|  |  |  | Throughput degradation (%) | 1.5 | 0.6 | 0.4 |  |  |  |  |  | 9.5 |
|  |  |  | 50% SINR degradation (dB) | 0.12 | 0.11 | 0.08 |  |  |  |  |  | 0.8 |
|  |  |  | Mean Throughput degradation (%) | 0.37 | 0.29 | 0.2 |  |  |  |  |  | 2.1 |

# Conclusion

In this contribution we present simulation results based on assumptions agreed last meeting for scenario 1, 2, 6 and 7. From the simulation results following observations have been captured:

Observation 1: Coexistence of an SBFD network with a DL legacy TDD network is possible in a scenario where users are uniformly distributed. In this case, the UE-to-UE CLI does not cause harmful impact against the DL of the legacy network. However, this scenario may hide coexistence issues, because when users are uniformly distributed over a wide area, the probability that two users active in UL and DL at the same time are dropped close enough to each other to generate UE-to-UE CLI is extremely low.

Observation 2: For coexistence Case 1, where DL TDD is the victim, it is of interest to study as well the Urban Hotspot scenario: where the users are clustered, and the distance among them is reduced, it is higher the probability that users can interfere among each other. In this case, we observe that the DL mean user throughput is not affected by the coexistence with SBFD, and the 5%-tile throughput is, with up to 8.7% degradation with respect to the baseline. This requires an ACIR increment up to at least 34 dB. To achieve 34 dB assumption on UE ACS for TDD users should be improved.

Observation 3: In FR1, the UL of a TDD network is highly impacted by the coexistence with an SBFD network. 17-18% degradation is observed (in different scenarios) in terms of mean user throughput, and 67-73% in terms of 5%-tile user throughput.

Observation 4: In FR1, when blocking is modelled at SBFD and TDD BS, approximately a 2% of blocking probability is observed in our simulation, due to the CLI generated by the DL of SBFD neighbour operator. This probability of blocking, together with the increase in noise figure, harmfully impacts the UL performance of the TDD network in such a way that even increasing the ACIR to very high values, it is not possible to reduce the degradation to an acceptable level, below 5%, with respect to the baseline.

Observation 5: In FR1, the UL of a SBFD network is highly impacted by the coexistence with a legacy TDD network. Approximately 15% degradation is observed in terms of mean user throughput, and 60% in terms of 5%-tile user throughput.

Observation 6 In FR1, when blocking is modelled at SBFD and TDD BS, approximately a 2% of blocking probability is observed in our simulation, due to the CLI generated by the DL of legacy TDD neighbour operator. This probability of blocking, together with the increment in resulting noise figure, as defined by the blocking model, harmfully impacts the UL performance of SBFD network in such a way that even increasing the BS-BS ACIR, it is not possible to reduce the degradation to an acceptable level, below 5%, with respect to the baseline.

Observation 7 It is important to understand the difference between the interference captured by the BS receiver chain, which degrades the signal SINR and consequently the throughput performance, and the total received signal in the operation band, which can potentially saturate the BS receiver. The increment of ACIR_BS_BS can reduce the impact of BS-to-BS interference, but cannot reduce the risk of blocking of the receiver, and this is what we observe in the UL results reported for cases 2 and 3, when the UL of TDD and SBFD are victim of the DL transmissions from the other operator.

Observation 8: In FR1, and Urban Macro scenario, the TDD legacy UL does not generate disruptive interference against the DL or UL of SBFD. On the other hand, in urban hotspot scenario the DL 5%-tile can be slightly degraded due to the presence of UE-to-UE CLI generated by TDD legacy UL.

Observation 9: In FR2 scenarios, when users are not clustered, the operation of SBFD does not impact the DL of the legacy TDD operator. When users are clustered, the chances for UE-to-UE interference increase and while the DL mean user throughput is not impacted by the operation of SBFD, the 5%-tile slightly is, which would require an increment of 2 dB in ACIR_UE_UE, and so the ACLR of SBFD users.

Observation 10: In FR2, when SBFD DU is the aggressor, and TDD UL the victim, depending on the deployment of the users, not only DL, but also UL can generate harmful interference against the victim UL. The two UL have different bandwidths (20 MHz the aggressor and 100 MHz the victim), so it is important to define the scaling of ACIR_UE_BS and for all other similar cases.

Observation 11: The coexistence between SBFD DU and TDD UL is extremely sensitive to parameters like the GS and the BS transmission power. If those values are set to 10% and 40 dBm, respectively, there is a 4% percentage of complete blocking of the receiver in our simulations, which, together with the modelled increment of noise figure, makes that even increasing the ACIR_BS_BS, it is not possible to reduce the degradation due to ACI below 5%.

Observation 12: In FR2, when SBFD is the victim of DL TDD, the transmission power of the BS is not high (30 dBm), and the grid-shift between operators is maximum (100%), the ACI is not so relevant for the UL performance, and the internal interference of SBFD dominates the interference pattern, so that the coexistence is possible while assuming certain improvements, compared to the current assumptions.

Observation 13: In FR2, when SBFD is the victim of DL TDD, if the transmission power of the BS is increased (40 dBm) and the grid-shift between operators is reduced (10%), the ACI becomes more relevant, the probability of blocking of the receiver BS increases to 4% in our simulation, and together with the increment of noise figure to model the blocking, makes that even when increasing the ACIR_BS_BS the degradation cannot be eliminated.

In addition, the results have been formatted in table format agreed last meeting in section 2.3.

Based on the detailed analysis of these results, we conclude the following:

- FR1 scenarios are more challenging than FR2 for coexistence when CLI is involved, due to higher transmission power, and reduced directionality of transmissions compared to FR2.

- FR1 scenarios where UL is the victim are impacted from both mean and 5%-tile throughput perspectives, since BS-to-BS CLI is the most limiting interference component.

- DL FR1 is less vulnerable to CLI than UL. Scenarios where users are far away do not suffer from degradation of performance due to UE-to-UE CLI. However, when users are closer due to deployments that cluster them, 5%-tile throughput is impacted by coexistence.

- FR2 coexistence results in both UL and DL are extremely sensitive to simulation assumptions: if the scenario considers users deployed far away from each other (uniform distribution), the GS is optimistically set to 100%, the transmission power is low, the LoS model tends to increase the isolation is BS-BS links, as is defined in the first priority assumptions, coexistence performance is quite optimistic. However, when less optimistic assumptions are considered, coexistence becomes more challenging, so that it is important to study more scenario configurations including different options, before drawing conclusions.

- Assuming clustered UEs (hot spot scenario) we see impact for both FR1 and FR2.

# References

[1]	R4-2309794, “Ad-hoc minutes for co-existence study”, CMCC

[2] 	R4-2307702, “Additional SBFD coexistence simulation results based on updated assumptions”, Ericsson

[3]	R4-2309793, “WF for SBFD co-existence study”, CMCC

[4]	R4-2220247, “Simulation assumptions for SBFD adjacent channel co-existence study”, Samsung

[5]	R4-2312377, “TP to TR 38.858: Addition of coexistence simulation assumptions to Annex E”, Ericsson

# Annex

In Table 5-1 and Table 5-2, parameters considered for simulation campaign relevant for urban macro and urban hotspot scenarios, FR1 and FR2 are listed.

Table 5-1: Urban Macro/Urban Hotspot scenario FR1

|  | Parameters | Scenario |
| --- | --- | --- |
| System parameters | Scenario | UMa, Hexagonal layout, 19 BS per operator, 3 sectors per site, with wrapping |
|  | ISD | 500 m |
|  | Carrier Frequency | 4 GHz |
|  | Duplex Type | Static TDD (DDDDU), SBFD (XXXXX) |
|  | Base Static TDD pattern | 80:20 DL:UL |
|  | SBFD pattern | 100% SBFD slots |
|  | Channel bandwidth | 100 MHz for STDD 80:20 MHz (DU) for SBFD |
|  | Available resource blocks | 273 for STDD 218:55 (DU) for SBFD |
|  | Switching time | DL->UL: 2OS in the D slot |
|  | Sub-Carrier spacing | 30 kHz |
|  | Number of active UEs | 1 active users in UL or DL per cell at a time |
|  | Channel model | gNB-UE: UMa TR 38.803/38.828 gNB-gNB: UMa TR 38.803/38.828 UE-UE: UMi TR 38.803/38.828 (for d>10 m), FSPL (for d<10m) |
|  | UE to BS min 2D distance | 35 m |
|  | Grid-shift | 100% |
| BS | (Mg, Ng, M, N, P) | (1,1,8,8,2) (same antenna gain) (1,1,4,8,2) (same antenna area) |
|  | Sub-array configuration | 1x1 |
|  | gNB Tx Power | 49 dBm (same antenna gain) |
|  | (dv, dh) | (0.5λ, 0.8λ) |
|  | Antenna element gain | 5 dBi |
|  | Antenna element | TR 38.803 |
|  | Subarray electrical down-tilt | N/A |
|  | Mechanical down-tilt | 6 deg |
|  | Beamforming method | Frequency domain |
|  | Noise figure | 5 dB |
|  | Link level model | As per TR 38.803 |
|  | BS height | 25 m |
|  | Panel HW assumptions | Same antenna gain, same antenna area |
| UE | UE antenna | 1TX 2RX |
|  | Antenna model | isotropic |
|  | Antenna element gain | 0 dBi |
|  | Max UE TX Power | 23 dBm |
|  | UE power control | Sec. 9.1 TR36.942 |
|  | SNR target | 15 dB |
|  | Noise figure | 9 dB |
|  | Link level model | As per TR 38.803 |
|  | UE distribution outdoor/indoor | 80:20 for uniform distribution 20: 80 for clustered distribution |
|  | Clusters | Circular zones with radius 25 m, 1 cluster per cell |

Table 5-1: Urban Macro/Urban Hotspot scenario FR1

|  | Parameters | Scenario |
| --- | --- | --- |
| System parameters | Scenario | UMa, Hexagonal layout, 19 BS per operator, 3 sectors per site, with wrapping |
|  | ISD | 200 m |
|  | Carrier Frequency | 30 GHz |
|  | Duplex Type | Static TDD (DDDDU), SBFD (XXXXX) |
|  | Base Static TDD pattern | 80:20 DL:UL |
|  | SBFD pattern | 100% SBFD slots |
|  | Channel bandwidth | 200 MHz for STDD 80:20 MHz (DU) for SBFD |
|  | Available resource blocks | 132 for STDD 106:26 (DU) for SBFD |
|  | Switching time | DL->UL: 2OS in the D slot |
|  | Sub-Carrier spacing | 120 kHz |
|  | Number of active UEs | 1 active users in UL or DL per cell at a time |
|  | Channel model | gNB-UE: UMa TR 38.803/38.828 gNB-gNB: UMa TR 38.803/38.828 UE-UE: UMi TR 38.803/38.828 (for d>10 m), FSPL (for d<10m) |
|  | UE to BS min 2D distance | 35 m |
|  | Grid-shift | 100% |
| BS | (Mg, Ng, M, N, P) | (1,1,16,8,2) (same antenna gain) |
|  | Sub-array configuration | 1x1 |
|  | Max gNB Tx Power | 30 dBm (same antenna gain) |
|  | (dv, dh) | (0.5λ, 0.5λ) |
|  | Antenna element gain | 3 dBi |
|  | Antenna element | TR 38.803 |
|  | Subarray electrical down-tilt | N/A |
|  | Mechanical down-tilt | 6 deg |
|  | Beamforming method | Frequency domain |
|  | Noise figure | 10 dB |
|  | Link level model | As per TR 38.803 |
|  | BS height | 25 m |
|  | Panel HW assumptions | Same antenna gain |
| UE | UE antenna | 1TX 2RX |
|  | Antenna model | (1,1,2,2,2) |
|  | Antenna element gain | 5.5 dBi |
|  | Max UE TX Power | 22.4 dBm EIRP |
|  | UE power control | Sec. 9.1 TR 36.942 |
|  | SNR target | 15 dB |
|  | Noise figure | 10 dB |
|  | Link level model | As per TR 38.803 |
|  | UE distribution outdoor/indoor | 100:0 for uniform distribution 100: 0 for clustered distribution |
|  | Clusters | Circular zones with radius 25 m, 1 cluster per cell |