3GPP TSG-RAN WG4 Meeting # 108	R4-2312405

# Toulouse, France, August 21 – August 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 7232 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [NB_IOTenh4_LTE_eMTC6-Core] CR on maintenance of neighbour cell measurement for NB-IoT R17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh4_LTE_eMTC6-Core |  |  |  |  |  | Date: |  |  | 2023-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The reference of detectable conditions is TBD - The variables used in intra-frequency and inter-frequency requirements are misaligned when referred to each other. - There are typos in the requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the reference of detectable conditions. - Align the variables used in intra-frequency and inter-frequency requirements - Fix the typos |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements are incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.14.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Change 1>

### 8.14.6	NB-IoT neighbour cell measurements

#### 8.14.6.1	Introduction

This clause contains requirements for the neighbour cell measurements performed by the UE category NB1 in RRC_CONNECTED state. The requirements in this clause are applicable when:

- the UE is in normal coverage or in enhanced coverage on the serving cell and

- the target cell fulfils the criteria for normal coverage.

#### 8.14.6.2	Requirements

The UE supporting connectedModeMeasurements-r17 [31] shall measure neighbour cells when the criterion for triggering the neighbour cell measurements defined in [1] is fulfilled and the UE is not in relaxed monitoring mode as defined in [1]. The UE may trigger the neighbour cell measurements before or during radio link failure depending on the fulfilment of the criterion.

The measurement quantities are defined in [4], the measurement model is defined in [22].

The requirements for intra-frequency neighbour cell measurement when the target carrier is same as serving carrier is defined in clause 8.14.6.3.

The requirements for inter-frequency neighbour cell measurement when the target carrier is different from serving carrier is defined in clause 8.14.6.4.

#### 8.14.6.3	Intra-frequency neighbour cell measurements

The UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra when the criteria for intra-frequency measurements is fulfilled [1]. An intra frequency cell is considered to be detectable according to NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot defined in Annex B.2.24 for a corresponding Band.

Tidentify_intra = Tdetect_intra_NB1-NC + Tmeasure _intra_NB1-NC

When DRX is not used, Tdetect_intra_NB1-NC is 1400 ms, and Tmeasure _intra_NB1-NC is 800 ms and 1600 ms for NRS-based measurement and NSSS-based measurement respectively.

When DRX is used, Tdetect_intra_NB1-NC and Tmeasure _intra_NB1-NC are defined in table 8.14.6.3-1 and 8.14.6.3-2.

Table 8.14.6.3-1: Requirement for intra-frequency detection

| DRX cycle length (s) | Tdetect_intra (s) (DRX cycles) |
| --- | --- |
| 0.256<DRX-cycle≤10.24 | (6)Note 1 |
| Note1: Time depends upon the DRX cycle in use |  |

Table 8.14.6.3-2: Requirement for intra-frequency measurement

| DRX cycle length (s) | Tmeasure_intra (s) (DRX cycles) |
| --- | --- |
| 0.256<DRX-cycle≤10.24 | (5)Note 1 |
| Note1: Time depends upon the DRX cycle in use |  |

When UE is monitoring multiple carriers, Tidentify_intra_NB1-NC = Tdetect_NB1-NC + Tmeasure_NB1-NC, where Tdetect_NB1-NC = Tdetect _intra_NB1-NC +Nfreq* Tdetect_inter_NB1-NC and Tmeasure = Tmeasure _intra_NB1-NC +Nfreq* Tmeasure_inter_NB1-NC. Nfreq is number of inter-frequency carriers to be measured according to the measurement capability, and Tdetect_inter_NB1-NC and Tmeasure_inter_NB1-NC are defined in clause 8.14.6.4

#### 8.14.6.4	Inter-frequency neighbour cell measurements

The UE shall be able to identify a new detectable inter-frequency cell within Tidentify_inter_NB1-NC when the criteria for inter-frequency measurement is fulfilled [1]. An inter frequency cell is considered to be detectable according to NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot defined in Annex B.2.25 for a corresponding Band.

Tidentify _inter_NB1-NC = Tdetect_inter_NB1-NC + Tmeasure _inter_NB1-NC

Where

Tdetect_inter_NB1-NC=iNMin(5000, Ta,i) ms

-	N = 70,

-	Ta,i is the interval between available measurement samples in measurement occasions (MOdetect_inter_NB1-NC) for inter-frequency detection, where

40 ms ≤  Ta,i ≤ 5000 ms

-	The UE shall restart the cell detection when the interval between two samples are larger than 5000 ms.

-	The UE is not required to monitor NPSS/NSSS more frequent than once per 40ms.

-	MOdetect_inter_NB1-NC are time occasions containing NPSS/NSSS and fulfil the following conditions:

-	Resources on which the UE is not scheduled for data transmission or reception,

-	Resources on which the UE is not required to do NPDCCH monitoring,

-	Resources occurring during the DRX inactive period

-	Length of MOdetect_inter_NB1-NC  is at least 200 ms.

-	The inter-frequency detection requirements apply when Tdetect_inter_NB1-NC ≤ 60 seconds per inter-frequency carrier.

Tmeasure_inter_NB1-NC is the physical layer measurement period of NRSRP on the detected inter-frequency cell as defined below:

Tmeasure_inter_NB1-NC=iMMin(5000, Tb,i) ms

-	M = 60 for NRS-based RRM measurement and M = 40 for NSSS based RRM measurement,

-	Tb,i is the interval between available measurement samples in measurement occasions (MOmeasure_inter_NB1-NC) for inter-frequency measurement, where

20 ms ≤  Tb,i ≤ 5000 ms for NRS based measurement or

40 ms ≤ Tb,i ≤ 5000 ms for NSSS-based measurement

-	The UE shall restart the measurement when the interval between two samples are larger than 5000 ms.

-	The UE is not required to monitor NRS more frequent than once per 20ms for NRS-based measurement and NSSS more frequent than 40 ms for NSSS-based measurement.

-	MOmeasure_inter_NB1-NC are time occasion containing at least NRS or NSSS that fulfil the following conditions:

-	Resources on which the UE is not scheduled for data transmission or reception,

-	Resources on which the UE is not required to do NPDCCH monitoring,

-	Resources occurring during the DRX inactive period,

-	Length of MOmeasure_inter_NB1-NC  is at least 50 ms.

-	The inter-frequency measurement requirements apply when Tmeasure_inter_NB1-NC ≤ 50 seconds per inter-frequency carrier.

When UE is monitoring multiple carriers, Tidentify_inter_NB1-NC = Tdetect_NB1-NC + Tmeasure_NB1-NC, where Tdetect_NB1-NC = Tdetect _intra_NB1-NC +Nfreq* Tdetect_inter_NB1-NC and Tmeasure_NB1-NC = Tmeasure _intra_NB1-NC +Nfreq* Tmeasure_inter_NB1-NC. Nfreq is number of inter-frequency carriers to be measured according to the measurement capability, and Tdetect_intra_NB1-NC and Tmeasure_intra_NB1-NC are defined in clause 8.14.6.3.

#### 8.14.6.5	 Requirements for monitoring multiple carriers

For RRC_CONNECTED state, the UE shall be capable of monitoring at least:

-	Depending on UE capability, an intra-frequency carrier.

-	Depending on UE capability, at least 2 inter-frequency carriers.

### <End of Change 1>