3GPP TSG-RAN WG4 Meeting # 108	R4-2312417

# Toulouse, France, August 21 – August 25, 2023

Agenda item:	8.33.4

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM requirements for mIAB

Document for:	Discussion

# Introduction

The Rel-18 WI Mobile IAB for NR was approved to support mobility scenarios where IAB-nodes are mounted on vehicles. The objectives are described in the latest WID [1]. In last RAN4 meeting, good progress had been made in identifying the scope the impacted RRM requirements [2]. In this, paper we further provide our views on remaining issues.

# Discussion

In RAN4#107, most of RRM requirements for mobile IAB are concluded with agreements as follows:

Intra-frequency HO requirements Reuse the UE intra-frequency HO requirements from TS 38.133 clauses 6.1.1.2 and 6.1.1.4 as baseline. RRC Re-establishment Re-use the UE RRC re-establishment requirements in TS 38.133 clause 6.2.1 for the mobile IAB as baseline. Inter-frequency RRC re-establishment/RRC release with re-direction Do not introduce requirements for inter-frequency RRC re-establishment and for RRC release with re-direction Active TCI state switching Reuse UE requirements for the active TCI state switching delay from TS 38.133 Clause 8.10 as baseline. RLM Requirements UE RLM requirements from TS 38.133 Clause 8.1 shall be taken as baseline and adapted as follows: - Discard requirements related to DRX: requirements depending on DRX cycle length, requirement at transitions - Discard requirements related to measurement gaps and collisions between RLM-RS and measurement gaps - Discard scheduling availability requirements related to FR1-FR2 inter-band CA and NR-DC (clause 8.1.7.4) - Discard Requirements when highSpeedMeasFlagFR2-r17 is configured Link recovery procedure UE link recovery procedures requirements from TS 38.133 Clause 8.5 shall be taken as baseline and adapted as follows: - Discard requirements related to DRX - Discard requirements related to measurement gaps - Discard requirements related to deactivated PSCells/SCells, inter-band CA, FR2-2, etc - Discard requirements when highSpeedMeasFlagFR2-r17 is configured L1-RSRP measurement requirements UE requirements defined in TS 38.133 Rel-15 Clause 9.5 shall be taken as baseline and adapted as follows: - Keep the beam sweeping factors for FR2 - Discard requirements related to DRX - Discard requirements related to measurement gaps(gap sharing) and inter-frequency/inter-RAT measurements Intra-frequency measurement requirements UE intra-frequency measurement requirements defined in TS 38.133 Clause 9.2 should be taken as baseline for the L3 measurement requirements with the following amendments: - Keep the beam sweeping factors - Discard the DRX requirements - Discard the requirements for SFTD and measurements with gaps. Handling of measurement gaps Do not mention measurement gaps impacts in the specification Handling of HST Do not introduce HST requirements unless the WID is updated to include this scenario CA Related Requirements Do not introduce CA requirements Timing related Reuse the transmit timing and timing advance requirements from 38.174 Random access requirements Re-use requirements from TS 38.174

One generic issue which has not been discussed is about the applicability of different IAB types. In Rel-16/17 IAB requirements, IAB-MT are categorized into wide area IAB-MT and local area IAB-MT, and some requirements only apply to local area IAB (RLM/BFR). For R18 mobile IAB, it should be clarified that whether the requirements to be introduced are same for wide area IAB-MT and local area IAB-MT.

Proposal 1: RAN4 to discuss whether to differentiate wide area IAB-MT and local area IAB-MT for RRM requirements to be introduced.

# Conclusions

Proposal 1: RAN4 to discuss whether to differentiate wide area IAB-MT and local area IAB-MT for RRM requirements to be introduced.

# References

[1] RP-222671 Mobile IAB (Integrated Access and Backhaul) for NR

[2] R4-2310085, WF on Mobile IAB RRM requirements