3GPP TSG-RAN WG4 Meeting # 108	R4-2312432

Toulouse, France, August 21 – August 25, 2023

Title:	Reply LS on Monitoring of paging occasions for CG-SDT with HD-FDD Redcap UEs

Response to:	R2-2304562

Release:	Rel-17

Work Item:	NR_SmallData_INACTIVE-Core

Source:	RAN WG4

To:	RAN WG2

Cc:	RAN WG1

Contact Person:

Name:	Jing Han

E-mail Address:	hw.hanjing@huawei.com

Attachments:

1. Overall Description:

RAN4 would like to thank RAN1 for the LS on Monitoring of paging occasions for CG-SDT with HD-FDD Redcap UEs. RAN4 had the following understanding:

For RedCap UE in HD-FDD mode, the UE is required to monitor for SI change indication in any paging occasion at least once per modification period during SDT, if the initial downlink BWP on which the SDT procedure is ongoing is associated with a CD-SSB. In case the paging occasions overlap with CG-SDT transmission, the UE is allowed to drop the CG-SDT transmission.

2. To RAN WG1 and WG2 group.

ACTION: 	RAN4 respectfully ask RAN2 and RAN1 to take the above agreements into account.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 Meeting #108-bis		Oct. 09 – Oct. 13, 2023	Xiamen, CN

TSG-RAN4 Meeting #109			Nov. 13 – Nov. 17, 2023	Chicago, US