3GPP TSG- RAN4 Meeting # 108	R4-2312433

# Toulouse, France, August 21 – August 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 3974 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification on interruption in paging reception for HD-FDD RedCap UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Core |  |  |  |  |  | Date: |  |  | 2023-8-4 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current paging requirements on monitoring of paging occasions for CG-SDT with HD-FDD Redcap UEs are not alligned with RAN2 |  |  |  |  |  |  |  |  |
| s |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | For RedCap UE in HD-FDD mode, the UE is required to monitor for SI change indication in any paging occasion at least once per modification period during SDT,if the initial downlink BWP on which the SDT procedure is ongoing is associated with a CD-SSB. In the following case, UE is allowed to drop the CG-SDT transmission when the paging occasions overlap with CG-SDT transmission: - All paging occasions are overlapped with CG-SDT tansmission - There is one available PO for UE receiving SI change indication where no SDT is to be transmitted , however UE failed to correctly decode the indication. UE needs to wait for another PO, however other PO happened to overlapped with SDT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect implementation for RedCap UE in HD-FDD mode. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1B.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

5.1B.2.6	Maximum interruption in paging reception

The requirements in clause 4.2B.2.6 shall apply for RedCap UEs.

For RedCap UE in HD-FDD mode, the UE is required to monitor for SI change indication in any paging occasion at least once per modification period during SDT if the initial downlink BWP on which the SDT procedure is ongoing is associated with a CD-SSB or if the separate downlink BWP on which the SDT procedure is ongoing is associated with a CD-SSB. In case the paging occasions overlap with CG-SDT transmission, the UE is allowed to drop the CG-SDT transmission.

<End of Change 1>