3GPP TSG-RAN WG4 #108	R4-2312439

# Toulouse, France, August 21st – 25th 2023

Agenda item:	8.5.2.1

Source: 	Qualcomm Incorporated

Title: 	Channel Raster Enhancements

Document for:	Discussion

# Introduction

A WF listing different options for the channel raster enhancement was agreed in [1]. In this paper we further discuss these options and present a channel raster enhancement proposal. The analysis in this paper is based on what was already presented in [2].

# Discussion

## Channel Raster Enhancements/Changes (Approach 1 in [1])

The options for channel raster that were left for analysis are listed below:

- Option 1: 5 kHz

- Option 2: 10 kHz

- Option 3: 50 kHz

As already shown in [2], all these options enable the placement of a narrower channel within a wider channel. These options lead to a tradeoff between flexibility of channel placement and number of added channel raster points(design complexity). Options 1 and 2 offer better flexibility for channel placement, however, not all channel raster points can be used in practice due to several other deployment related restrictions such as channels having to cover an SSB placed on a valid global sync raster entry [3]. Option will lead to only a 2x increase in the number of channel raster points, limiting the number of extra raster points is desirable to minimize the implementation impact of this feature and enabling earlier support of the feature.

Proposal 1: Introduce new channel raster entries with 50kHz granularity.

Regarding whether the channel raster should be specified for the UE or base station, it should be specified for both to enable forward compatibility. If a new channel BW is added to the specifications, in order to enable the placement of the  new channel within/on top of the legacy channel while maintaining the legacy channel on a legacy channel raster entry, the new channel would have to be placed on a new channel raster entry at the base station side also.

Proposal 2: The new channel raster entries should be specified for both UE and base station.

Regarding the coverage of the changes, this should be based on operator request to minimize the changes. As this issue was not brought up until recently, the placement of a narrower channel within a wider channel seems to be a corner case that only applies to a few operators/deployments in the FDD bands which use the 100kHz “legacy” raster. This approach would minimize the UE changes and lead to a faster adoption of this feature.

Proposal 3: The channel raster changes(additions of new channel raster entries) should be based on operator requests and not apply generically to all bands.

The specification changes required by Approach 1 in [1] are trivial, only the granularity of the channel raster entries in the corresponding tables (Table 5.4.2.3-1 of TS 38.101-1) needs to be modified. The changes are shown in the Annex based on proposal 1 (Option 3). For the other options, the step size has to be modified accordingly.

## No new channel raster entries (Approach 2 in [2])

Multiple options are listed in the WF under this approach, however, they all suffer from some fundamental issues as already explained in [2]. The RAN4 specifications are very clear that the RF requirements apply based on the condition that the channel(for both UE and BS) are placed on a valid channel raster entry given in Table 5.4.2.3-1 of TS 38.101-1 or -2. An “off raster” placement will violate one of the basic principles of the RAN4 specifications under which the requirements apply. Also, for testing of the UE RF requirements, the channel raster position is needed to enable the placement of the actual Tx/Rx bandwidth.

Observation 1: UE/gNB RF requirements apply based on the condition that the channel is placed on a valid channel raster entry.

Observation 2: “Offraster” placement would violate one of the basic principles of the RAN4 requirements.

Another argument used in the previous meetings to justify the channel placement “offraster” was that this is enabled by the RRC specifications (TS 38.331). Different 3GPP specifications work together, only one of the specifications cannot be used just by itself. The RRC specifications allows the configuration of many parameters in many different ways, however, this does not mean that all configurations have to be supported in the RAN4 specifications(or other specs). For example, the RRC signaling allows the configuration of intra-band “overlapping” CA, however, this is not supported by the RAN4 specification.

Observation 3: The RRC specifications (TS 38.331) do not “supersede” the RAN4 specifications. Not any possible RRC parameter configuration is defined/supported by the RAN4 specifications.

Furthermore, from a UE designer point of view, the reference for design and requirement compliance is the RAN4 specifications. If any configuration is allowed based on what the signaling allows, it would be very difficult to understand what is the actual set of valid frequencies on which a channel can be configured. Such an approach will also create forward/backward compatibility issues if the NR-ARFCN granularity or the signaling of the channel placement/configuration is modified/extended in the future.

Observation 4: Allowing any channel placement that is enabled by the currently defined signaling will create design/testing issues and could lead to forward/backward compatibility problems in the future.

# Conclusion

In this paper we analyzed the channel raster enhancement options. We made the following proposals and observations:

Proposal 1: Introduce new channel raster entries with 50kHz granularity.

Proposal 2: The new channel raster entries should be specified for both UE and base station.

Proposal 3: The channel raster changes(additions of new channel raster entries) should be based on operator requests and not apply generically to all bands.

Observation 1: UE/gNB RF requirements apply based on the condition that the channel is placed on a valid channel raster entry.

Observation 2: “Offraster” placement would violate one of the basic principles of the RAN4 requirements.

Observation 3: The RRC specifications (TS 38.331) do not “supersede” the RAN4 specifications. Not any possible RRC parameter configuration is defined/supported by the RAN4 specifications.

Observation 4: Allowing any channel placement that is enabled by the currently defined signaling will create design/testing issues and could lead to forward/backward compatibility problems in the future.

The specification changes required by Approach 1 in [1] are trivial, only the granularity of the channel raster entries in the corresponding tables (Table 5.4.2.3-1 of TS 38.101-1) needs to be modified. The changes are shown in the Annex based on proposal 1 (Option 3). For the other options, the step size has to be modified accordingly.

# References

- R4-2310269, “WF on Channel Raster Enhancements”, Ericsson, RAN4#107

- R4-2308787, “Channel Raster Enhancements”, Qualcomm Incorporated, RAN4#107

- R4-2305517, “Channel Raster Enhancements”, Qualcomm Incorporated, RAN4#106-bis-e

# Annex

Specification changes needed to enable 50kHz channel raster granularity (Option 3)for Bands n1,n2,n3,n5,n7. Different channel raster granularity can be enable with a corresponding step size. Support for each band is indicated through Note 5.

#### 5.4.2.3	Channel raster entries for each operating band

The RF channel positions on the channel raster in each NR operating band are given through the applicable NR-ARFCN in Table 5.4.2.3-1, using the channel raster to resource element mapping in clause 5.4.2.2.

For NR operating bands with 100 kHz channel raster, ΔFRaster = 20 × ΔFGlobal. In this case every 20th NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-1 is given as <20>.

For NR operating bands with 100 kHz channel raster, ΔFRaster = 10 × ΔFGlobal is supported if the UE is capable of  “enhancedChannelRasterSupport-rel18”. In this case every 10th NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-1 is <10>.

For NR operating bands with 15 kHz channel raster below 3GHz, ΔFRaster = I × ΔFGlobal, where I ϵ {3,6}. Every Ith NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-1 is given as < I >.

For NR operating bands with 15 kHz channel raster above 3GHz, ΔFRaster = I × ΔFGlobal, where I ϵ {1,2}. Every Ith  NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in table 5.4.2.3-1 is given as <I>.

In frequency bands with two or more ΔFRaster, the higher ΔFRaster: For 15 kHz and 30 kHz channel raster applies to channels using only the SCS that is equal to or larger than the higher ΔFRaster and SSB SCS is equal to the higher ∆FRaster.

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n15 | 100 | 384000 – <20> – 396000 | 422000 – <20> – 434000 |
| n25 | 100 | 370000 – <20> – 382000 | 386000 – <20> – 398000 |
| n35 | 100 | 342000 – <20> – 357000 | 361000 – <20> – 376000 |
| n55 | 100 | 164800 – <20> – 169800 | 173800 – <20> – 178800 |
| n75 | 100 | 500000 – <20> – 514000 | 524000 – <20> – 538000 |
| n8 | 100 | 176000 – <20> – 183000 | 185000 – <20> – 192000 |
| n12 | 100 | 139800 – <20> – 143200 | 145800 – <20> – 149200 |
| n13 | 100 | 155400 – <20> – 157400 | 149200 – <20> – 151200 |
| n14 | 100 | 157600 – <20> – 159600 | 151600 – <20> – 153600 |
| n18 | 100 | 163000 – <20> – 166000 | 172000 – <20> – 175000 |
| n20 | 100 | 166400 – <20> – 172400 | 158200 – <20> – 164200 |
| n24 | 100 | 325300 – <20> – 332100 | 305000 – <20> – 311800 |
| n25 | 100 | 370000 – <20> – 383000 | 386000 – <20> – 399000 |
| n26 | 100 | 162800 – <20> – 169800 | 171800 – <20> – 178800 |
| n28 | 100 | 140600 – <20> – 149600 | 151600 – <20> – 160600 |
| n29 | 100 | N/A | 143400 – <20> – 145600 |
| n30 | 100 | 461000 – <20> – 463000 | 470000 – <20> – 472000 |
| n34 | 100 | 402000 – <20> – 405000 | 402000 – <20> – 405000 |
| n38 | 100 | 514000 – <20> – 524000 | 514000 – <20> – 524000 |
| n39 | 100 | 376000 – <20> – 384000 | 376000 – <20> – 384000 |
| n40 | 100 | 460000 – <20> – 480000 | 460000 – <20> – 480000 |
| n41 | 15 | 499200 – <3> – 537999 | 499200 – <3> – 537999 |
|  | 30 | 499200 – <6> – 537996 | 499200 – <6> – 537996 |
| n462 | 15 | 743334 – <1> – 795000 | 743334 – <1> – 795000 |
| n47 | 15 | 790334 – <1> – 795000 | 790334 – <1> – 795000 |
| n48 | 15 | 636667 – <1> – 646666 | 636667 – <1> – 646666 |
|  | 30 | 636668 – <2> – 646666 | 636668 – <2> – 646666 |
| n50 | 100 | 286400 – <20> – 303400 | 286400 – <20> – 303400 |
| n51 | 100 | 285400 – <20> – 286400 | 285400 – <20> – 286400 |
| n53 | 100 | 496700 – <20> – 499000 | 496700 – <20> – 499000 |
| n54 | 100 | 334000 – <20> – 335000 | 334000 – <20> – 335000 |
| n65 | 100 | 384000 – <20> – 402000 | 422000 – <20> – 440000 |
| n66 | 100 | 342000 – <20> – 356000 | 422000 – <20> – 440000 |
| n67 | 100 | N/A | 147600 – <20> – 151600 |
| n70 | 100 | 339000 – <20> – 342000 | 399000 – <20> – 404000 |
| n71 | 100 | 132600 – <20> – 139600 | 123400 – <20> – 130400 |
| n74 | 100 | 285400 – <20> – 294000 | 295000 – <20> – 303600 |
| n75 | 100 | N/A | 286400 – <20> – 303400 |
| n76 | 100 | N/A | 285400 – <20> – 286400 |
| n77 | 15 | 620000 – <1> – 680000 | 620000 – <1> – 680000 |
|  | 30 | 620000 – <2> – 680000 | 620000 – <2> – 680000 |
| n78 | 15 | 620000 – <1> – 653333 | 620000 – <1> – 653333 |
|  | 30 | 620000 – <2> – 653332 | 620000 – <2> – 653332 |
| n79 | 15 | 693334 – <1> – 733333 | 693334 – <1> – 733333 |
|  | 30 | 693334 – <2> – 733332 | 693334 – <2> – 733332 |
| n80 | 100 | 342000 – <20> – 357000 | N/A |
| n81 | 100 | 176000 – <20> – 183000 | N/A |
| n82 | 100 | 166400 – <20> – 172400 | N/A |
| n83 | 100 | 140600 – <20> –149600 | N/A |
| n84 | 100 | 384000 – <20> – 396000 | N/A |
| n85 | 100 | 139600 – <20> – 143200 | 145600 – <20> – 149200 |
| n86 | 100 | 342000 – <20> – 356000 | N/A |
| n89 | 100 | 164800 – <20> – 169800 | N/A |
| n90 | 15 | 499200 – <3> – 537999 | 499200 – <3> – 537999 |
|  | 30 | 499200 – <6> – 537996 | 499200 – <6> – 537996 |
|  | 100 | 499200 – <20> – 538000 | 499200 – <20> – 538000 |
| n91 | 100 | 166400 – <20> – 172400 | 285400 – <20> – 286400 |
| n92 | 100 | 166400 – <20> – 172400 | 286400 – <20> – 303400 |
| n93 | 100 | 176000 – <20> – 183000 | 285400 – <20> – 286400 |
| n94 | 100 | 176000 – <20> – 183000 | 286400 – <20> – 303400 |
| n95 | 100 | 402000 – <20> – 405000 | N/A |
| n963 | 15 | 795000 – <1> – 875000 | 795000 – <1> – 875000 |
| n97 | 100 | 460000 – <20> – 480000 | N/A |
| n98 | 100 | 376000 – <20> – 384000 | N/A |
| n99 | 100 | 325300 – <20> – 332100 | N/A |
| n100 | 100 | 174880 – <20> – 176000 | 183880 – <20> – 185000 |
| n101 | 100 | 380000 – <20> – 382000 | 380000 – <20> – 382000 |
| n1024 | 15 | 795000 – <1> – 828333 | 795000 – <1> – 828333 |
| n104 | 15 | 828334 – <1> – 875000 | 828334 – <1> – 875000 |
|  | 30 | 828334 – <2> – 875000 | 828334 – <2> – 875000 |
| n105 | 100 | 132600 – <20> – 140600 | 122400 – <20> – 130400 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. NOTE 2: The following NREF are allowed for operation in Band n46: see Table 5.4.2.3-2. NOTE 3: The following NREF are allowed for operation in Band n96: see Table 5.4.2.3-3. NOTE 4: The following NREF are allowed for operation in Band n102: see Table 5.4.2.3-4. NOTE 5: The band supports the enhanced channel raster with step size <10>. |  |  |  |