3GPP TSG-RAN WG4 Meeting #108	R4-2312443

# Toulouse, France, August 21st – August 25th, 2023

# Title:	Updates on NTN calibration and coexistence simulation results for above 10 GHz

# Source:	THALES, Magister Solutions Ltd

# Type:	Discussion

# Document for:	Discussion

# Agenda Item:	8.26.2

Release	Rel-18

# Introduction

The following Satellite 5G NR NTN (Non-Terrestrial Networks) technical documents have been approved at the 3GPP RAN-Plenary #96 (Budapest, 6th-9th of June 2022) for the Release-17 NTN satellite connectivity using FR1 S-band (n256) and FR1 L-band (n255):

- Technical Specification TS 38.108 (NR; Satellite Node radio transmission and reception);

- Technical Specification TS 38.101-5 (NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements);

- Technical Report TR 38.863 (Non-terrestrial networks (NTN) related RF and co-existence aspects);

From Article 5 of the ITU Radio Regulations (RR) the mobile operation of ESIMs is also described in the following footnotes,

- 5.527A The operation of earth stations in motion communicating with the FSS is subject to Resolution 156 (WRC-15). (WRC-15)

- 5.517A The operation of earth stations in motion communicating with geostationary fixed-satellite service space stations within the frequency bands 17.7-19.7 GHz (space-to-Earth) and 27.5-29.5 GHz (Earth-to-space) shall be subject to the application of Resolution 169 (WRC-19). (WRC-19)

At RAN4#105 the following WFs have been approved:

- R4-2220239 (source: THALES, “WF for above 10GHz band definition and system parameters”) with the latest references for Ka-Band frequency bands;

- R4-2220241 (source: Samsung, “Simulation assumptions for NTN co-existence study in bands above 10GHz”) with the latest discussions for the coexistence studies.

Agreements R4-2220239 1/ Companies are invited to study (general) ITU/national regulations and bring contributions/provide more information at this meeting (RAN#105) and next RAN4 meeting. 2/ Since it has been agreed to define several NTN Ka-bands to address the diversity of spectrum allocation (see agreement 15/11/2022 during online session), moderator proposes recommendation based on Option 2b. Option 2b (as starting point for next meeting discussion): RAN4 to consider defining/continue to discuss at next meeting (all options have same priority, companies need more time to check): - n511 with consideration of US/FCC regulations. - [n512 with consideration of CEPT regulations.] Note 1: All companies to further check impacts of latest revision of ECC Decision(05)01 on ECC Decision(13)01 cited in the current WF. - [n510 with consideration of US/FCC regulations.] Note 1: Provide the 3GPP definition that specifies the prevention of the use of a fixed terminal in the Fixed Service Spectrum (FSS). Note 2: Also provide information of a movable NTN user terminal in FSS spectrum in the US. with the following considerations: - DL: 17.7-20.2 GHz (n512, n511, n510); - UL: 27.5-30.0 GHz (n512), 28.35-30.0 GHz (n511), 27.5-28.35 GHz (n510).

Agreements R4-2220241 Table 1.1-1 Aggressor and victim combination No.; Combination; Aggressor; Victim; Notes; Study Phase. 1; TN with NTN; NTN UL; TN UL. 2; TN with NTN; TN UL; NTN UL. 3; TN with NTN; NTN UL; TN DL. 4; TN with NTN; TN DL; NTN UL. 5; TN with NTN; TN DL; NTN DL. 6; TN with NTN; NTN DL; TN DL. 7; TN with NTN; NTN DL; TN UL. 8; TN with NTN; TN UL; NTN DL. NOTE 1: For coexistence between Ka band DL and surrounding TN bands, this need more discussions since currently there are no 3GPP defined TN bands specified..

Moreover, at RAN4#106 the following agreements have been made:

Agreement 1: Study NTN-TN coexistence by assuming a reference frequency of 17 GHz for NTN DL cases and 27 GHz NTN UL cases, as well as the consideration of ACLR and ACS assumptions as following:

| No. | Combination | Aggressor | Victim | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | ACLR NTN UE to be varied/defined ACS TN gNB fixed |
| 2 | TN with NTN | TN UL | NTN UL | ACLR TN UE fixed ACS NTN SAN to be varied/defined |
| 3 | TN with NTN | NTN UL | TN DL | ACLR NTN UE to be varied/defined ACS TN UE fixed |
| 4 | TN with NTN | TN DL | NTN UL | ACLR TN gNB fixed ACS NTN SAN to be varied/defined |
| 5 | TN with NTN | TN DL | NTN DL | ACLR TN gNB fixed ACS NTN UE to be varied/defined |
| 6 | TN with NTN | NTN DL | TN DL | ACLR NTN SAN to be varied/defined ACS TN UE fixed |
| 7 | TN with NTN | NTN DL | TN UL | ACLR NTN SAN to be varied/defined ACS TN gNB fixed |
| 8 | TN with NTN | TN UL | NTN DL | ACLR TN UE fixed ACS NTN UE to be varied/defined |
| NOTE 1: For coexistence between Ka-Band DL and adjacent TN bands, there are no 3GPP defined/specified TN bands. |  |  |  |  |

Furthermore, for the assumption on TN ACLR/ACS for co-existence simulation, the values for 17 GHz in below table are considered as starting point for co-existence simulation purpose yet other options not precluded.

| Frequency band | BS |  | UE |  | ACIR |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ACLR | ACS | ACLR | ACS | BS ACLR UE ACS | UE ACLR BS ACS |
| 17 GHz | [30] | [26] | [19] | [25] | [23.8] | [18.2] |
| 27 GHz | 28 | 24 | 17 | 23 | 21.8 | 16.2 |

Agreement 2: RAN4 to update the NTN-TN coexistence scenarios for above 10 GHz bands with the following figures:

Figure 1. Coexistence scenarios for use cases 1-4 (in above 10 GHz)

Figure 2. Coexistence scenarios for use cases 5-8 (in above 10 GHz)

The following WF documents shall be also taken into account:

- R4-2217468, WF on [313] NR_NTN_enh_Part2 (Samsung), 3GPP TSG-RAN WG4 Meeting #104-bis-e, Electronic Meeting, 10 - 19 October, 2022;

- R4-2220241, Way forward on [105][313] NR_NTN_enh_Part2 (Samsung), 3GPP TSG-RAN WG4 Meeting #105, Toulouse, France, November 14 – November 18, 2022;

- R4-2302878, WF for above 10GHz NTN-TN co-existence study (Samsung), 3GPP TSG-RAN WG4 Meeting #106, Athens, Greece, 27th Feb – 3rd March 2023.

In previous contribution R4-2302535 (“NTN Simulation Parameters for above 10 GHz Coexistence Studies”, THALES), several NTN simulation parameters were proposed for above 10 GHz coexistence studies. The contribution R4-2305847 (THALES, Magister Solutions Ltd) provided material for discussion with respect to Ka-Band NTN-TN coexistence simulations and NTN UE terminal and Satellite antenna parameters. Parameters from R4-2302535 then were updated in R4-2305847, together with initial simulation results.

For other information, please also check (RAN4#106-bis-e):

- R4-2306002 (“WF on [311] NR_NTN_enh_Part3”, Samsung):

- R4-2306003 (“Simulation assumptions for NTN co-existence study in bands above 10GHz”, Samsung).

And also (at RAN4#107):

- R4-2309700 (“Initial NTN calibration results for above 10 GHz Coexistence Studies”, THALES, Magister Solutions Ltd);

- R4-2309768 (“Collection table for NTN co-existence in above 10GHz calibration results”, Samsung);

- R4-2310449 (“Summary for [107][311] NR_NTN_enh_Part3”, Samsung);

- RAN4_107_BSRF_Demod_Test_Session Report_11_EOM.docx;

- R4-2309771 (“Simulation assumptions for NTN co-existence study in bands above 10GHz”, Samsung);

- R4-2309767 (“WF for co-existence study of Above 10GHz NTN band”, Samsung);

The following agreements can be considered for reference only:

- R4-2309770 (“WF on above 10GHz SAN RF requirements“, CATT);

- R4-2309766 (“WF for system parameters on Ka band”, THALES).

The scope of this document is to provide updated calibration/simulation results and proposals to clarify the calibration/simulation assumptions.

# Few proposals for the WF/parameter update

Proposal 1: Preferably use NF SAN=3.5 dB for all constellations (GEO, LEO@1200, LEO@600) for NTN coexistence analysis in above 10 GHz.

Proposal 2: If companies still insist to have NF=5.9 dB for NTN SAN coexistence simulations in above 10 GHz, propose to define 2 SAN classes for such purpose.

Proposal 3: For coexistence simulations in above 10 GHz, preferably use the same number of RBs for calibration and simulation (currently the calibration uses 13 RBs in UL and the simulation uses 2 RBs in UL).

Proposal 4: If no strong concern, coexistence simulations should consider more RBs allocated per UE in order to have representative NTN broadband service type and throughput as per Ka-band (see Table B.2-1 “Non-Terrestrial network target performances per usage scenarios” from TR 38.821 for Ka-band). This is specifically applicable for UL.

Proposal 5: For coexistence simulations in above 10 GHz, preferably use the same NTN UE NF value for both calibration and simulation.

Proposal 6: For coexistence simulations in above 10 GHz, preferably use a 3D isolation distance of 35m between VSAT and TN BS (this is similar to UE-BS distance assumption) due to geometrical constraints (NTN VSAT height of 1.5m and BS height of 25m).

| BS-MS min distance in meters | 35 |
| --- | --- |
| BS noise figure in dB | 10 |
| UE noise figure in dB | 10 |
| BS-UE path-loss model | TR 38.901 |
| Standard deviation of BS-UE log-normal shadow fading in dB | Deployment scenario related, referring to TR 38.901. |
| Shadowing correlation | Between cells: 1.0 Between sites: 0.5 |
| Link-level performance model | See Section 2.10 |
| UE distribution | Uniform |
| Evaluation metrics | See Section 2.9 |

Proposal 7: Need to confirm the total TXP before antenna gains for the TN gNB Tx (and if 2 dB ohmic loss is considered or not). This will result into 41.07 dBm total TN gNB TX power before antenna gains (if ohmic loss is considered: 22 dBm per element + 10*log10(16x8 elements) – 2 dB ohmic loss) or 43.07 dBm total TN gNB TX power before antenna gains (if no ohmic loss is considered: 22 dBm per element + 10*log10(16x8 elements)).

Proposal 8: Atmospheric attenuation OFF to be confirmed with other companies for calibration and for coexistence.

Proposal 9: NTN UL Power Control equation for CLx-ile (and the exact X and Y values) to be confirmed with other companies. The exact value/methodology should be agreed and then NTN UL calibration results updated. The current values are:

### TN UL TPC

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 could be applied for TN with following parameters.

Where, Pmax = 23dBm, Rmin = TBD dB, CLx-ile and γ are set as following:

-	CLx-ile = 88 + 10*log10 (200/X) + 11 – Y,

where X is UL transmission BW (MHz) and Y is the BS noise figure

-	γ = 1For uplink scenario,

| Samsung | Ericsson | Thales | Huawei |  |
| --- | --- | --- | --- | --- |
| Checked | Checked | Checked. | Checked |  |

### NTN UL TPC

For calibration purpose, reuse the same TN TPC for NTN with SNR target 15dB.

For the coexistence study, the same TPC model of TN for NTN UL scenarios is adopted but needs to revise CLx-ile to align with UE UL power control parameters used in TR38.821. The CLx-ile value should be adapted for rural scenario.

| Samsung | Ericsson | Thales | Huawei | ZTE |
| --- | --- | --- | --- | --- |
| Checked | Checked | Checked | Checked | Checked |

However, important miss-alignment still seems to exist in UL between companies. And as a matter of fact, parameterization seems not the same as for TPC model specified in Section 9.1 of TR 36.942.

The following options were mentioned offline:

- Option 1 (suggested by Company 1): CLx-ile = Pmax[dBm]-Po[dBm]=Pmax-(SNRtarget+N)=Pmax-(SNRtarget-174dBm/Hz+10*log10(BW)+NF), where NF=3.5 or 5.9dB and BW is 13RBs or 2RBs (allocated UL NTN VSAT UE BW), SNRtarget=15dB and Pmax=33dBm

- Option 2 (suggested by Company 2): CLx-ile = 10*log10(Pmax) – (SNRtarget + 10*log10(kTBF))= 33dBm – (15 - 85.1dBm) = 103.1 dB (probably assumed that BW is 200MHz)

- Option 3: TBD, other options not precluded

Proposal 9: Companies to decide the exact SINR-Throughput performance metrics. For the time being:

## Throughput ~ SNR mapping

Adopt Section 5.2.7 of TR 38.803[20] as the SINR-Throughput performance metrics

| Samsung | Ericsson | Thales | Huawei | ZTE |
| --- | --- | --- | --- | --- |
| Checked | Checked | Checked | Checked | Checked |

However, miss-alignment may still exist. It should confirmed that this section is still valid:

The throughput of a modem with link adaptation can be approximated by an attenuated and truncated form of the Shannon bound. (The Shannon bound represents the maximum theoretical throughput than can be achieved over an AWGN channel for a given SNIR). The following equations approximate the throughput over a channel with a given SNIR, when using link adaptation:

Throughput SNIR, bps/Hz =0                                  for SNIR                                         ∝∙SSNIR                     for SNIRMIN≤SNIR<SNIRMAX ∝∙SSNIRMAX               for SNIR ≥ SNIRMAX

Where:

S(SNIR)   Shannon bound, S(SNIR) =log2(1+SNIR)  bps/Hz
			Attenuation factor, representing implementation losses
SNIRMIN  	Minimum SNIR of the code set, dB
SNIRMAX  Maximum SNIR of the code set, dB

The parameters α, SNIRMIN and SNIRMAX can be chosen to represent different modem implementations and link conditions. The parameters proposed in table 5.2.7-1 represent a baseline case, which assumes: 1:1 antenna configurations, AWGN channel model, Link Adaptation (see table 5.2.7-1 for details of the highest and lowest rate codes), No HARQ.

Table 5.2.7-1: Parameters describing baseline Link Level performance for 5G NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α, attenuation | 0.6 | 0.4 | Represents implementation losses |
| SNIRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SNIRMAX, dB | 30 | 22 | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

[the exact applicability of the table above should be further discussed]

Note that the parameters proposed in table 5.2.7-1 are targeted for eMBB coexistence scenario.

Proposal 10: Try to align (as much as possible) calibration with simulation parameters and viceversa.

Proposal 11: RAN4 to clarify the size of the TN, and if 1/ all the NTN beam has to be filled with multiple TN clusters or if 2/ only 1 cluster with 19 TN cells (57 sectors are used).

Clarification Note: It does not seem realistic (there is not such FR2 NR deployment to our knowledge) entirely covering an NTN beam, especially in GEO scenario.

- This is particularly important for instance for Scenario #4 & Scenario #8 (“All active TN cells in central NTN beam”) or

- Scenario #2 (with “Only the active TN cells in central NTN beam”) – consider the active TN cells from all clusters? or the active TN cells from only one cluster?

# 3 	Parameters for calibration phase

The following parameters have been used for calibration phase:

NTN Common Parameters

| Bandwidth | 200MHz, numerology 3 (132 PRBs) |
| --- | --- |
| Downlink center frequency | 17 GHz |
| Uplink center frequency | 27 GHz |
| UEs per cell | 1 in DL, 10 in UL |
| UE height | 1.5 m |
| UE Tx power | 33 dBm |
| UE antenna gain | Tx = 43.2 dBi, Rx = 39.7 dBi |
| UE antenna diameter | 0.60 m |
| UE noise figure | 4 dB |
| Downlink allocation size | 132 PRBs |
| Uplink allocation size | 13 PRBs |
| NTN scenario | Urban (was Dense Urban) |
| Satellite antenna type | Parabolic (TR 38.811 Bessel) |
| UE antenna type | Parabolic (TR 38.811 Bessel) |
| Shadowing | Enabled |
| Uplink Tx power control | Enabled, CLxile = 88.0 + 10*log10(200/18.72[MHz]) + 11 – 5.9 = 103.4 dB |
| Cell selection | RSRP |
| Frequency re-use | 1 |

NTN Scenario Specific Parameters

|  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- |
| Satellite antenna diameter | 5.9/3.66 m DL/UL | 0.6/0.36 m | 0.6/0.36 m |
| Satellite Tx power | 40 dBW/MHz | 10 dBW/MHz | 4 dBW/MHz |
| Satellite antenna gain dBi | 58.5 | 38.5 | 38.5 |
| Noise figure | 4 dB | 4 dB | 4 dB |
| G/T dB/K | 27.976 (=5.9 dB NF) | 7.976 (=5.9 dB NF) | 7.976 (=5.9 dB NF) |
| Beamwidth deg | 0. 1763 | 1.7337 | 1.7337 |

TN Common Parameters

| UE height | 1.5 m |
| --- | --- |
| Bandwidth | 200 MHz, numerology 3 (132 PRBs) |
| Downlink center frequency | 17 GHz |
| Uplink center frequency | 27 GHz |
| gNB antenna element gain | 5.5 dBi |
| gNB antenna elements | 16x8 |
| gNB total TXP | 41.07 dBm (22 dBm per element /w 2dB ohmic loss) |
| gNB antenna beamwidth | Horizontal = 90.0°, Vertical = 90.0° |
| Shadowing | Enabled |
| UE Tx power | 23dBm (not 22.4 dBm) |
| Tx power control | R4-2120671, CLxile = 88.0 + 10*log10(200/190.08[Mhz]) + 11 –10 = 89.22 dB |
| Simulation time | One step, 20 drops |
| UE antenna type | 3GPP TR38.901, 2x2 antenna array with 5.0 dBi element gain |
| gNB antenna type | 3GPP TR38.901, 16x8 antenna array with RAN4 agreements |
| Downlink allocation size (PRB) | 132 PRB = Full bandwidth |
| Uplink allocation size (PRB) | 132 PRBs = Full bandwidth |
| UE height | 1.5 m |

1/ Calibration results NTN GEO DL VSAT 17 GHz:

2/ Calibration results NTN GEO UL VSAT 27 GHz:

3/ Calibration results NTN LEO@1200 DL VSAT 17 GHz:

4/ Calibration results NTN LEO@1200 UL VSAT 27 GHz:

5/ Calibration results NTN LEO@600 DL VSAT 17 GHz:

6/ Calibration results NTN LEO@600 UL VSAT 27 GHz:

# Initial Simulation Results

Scenario 1 – NTN UL to TN UL

Scenario 1 co-existence configuration

| Scenario | Freq | Aggressor | Victim | Variate | TN ACI config |
| --- | --- | --- | --- | --- | --- |
| SC1 | 27 | NTN UL | TN UL | NTN UT ACLR | TN gNB ACS 24 dB |

SC1 LEO@600 & LEO@1200 TN UL throughput loss

SC1 GEO TN UL throughput and SINR loss

SC1 GEO NTN UL total transmission power

SC1 GEO TN UL ACI and interference distributions

SC1 GEO TN UL SINR distribution

SC1 GEO UL ACI transmission TX and RX antenna gain distributions

Scenario 2 – TN UL to NTN UL

Scenario 2 co-existence configuration

| Scenario | Freq | Aggressor | Victim | Variate | TN ACI config |
| --- | --- | --- | --- | --- | --- |
| SC2 | 27 | TN UL | NTN UL | NTN SAN ACS | TN UE ACLR 17 dB |

SC2 LEO@600 NTN UL ACI and interference distributions

Scenario 3 – NTN UL to TN DL

Scenario 3 co-existence configuration

| Scenario | Freq | Aggressor | Victim | Variate | TN ACI config |
| --- | --- | --- | --- | --- | --- |
| SC3 | 27 | NTN UL | TN DL | NTN UT ACLR | TN UE ACS 23 dB |

SC3 LEO@600 & LEO@1200 TN DL throughput loss

SC3 GEO TN DL throughput loss

SC3 GEO TN DL ACI and interference distributions

Scenario 4 – TN DL to NTN UL

Scenario 4 co-existence configuration

| Scenario | Freq | Aggressor | Victim | Variate | TN ACI config |
| --- | --- | --- | --- | --- | --- |
| SC4 | 27 | TN DL | NTN UL | NTN SAN ACS | TN gNB ACLR 28 dB |

SC4 LEO@600 & LEO@1200 TN DL throughput loss

SC4 GEO TN DL throughput loss

SC4 LEO@600 NTN UL ACI and interference distributions

SC4 GEO NTN UL ACI and interference distributions

Scenario 5 – TN DL to NTN DL

Scenario 5 co-existence configuration

| Scenario | Freq | Aggressor | Victim | Variate | TN ACI config |
| --- | --- | --- | --- | --- | --- |
| SC5 | 17 | TN DL | NTN DL | NTN UT ACS | TN gNB ACLR 30 dB |

SC5 LEO@600 & LEO@1200 NTN DL throughput loss

SC5 GEO NTN DL throughput loss

SC5 LEO600 DL ACI and NTN DL SINR distributions

Scenario 6 – NTN DL to TN DL

Scenario 6 co-existence configuration

| Scenario | Freq | Aggressor | Victim | Variate | TN ACI config |
| --- | --- | --- | --- | --- | --- |
| SC6 | 17 | NTN DL | TN DL | NTN SAN ACLR | TN UE ACS 25 dB |

SC6 LEO@600 TN DL ACI and interference distributions

Scenario 7 – NTN DL to TN UL

Scenario 7 co-existence configuration

| Scenario | Freq | Aggressor | Victim | Variate | TN ACI config |
| --- | --- | --- | --- | --- | --- |
| SC7 | 17 | NTN DL | TN UL | NTN SAN ACLR | TN gNB ACS 26 dB |

SC7 LEO@600 & LEO@1200 TN UL throughput loss

SC7 GEO TN UL throughput loss

SC7 LEO@600 TN UL ACI distribution

Scenario 8 – TN UL to NTN DL

Scenario 8 co-existence configuration

| Scenario | Freq | Aggressor | Victim | Variate | TN ACI config |
| --- | --- | --- | --- | --- | --- |
| SC8 | 17 | TN UL | NTN DL | NTN UT ACS | TN UE ACLR 19 dB |

SC8 LEO@600 & LEO@1200 NTN DL throughput loss

SC8 GEO NTN DL throughput loss

Results conclusions and sensitivity analysis

NTN ACLR / ACS requirements from co-existence simulations

| NTN SAN ACLR | < 10 dB |
| --- | --- |
| NTN SAN ACS | < 10 dB |
| NTN UE ACLR | ~10 dB |
| NTN UE ACS | ~20 dB |

However, this simulation campaign is the first iteration, and the work will continue in 3GPP. There are still many (on-going) assumptions that can change the results, including the density of the terrestrial network, and the UL power control scheme.

# Conclusions

Proposal 1: Preferably use NF SAN=3.5 dB for all constellations (GEO, LEO@1200, LEO@600) for NTN coexistence analysis in above 10 GHz.

Proposal 2: If companies still insist to have NF=5.9 dB for NTN SAN coexistence simulations in above 10 GHz, propose to define 2 SAN classes for such purpose.

Proposal 3: For coexistence simulations in above 10 GHz, preferably use the same number of RBs for calibration and simulation (currently the calibration uses 13 RBs and the simulation uses 2 RBs).

Proposal 4: If no strong concern, coexistence simulations should consider more RBs allocated per UE in order to have representative NTN broadband service type and throughput as per Ka-band (see Table B.2-1 “Non-Terrestrial network target performances per usage scenarios” from TR 38.821 for Ka-band).

Proposal 5: For coexistence simulations in above 10 GHz, preferably use the same NTN UE NF value for both calibration and simulation.

Proposal 6: For coexistence simulations in above 10 GHz, preferably use an isolation distance of 35m between VSAT and TN BS (this is similar to UE-BS distance assumption) due to geometrical constraints (NTN VSAT height of 1.5m and BS height of 25m).

| BS-MS min distance in meters | 35 |
| --- | --- |
| BS noise figure in dB | 10 |
| UE noise figure in dB | 10 |
| BS-UE path-loss model | TR 38.901 |
| Standard deviation of BS-UE log-normal shadow fading in dB | Deployment scenario related, referring to TR 38.901. |
| Shadowing correlation | Between cells: 1.0 Between sites: 0.5 |
| Link-level performance model | See Section 2.10 |
| UE distribution | Uniform |
| Evaluation metrics | See Section 2.9 |

Proposal 7: Need to confirm the total TXP before antenna gains for the TN gNB Tx (and if 2 dB ohmic loss is considered or not). This will result into 41.07 dBm total TN gNB TX power before antenna gains (if ohmic loss is considered: 22 dBm per element + 10log10(16x8 elements) - 2dB ohmic loss) or 43.07 dBm total TN gNB TX power before antenna gains (if no ohmic loss is considered: 22 dBm per element + 10log10(16x8 elements)).

Proposal 8: Atmospheric attenuation OFF to be confirmed with other companies for calibration and for coexistence.

Proposal 9: NTN UL Power Control equation for CLx-ile (and the exact X and Y values) to be confirmed with other companies. The exact value/methodology should be agreed and then NTN UL calibration results updated. The current values are:

### TN UL TPC

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 could be applied for TN with following parameters.

Where, Pmax = 23dBm, Rmin = TBD dB, CLx-ile and γ are set as following:

-	CLx-ile = 88 + 10*log10 (200/X) + 11 – Y,

where X is UL transmission BW (MHz) and Y is the BS noise figure

-	γ = 1For uplink scenario,

| Samsung | Ericsson | Thales | Huawei |  |
| --- | --- | --- | --- | --- |
| Checked | Checked | Checked. | Checked |  |

### NTN UL TPC

For calibration purpose, reuse the same TN TPC for NTN with SNR target 15dB.

For the coexistence study, the same TPC model of TN for NTN UL scenarios is adopted but needs to revise CLx-ile to align with UE UL power control parameters used in TR38.821. The CLx-ile value should be adapted for rural scenario.

| Samsung | Ericsson | Thales | Huawei | ZTE |
| --- | --- | --- | --- | --- |
| Checked | Checked | Checked | Checked | Checked |

However, important miss-alignment still seems to exist in UL between companies. And as a matter of fact, parameterization seems not the same as for TPC model specified in Section 9.1 of TR 36.942.

The following options were mentioned offline:

- Option 1 (suggested by Company 1): CLx-ile = Pmax[dBm]-Po[dBm]=Pmax-(SNRtarget+N)=Pmax-(SNRtarget-174dBm/Hz+10*log10(BW)+NF), where NF=3.5 or 5.9dB and BW is 13RBs or 2RBs (allocated UL NTN VSAT UE BW), SNRtarget=15dB and Pmax=33dBm

- Option 2 (suggested by Company 2): CLx-ile = 10*log10(Pmax) – (SNRtarget + 10*log10(kTBF))= 33dBm – (15 - 85.1dBm) = 103.1 dB (probably assumed that BW is 200MHz)

- Option 3: TBD, other options not precluded

Proposal 9: Companies to decide the exact SINR-Throughput performance metrics. For the time being:

## Throughput ~ SNR mapping

Adopt Section 5.2.7 of TR 38.803[20] as the SINR-Throughput performance metrics

| Samsung | Ericsson | Thales | Huawei | ZTE |
| --- | --- | --- | --- | --- |
| Checked | Checked | Checked | Checked | Checked |

However, miss-alignment may still exist. It should confirmed that this section is still valid:

The throughput of a modem with link adaptation can be approximated by an attenuated and truncated form of the Shannon bound. (The Shannon bound represents the maximum theoretical throughput than can be achieved over an AWGN channel for a given SNIR). The following equations approximate the throughput over a channel with a given SNIR, when using link adaptation:

Throughput SNIR, bps/Hz =0                                  for SNIR                                         ∝∙SSNIR                     for SNIRMIN≤SNIR<SNIRMAX ∝∙SSNIRMAX               for SNIR ≥ SNIRMAX

Where:

S(SNIR)   Shannon bound, S(SNIR) =log2(1+SNIR)  bps/Hz
			Attenuation factor, representing implementation losses
SNIRMIN  	Minimum SNIR of the code set, dB
SNIRMAX  Maximum SNIR of the code set, dB

The parameters α, SNIRMIN and SNIRMAX can be chosen to represent different modem implementations and link conditions. The parameters proposed in table 5.2.7-1 represent a baseline case, which assumes:

-	1:1 antenna configurations

-	AWGN channel model

-	Link Adaptation (see table 5.2.7-1 for details of the highest and lowest rate codes)

-	No HARQ

Table 5.2.7-1: Parameters describing baseline Link Level performance for 5G NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α, attenuation | 0.6 | 0.4 | Represents implementation losses |
| SNIRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SNIRMAX, dB | 30 | 22 | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

[the exact applicability of the table above should be further discussed]

Note that the parameters proposed in table 5.2.7-1 are targeted for eMBB coexistence scenario.

Proposal 10: Try to align (as much as possible) calibration with simulation parameters and viceversa.

Proposal 11: RAN4 to clarify the size of the TN, and if 1/ all the NTN beam has to be filled with multiple TN clusters or if 2/ only 1 cluster with 19 TN cells (57 sectors are used).

Clarification Note: It does not seem realistic (there is not such FR2 NR deployment to our knowledge) entirely covering an NTN beam, especially in GEO scenario.

- This is particularly important for instance for Scenario #4 & Scenario #8 (“All active TN cells in central NTN beam”) or

- Scenario #2 (with “Only the active TN cells in central NTN beam”) – consider the active TN cells from all clusters? or the active TN cells from only one cluster?

# References

- R4-2220239, “WF for above 10GHz band definition and system parameters”, THALES.

- R4-2220241, “Simulation assumptions for NTN co-existence study in bands above 10GHz”, Samsung.

- R4-2213361, “Discussion on Ka adjacent band NTN-TN NR coexistence scenarios”.

- TR 38.811.

- TR 38.821.

- Ka-Band Low Noise Amplifier Sub-System Module for Communication Satellite payload”, S. K. Jain and all.

- ITU-R Rec. P.618-12.

- ITU-R Rec. S.465-6.

- R.C. Hansen “Phased Arrays” in “Antenna Engineering Handbook” by John L. Volakis, editor. McGraw Hill. 4th edition. 2007.

- R4-2305847, “Updated NTN Simulation Parameters for above 10 GHz Coexistence Studies and Initial Simulation Results”, THALES, Magister Solutions Ltd.

- R4-2306002, “WF on [311] NR_NTN_enh_Part3”, Samsung.

- R4-2306003, “Simulation assumptions for NTN co-existence study in bands above 10GHz”, Samsung.

# ANNEX - Initial NTN simulation results in above 10 GHz (from R4-2305847, RAN4#106-bis-e)

First, coexistence parameters should be chosen to allow NTN broadband connectivity in above 10 GHz.

NTN Common Parameters

| Bandwidth | 200 MHz, numerology 3 (132 PRBs) |
| --- | --- |
| Downlink center frequency | 17 GHz |
| Uplink center frequency | 27 GHz |
| UEs per cell | 10 |
| UE height | 1.5 m |
| UE Tx power | 3.0 dBW |
| UE antenna gain dBi | Tx = 42.9, Rx = 39.0 |
| UE antenna diameter | 0.60 m |
| UE noise figure | 2.1dB |
| Satellite antenna type | Circular aperture RAN4 THALES |
| UE antenna type | ITU-R S.465-6 |
| Shadowing | Enabled |
| Uplink Tx power control | Disabled |
| Cell selection | RSRP |
| Frequency re-use | 1 and 3 |

NTN Scenario Specific Parameters

|  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- |
| Satellite Tx power | 40 dBW/MHz | 10 dBW/MHz | 4 dBW/MHz |
| Satellite antenna gain dBi | 58.5 | 38.5 | 38.5 |
| Noise figure | 3.5 dB | 3.5 dB | 3.5 dB |
| G/T dB/K | 30.4 | 10.4 | 10.4 |
| Beamwidth deg | 0.1884 | 1.9012 | 1.9012 |

As seen from the next simulation results, FRF>1 is preferable for broadband NTN communications above 10 GHz.

3.1 Results for FRF1

1/ Coupling loss

2/ Carrier signal power

3/ SINR

As seen from simulation results, FRF=1 is not preferable for broadband NTN communications above 10 GHz, since the expected SINR is quite low for broadband communication purpose.

3.2 Results for FRF3

1/ Coupling loss

2/ Carrier signal power

3/ SINR

As seen from simulation results, FRF=3 is an option for broadband NTN communications above 10 GHz.

3.3 Results for FRF2 with dual polarization

1/ Coupling loss

2/ Carrier signal power

3/ SINR

As seen from simulation results, FRF=2 with dual polarization is an option for broadband NTN communications above 10 GHz.

Observation 1: FRF=1 is not preferable for broadband NTN communications above 10 GHz, since the SINR values are limited due to inter-neighbour beam interference.

Observation 2: FRF=3 is an option for broadband NTN communications above 10 GHz.

Observation 3: FRF=2 with dual polarization is an option for broadband NTN communications above 10 GHz.

# ANNEX - Initial TN simulation results in above 10 GHz (from R4-2305847, RAN4#106-bis-e)

TN Common Parameters

| Bandwidth | 200 MHz, numerology 3 (132 PRBs) |
| --- | --- |
| Downlink center frequency | 17 GHz |
| Uplink center frequency | 27 GHz |
| UE height | 1.5 m |
| gNB antenna element gain dBi | 5.5 |
| gNB antenna elements | 16x8 |
| gNB antenna beamwidth deg | Horizontal = 90.0, Vertical = 90.0 |
| Shadowing | Enabled |
| UE Tx power | 22.4 dBm EIRP |
| Tx power control | R4-2120671, CLxile = 71.0 + 10*log10(200/BW [Mhz]) dB |
| UE antenna type | 3GPP TR38.901, 2x2 antenna array with 5.0 dBi element gain |
| gNB antenna type | 3GPP TR38.901, 16x8 antenna array with RAN4 agreements |

1/ Coupling loss

2/ Carrier signal power

3/ SINR

# ANNEX – Calibration Results

These results are submitted to the Excel Results Collection file for NTN calibration in above 10 GHz (See R4-2313101, Samsung) at RAN4#108 meeting.

## TN Results

## NTN Results

-END-